| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0406 |
| --- | --- |

| Title: | AHG12: On CABAC zero words for subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution asserts that subpicture extraction could result in violating the bin-to-bit ratio constraint as CABAC zero words are not associated to subpictures. Therefore, it is proposed to add a bitstream constraint for the VCL NAL units of subpictures to fulfil the bin-to-bit ratio constraint individually in addition to the picture unit level constraint when the following conditions apply:

- a subpicture has subpic_treated_as_pic_flag[ ] equal to 1.

- a subpicture level information SEI message is present in the CLVS.

# Introduction

Similar to HEVC, the current VVC draft specification contains a mechanism to insert CABAC zero word into the RBSP trailing bits of slices as a means to maintain a maximum bin-to-bit ratio for the VCL NAL units of a coded picture as follows.

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepth + 2 * BitDepth / ( SubWidthC * SubHeightC ) )	(89)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

Furthermore, VVC also features independently coded subpictures with boundaries that are treated similar to picture boundaries for motion compensated inter-prediction and associated level information. All these measures are intended to enable lightweight extraction and merging of such subpictures into conformant VVC bitstreams.

A design principle during the development of VVC has been that the extraction (and merging) of subpictures should not require changes to the VCL NAL units of the resulting subpicture bitstream.

However, in order to check the above bin-to-bit ratio constraint, the sub-bitstream would have to be CABAC decoded and when the VCL NAL units associated with a subpicture exceed the constraint, CABAC zero words will have to be added to the respective slices during extraction.

# Proposal

In line with the above stated design principle, the proposal is to add a bin-to-bit ratio constraint that is specific to the subpicture with index subPicA that fulfils the following conditions.

- subpic_treated_as_pic_flag[ subPicA ] equal to 1

- A subpicture level information SEI message is present in the CLVS.

The following highlighted changes to the semantivs of syntax element cabac_zero_word are proposed.

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInPicVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInPicNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepth + 2 * BitDepth / ( SubWidthC * SubHeightC ) )	(89)

The value of BinCountsInPicNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInPicVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

Let NumBytesInSubPicVclNalUnits be the sum of the values NumBytesInNalUnit for all VCL NAL units of a subpicture with index subPicA.

Let BinCountsInSubPicNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a subpicture with index subPicA.

The variable  of subPicSizeInMinCbsY of the subpicture with index subPicA is derived to be equal to ( subpic_width_minus1[ subPicA ] * CtbSizeY / MinCbSizeY * subpic_height_minus1[ subPicA ] * CtbSizeY/ MinCbSizeY )

For each subpicture with subpic_treated_as_pic_flag[ subPicA ] equal to 1 and that has an associated subpicture level information SEI message, the value of BinCountsInSubPicNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInSubPicVclNalUnits + ( RawMinCuBits * subPicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)