| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0417-v1 |
| --- | --- |

| Title: | AHG8/AHG9: On SPS level virtual boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Yan Ye, Ru-Ling Liao, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In VVC draft 7, virtual bounary is signaled in either SPS or PH. When virtual boundary is signaled in SPS, there is a constraint on the virtual boundary position that depends on the picture width and height, which are signaled in PPS. When the picture width or height is changed, it is not clear how the SPS virtual boundary constraint should be applied, that is, which width/height should be referred to. Three methods are proposed in this contribution to solve this problem.

Method 1 proposes to restrict the virtual boundary signaled in the SPS with the maximum picture width and height that are also signaled in the SPS and only consider the virtual boundaries within the current picture boundary when it is applied to each picture.

Method 2 proposes to scale the virtual boundaries signaled in SPS to fit the size of each picture. By doing this, virtual boundary could be better combined with reference picture resampling (RPR).

Method 3 proposes to use SPS level virtual boundary signaling and RPR in a mutually exclusive manner, that is, one is disallowed when the other is in use

Problem statement

Virtual boundary was adopted to VVC draft to disable the loop-filtering operation on the discontinuous boundaries between different faces for those project format with multiple faces to reduce the seam artifacts [1] in 360 video coding. In VVC draft 7 [2], virtual boundary may be signaled in SPS or PH. The syntax is shown in Table 1 and Table 2.

Table 1 SPS syntax for virtual boundary in VVC draft 7

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_x[ i ] | u(13) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations.

sps_num_ver_virtual_boundaries specifies the number of sps_virtual_boundaries_pos_x[ i ] syntax elements that are present in the SPS. When sps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

sps_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive. [Ed. (VD): pic_width_in_luma_samples is in the PPS, not in the SPS.]

sps_num_hor_virtual_boundaries specifies the number of sps_virtual_boundaries_pos_y[ i ] syntax elements that are present in the SPS. When sps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

sps_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive. [Ed. (VD): pic_height_in_luma_samples is in the PPS, not in the SPS.]

Table 2 PH syntax for virtual boundary in VVC draft 7

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |

ph_num_ver_virtual_boundaries specifies the number of ph_virtual_boundaries_pos_x[ i ] syntax elements that are present in the PH. When ph_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

The parameter VirtualBoundariesNumVer is derived as follows:

VirtualBoundariesNumVer = sps_virtual_boundaries_present_flag ?
	sps_num_ver_virtual_boundaries : ph_num_ver_virtual_boundaries

ph_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the vertical virtual boundary VirtualBoundariesPosX[ i ] in units of luma samples is derived as follows:

VirtualBoundariesPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
	sps_virtual_boundaries_pos_x[ i ] : ph_virtual_boundaries_pos_x[ i ] ) * 8

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

ph_num_hor_virtual_boundaries specifies the number of ph_virtual_boundaries_pos_y[ i ] syntax elements that are present in the PH. When ph_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

The parameter VirtualBoundariesNumHor is derived as follows:

VirtualBoundariesNumHor = sps_virtual_boundaries_present_flag ?
	sps_num_hor_virtual_boundaries : ph_num_hor_virtual_boundaries

ph_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of ph_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the horizontal virtual boundary VirtualBoundariesPosY[ i ] in units of luma samples is derived as follows:

VirtualBoundariesPosY[ i ] = ( sps_virtual_boundaries_present_flag ?
	sps_virtual_boundaries_pos_y[ i ] : ph_virtual_boundaries_pos_y[ i ] ) * 8

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

As in the above semantics, the value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) – 1 and the value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) – 1. However, due to the adoption of RPR in VVC, pic_width_in_luma_samples and pic_height_in_luma_samples are signaled in PPS and may have different values in different PPS. So it is not clear which pic_width_in_luma_samples and which pic_height_in_luma_samples are used here, since there could be multiple PPS refer to one SPS. This issue is also mentioned in the editing notes in VVC draft 7. If we simply restrict the virtual boundary positon with the minimum one of pic_width_in_luma_samples and pic_height_in_luma_samples in all the PPS, the virtual boundary can only be located within a small portion of the large picture on the top-left corner. To solve this problem, three methods are proposed in this contribution.

Proposed method

Method 1

In method 1, it is proposed to calculate the upper bound of virtual boundary positon (sps_virtual_boundaries_pos_x[ i ] and sps_virtual_boundaries_pos_y[ i ]) using maximum picture width and height, pic_width_max_in_luma_samples and pic_width_max_in_luma_samples, which are signaled in the SPS, instead of using pic_width_in_luma_samples and pic_width_in_luma_samples which is signaled in the PPS. And for each picture, if a virtual boundary position is beyond the picture boundary in either dimension, then that virtual boundary is removed (i.e. ignored); for those virtual boundaries that are within the picture boundary in either dimension, they can still be applied.

The proposed spec. changes are as follows.

First, the constraint of sps_virtual_boundaries_pos_x[ i ] and sps_virtual_boundaries_pos_y[ i ] are changed to dependent on pic_width_max_in_luma_samples and pic_height_max_in_luma_sample.

sps_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive

sps_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive

Then, the derivation process of virtual boundary number and location are changed as follows

The parameter VirtualBoundariesNumVer is derived as follows:

VirtualBoundariesNumVer = sps_virtual_boundaries_present_flag ?
	sps_num_ver_virtual_boundaries : ph_num_ver_virtual_boundaries

The location of the vertical virtual boundary VirtualBoundariesPosX[ i ] in units of luma samples is derived as follows:

VirtualBoundariesPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
	sps_virtual_boundaries_pos_x[ i ] : ph_virtual_boundaries_pos_x[ i ] ) * 8

The parameter VirtualBoundariesNumVer and the location of the vertical virtual boundary VirtualBoundariesPosX[ i ] in units of luma samples are derived as follows:

for(i=0,j=0;i< sps_num_ver_virtual_boundaries;i++){

if(sps_virtual_boundaries_pos_x[ i ]<= Ceil( pic_width_in_luma_samples ÷ 8 ) – 1){

VirtualBoundariesPosX[j++]= sps_virtual_boundaries_pos_x[ i ]

}

}

VirtualBoundariesNumVer=j

VirtualBoundariesNumVer = sps_virtual_boundaries_present_flag ?
	VirtualBoundariesNumVer : ph_num_ver_virtual_boundaries

VirtualBoundariesPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
	VirtualBoundariesPosX [ i ]: ph_virtual_boundaries_pos_x[ i ] ) * 8, (i=0.. VirtualBoundariesNumVer)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

The parameter VirtualBoundariesNumHor is derived as follows:

VirtualBoundariesNumHor = sps_virtual_boundaries_present_flag ?
	sps_num_hor_virtual_boundaries : ph_num_hor_virtual_boundaries

The location of the horizontal virtual boundary VirtualBoundariesPosY[ i ] in units of luma samples is derived as follows:

VirtualBoundariesPosY[ i ] = ( sps_virtual_boundaries_present_flag ?
	sps_virtual_boundaries_pos_y[ i ] : ph_virtual_boundaries_pos_y[ i ] ) * 8

The parameter VirtualBoundariesNumHor and the location of the horizontal virtual boundary VirtualBoundariesPosY[ i ] in units of luma samples is derived as follows:

for(i=0,j=0;i< sps_num_hor_virtual_boundaries;i++){

if(sps_virtual_boundaries_pos_y [ i ] <= Ceil( pic_height_in_luma_samples ÷ 8 ) – 1){

VirtualBoundariesPosY[j++]=  sps_virtual_boundaries_pos_y [ i ]

}

}

VirtualBoundariesNumHor=j

VirtualBoundariesNumHor = sps_virtual_boundaries_present_flag ?
	VirtualBoundariesNumHor : ph_num_hor_virtual_boundaries

VirtualBoundariesPosY[ i ] = ( sps_virtual_boundaries_present_flag ?
	VirtualBoundariesPosY [ i ] : ph_virtual_boundaries_pos_y[ i ] ) * 8, (i=0.. VirtualBoundariesNumHor)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

Method 2

Usually, when the picture resolution is reduced, the picture is re-sampled instead of directly cropped. Thus, to align the virtual boundaries with the same content among picture with different resolutions, the virtual boundary position is also scaled by the same scaling factor as picture resolutions in method 2.

In this method, the virtual boundary position signaled in SPS is calculated using pic_width_max_in_luma_samples and pic_height_max_in_luma_samples which are also signaled in SPS. For each picture, the virtual boundary position is derived by scaling the virtual boundary position signaled in SPS with the scaling factor which is calcuated as the ratio between pic_width_max_in_luma_samples (or pic_height_max_in_luma_samples) and pic_width _in_luma_samples (or pic_height_in_luma_samples). To simplify scaling factor calculation, the scaling factor of each picture is directly calculated by the maximum picture size and the picture size without considering scaling window as done in the case of scaling ratio calculation in RPR.

The proposed spec. changes are as follows.

First, the constraint of sps_virtual_boundaries_pos_x[ i ] and sps_virtual_boundaries_pos_y[ i ] are changed to dependent on pic_width_max_in_luma_samples and pic_height_max_in_luma_sample.

sps_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive.

sps_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive.

Then, the derivation process of the virtual boundary location and the number of virtual boundary are changed as follows.

The parameter VirtualBoundariesNumVer is derived as follows:

VirtualBoundariesNumVer = sps_virtual_boundaries_present_flag ?
	sps_num_ver_virtual_boundaries : ph_num_ver_virtual_boundaries

The location of the vertical virtual boundary VirtualBoundariesPosX[ i ] in units of luma samples is derived as follows:

VirtualBoundariesPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
	sps_virtual_boundaries_pos_x[ i ] : ph_virtual_boundaries_pos_x[ i ] ) * 8

The scaling factor VBScaleX and VBScaleY are derived as follows:

VBScaleX = ( (pic_width_max_in_luma_samples   <<  14 ) + (pic_width_in_luma_samples   >>  1 ) ) / pic_width_in_luma_samples

VBScaleY = ( (pic_height_max_in_luma_samples   <<  14 ) + (pic_height_in_luma_samples   >>  1 ) ) / pic_height_in_luma_samples

The parameters SPSVirtualBoundariesPosX[i] and SPSVirtualBoundariesPosY[i] are derived as follows:

SPSVirtualBoundariesPosX[i]= (sps_virtual_boundaries_pos_x[ i ]* VBScaleX + ( 1 << 13 )) >> 14

SPSVirtualBoundariesPosX[i]=(( SPSVirtualBoundariesPosX[i] + 4 ) >> 3) << 3

SPSVirtualBoundariesPosY[i]= (sps_virtual_boundaries_pos_y[ i ]* VBScaleX + ( 1 << 13 )) >> 14

SPSVirtualBoundariesPosY[i]=(( SPSVirtualBoundariesPosY[i] + 4 ) >> 3) << 3

The location of the horizontal virtual boundary VirtualBoundariesPosX[ i ] and vertical virtual boundary VirtualBoundariesPosY[ i ] in units of luma samples are derived as follows:

VirtualBoundariesPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
	           (SPSVirtualBoundariesPosX [ i ]: ph_virtual_boundaries_pos_x[ i ] ) * 8, (i=0.. VirtualBoundariesNumVer)

VirtualBoundariesPosY[ i ] = ( sps_virtual_boundaries_present_flag ?
	(SPSVirtualBoundariesPosY [ i ] : ph_virtual_boundaries_pos_y[ i ] ) * 8, (i=0.. VirtualBoundariesNumHor)

Method 3

Considering the fact that, in the real word application, virtual boundary and RPR have different use cases and are unlikely to be used together in one application, in method 3, it is proposed use SPS level signaling of virtual boundary and RPR in a mutually exclusive manner to disallow their combination. That is, if virtual boundary is enabled in SPS, then all the pictures that refers to the SPS shall have the same width and height. By doing this, the constraint of virtual boundary position signaled in SPS is only dependent on the maximum picture width and height which are also signaled in SPS.

The proposed spec. changes are as follows.

First, the constraint of sps_virtual_boundaries_pos_x[ i ] and sps_virtual_boundaries_pos_y[ i ] are changed to dependent on pic_width_max_in_luma_samples and pic_height_max_in_luma_sample.

sps_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive

sps_virtual_boundaries_pos_y[ i ] specifies the location of the i-th horizontal virtual boundary in units of luma samples divided by 8. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive

A bitstream conformance requirement of pic_width_in_luma_samples and pic_height_in_luma_samples is imposed as follows:

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0 or sps_num_ver_virtual_boundaries is not equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0 or sps_num_hor_virtual_boundaries is not equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

To further simplify this method, an alternative way is proposed in which RPR is disabled if there are virtual boundaries in either dimension. The bitstream constraint proposed is as follows.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0 or sps_virtual_boundaries_present_flag is not equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0 or sps_virtual_boundaries_present_flag is not equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

Conclusion

This contribution proposes to fix the problem in the syntax constraint of SPS virtual boundary. Three methods are proposed in this contribution. Method 1 proposes to discard the SPS signaled virtual boundary which are over picture boundary; method 2 proposes to scaling the virtual boundary signaled in SPS to fit the current picture size; method 3 proposes exclusive use of SPS level virtual boundary and picture resolution change. It is suggested to adopt one of the methods in this contribution to the next version of VVC draft.

Reference

S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C Chang, C.-C. Ju, P. Hanhart, Y. He, “AHG12: loop filter disabled across virtual boundary”, JVET-N0438, March 2019

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, October 2019.

G. J. Sullivan, J.-R. Ohm, “Meeting report of the 16th JVET Meeting”, JVET-P2000, October 2019

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).