| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0423-v3 |
| --- | --- |

| Title: | Interaction between ACT and BDPCM chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Xiang Li, Shan Liu | Tel: Email: | {aurali, xlxiangli, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to signal ACT flag first and turn off BDPCM chroma when ACT flag is turned on in SPS level. The experiment results reportedly show overall -0.2%, 0.1%, 0.1% and -0.1%, 0.1%, 0.1% bit rate changes for Y, U, V in AI, RA, respectively with RGB and YUV 444 test sequences. In addition, the encoding runtime overall is reduced to 97% and 99% in AI and RA.

# Introduction

In the last Geneva meeting, both ACT and BDPCM chroma are adopted for 444 sequences including RGB and YUV 444. In VVC Draft 7, CU level ACT flag is decoded first and when a CU uses ACT, BDPCM chroma is not applied. Therefore, in CU level, ACT and BDPCM chroma is mutually exclusive.

In this proposal, interaction between ACT and BDPCM chroma is studied by turning on/off SPS level control flag.

# Proposed Method

As ACT and BDPCM chroma cannot be used at block level at the same time, it is proposed to disable BDPCM chroma when ACT is applied in SPS.

Simulation result shows that encoding runtime is generally reduced to 97% in AI, 99% in RA. This is mainly because adopted chroma BDPCM has impact on encoding time in general.

# Experiment results

The proposed solution has been implemented into VTM 7.1 [2] and tests were conducted on RGB and YUV 444 sequences with test condition defined in CE2 [3].

Anchor: VTM 7.1 with additional configuration. DualITree = 0, ColorTransform = 1

Tested: anchor with proposed solution

Table 1: proposed method tested on VTM 7.1 with AI configuration

|  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y |  |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% |  |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.1% |  |  | -0.1% |  | 0.0% |
| RGB, Animation, 720p | 0.0% |  |  | 0.1% |  | 0.0% |
| RGB, camera captured, 1080p | -0.1% |  |  | 0.0% |  | 0.0% |
| TGM 1080p | -0.2% |  |  | 0.2% |  | 0.2% |
| TGM 720p | -0.4% |  |  | 0.3% |  | 0.4% |
| Animation | -0.2% |  |  | 0.1% |  | 0.1% |
| Mixed content | -0.7% |  |  | 0.4% |  | 0.3% |
| Camera-Captured content | -0.2% |  |  | 0.1% |  | 0.1% |
| Enc Time[%] | 97% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  |  | Time Ratio |  |  |
|  |  | Decoding | Encoding |  |
| RGB, text & graphics with motion, 1080p & 720p |  | 1.01 | 0.98 |  |
| RGB, mixed content, 1440p & 1080p |  | 1.00 | 0.98 |  |
| RGB, Animation, 720p |  | 0.92 | 0.95 |  |
| RGB, camera captured, 1080p |  | 1.01 | 0.95 |  |
| TGM 1080p |  | 1.04 | 0.96 |  |
| TGM 720p |  | 1.01 | 0.95 |  |
| Animation |  | 0.98 | 0.97 |  |
| Mixed content |  | 1.00 | 0.97 |  |
| Camera-Captured content |  | 0.99 | 0.97 |  |
| All |  | 1.00 | 0.97 |  |

Table 2: proposed method tested on VTM 7.1 with RA configuration

|  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y |  |  | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% |  |  | 0.1% |  | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% |  |  | 0.1% |  | 0.1% |
| RGB, Animation, 720p | 0.0% |  |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% |  |  | 0.0% |  | 0.0% |
| TGM 1080p | -0.1% |  |  | 0.2% |  | 0.1% |
| TGM 720p | -0.2% |  |  | 0.3% |  | 0.3% |
| Animation | -0.1% |  |  | 0.0% |  | 0.2% |
| Mixed content | -0.4% |  |  | 0.4% |  | 0.4% |
| Camera-Captured content | 0.0% |  |  | 0.0% |  | 0.0% |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  |  | Time Ratio |  |  |
|  |  | Decoding | Encoding |  |
| RGB, text & graphics with motion, 1080p & 720p |  | 1.00 | 1.00 |  |
| RGB, mixed content, 1440p & 1080p |  | 0.98 | 1.00 |  |
| RGB, Animation, 720p |  | 1.02 | 1.00 |  |
| RGB, camera captured, 1080p |  | 0.99 | 1.00 |  |
| TGM 1080p |  | 0.99 | 0.99 |  |
| TGM 720p |  | 1.01 | 1.00 |  |
| Animation |  | 0.99 | 0.99 |  |
| Mixed content |  | 0.99 | 0.99 |  |
| Camera-Captured content |  | 0.99 | 0.99 |  |
| All |  | 1.00 | 0.99 |  |

# Proposed Spec Text

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_bdpcm_enabled_flag | u(1) |
| … |  |
| if( chromaArrayType = = 3 ) { |  |
| … |  |
| sps_act_enabled_flag | u(1) |
| if (sps_bdpcm_enabled_flag && !sps_act_enabled_flag) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Conclusion

This contribution proposes to signal ACT flag first and turn off BDPCM chroma when ACT flag is turned on in SPS level. The results show that 3% encoding time reduction in AI with proposed method. Since ACT and BDPCM chroma are both intra coding tools, the runtime reduction is mostly evident in AI configuration.

It is recommended to adopt this proposal to the next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team (JVET), JVET-P2001-vE, Geneva, Switzerland, 2019.

- VTM 7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-7.1

- X. Z. Xu, Y.-H. Chao, Y.-C. Sun and J. Z. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding ”, JVET-P2022-v3, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).