| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0426-v2 |
| --- | --- |

| Title: | AHG9: Picture header enabled flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian Davood Saffar | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com davood.saffar@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to make the picture header optional. The following elements are proposed:

- Add a new SPS syntax element, sps_picture_header_enabled_flag, that specifies whether picture headers are present or not. Either there is one picture header in each PU, or there is no picture header in any PU.

- Make the current picture header syntax elements present in the slice header when sps_picture_header_enabled_flag is equal to 0. The syntax elements that are present in either the picture header or the slice header are proposed to be put in a separate common syntax structure in the specification.

- Add a new slice header flag, first_vcl_nal_unit_in_picture_flag, conditioned on sps_picture_header_enabled_flag being equal to 0, as the first syntax element in the slice header.

- When sps_picture_header_enabled_flag is equal to 1, the first VCL NAL unit of a picture is the first VCL NAL unit that follows the PH NAL unit in decoding order of the picture. Otherwise, the first VCL NAL unit of a picture is the VCL NAL unit with first_vcl_nal_unit_in_picture_flag equal to 1. The proponents recommend that sps_picture_header_enabled_flag is set equal to 1 in use cases where bitstream manipulation such as bitstream extraction and merging is expected.

- Move mvd_l1_zero_flag from the picture header to the slice header since that is only relevant for the B slice type.

The proponents claim three benefits of the proposal:

Firstly, and in line with other VVC tools, the proposal enables the picture header feature to be disabled.

Secondly, each picture header uses a 2-byte NAL unit header and on average 4 bits for byte alignment. By allowing disabling the picture header, at least 20 bits per picture is estimated by the proponents to be saved when a single slice per picture is used which is equivalent to 1.2 kbps for a 60 fps sequence. This is claimed to be beneficial for low bitrate use cases, such as 100 kbps video.

Thirdly, the proposal is claimed to be beneficial for error resilience use cases, where a picture is partitioned into multiple slices, since the header data of a slice is then conveyed in a single NAL unit instead of being partitioned into one PH NAL unit and one VCL NAL unit.

A revised contribution containing complete specification text on top of JVET-Q0041-v2_DraftText.docx is attached with version 2 of this contribution.

# Introduction and Problem Statement

The picture header was adopted into VVC at the 16th JVET meeting in Geneva. The picture header was adopted as a mandatory feature instead of an optional one. This is not in line with other VVC tools that are possible to be disabled. Also, a mandatory picture header is inefficient for the following reasons:

- Each picture header uses a 2-byte NAL unit header and on average 4 bits for byte alignment. By allowing disabling the picture header, 20 bits per picture can be saved when a single slice per picture is used which is equivalent to 1.2 kbps for a 60 fps sequence. This is claimed to be beneficial for low bitrate use cases, such as 100 kbps video where 1 kbps is equal to a 1% overhead

- In error resilience use cases, where a picture is partitioned into multiple slices, it is anticipated that picture headers are repeated within a picture. This means that the header data of a slice is carried in two NAL units rather than one.

# Proposal

We propose to add an sps_picture_header_enabled_flag in the SPS. When the flag is equal to 1, there shall be one picture header NAL unit in each picture unit, preceding all VCL NAL units in decoding order. When the flag is equal to 0, there shall be no picture header NAL units present in the CLVS.

The syntax elements that are present in the picture header or slice header depending on the value of the sps_picture_header_enabled_flag are proposed to be put into a joint syntax structure called common_slice_header_struct(). The picture header always includes the struct. In the slice header, the presence of the struct is conditioned on sps_picture_header_enabled_flag being equal to 0.

# Proposed syntax and semantics

Proposed syntax and semantics relative to JVET-Q0041-v2_DraftText.docx is shown below. Only semantics for added syntax elements and syntax elements with significant changes are included. A revised contribution containing complete specification text on top of JVET-Q0041-v2_DraftText.docx is attached with version 2 of this contribution.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| sps_picture_header_enabled_flag | u(1) |
| ... |  |

sps_picture_header_enabled_flag equal to 1 specifies that one picture header NAL unit shall be present in all PUs in the CLVS. sps_picture_header_enabled_flag equal to 0 specifies that there shall be no picture header in the CLVS.

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| ... |  |
| no_picture_header_constraint_flag | u(1) |
| ... |  |
| } |  |

no_picture_header_constraint_flag equal to 1 specifies that sps_picture_header_enabled_flag shall be equal to 0.  no_picture_header_constraint_flag equal to 0 does not impose such a constraint.

Table 1 – Proposed picture header showing changes relative the picture header in JVET-Q0041-v2_DraftText.docx

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| ph_gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| common_slice_header_struct( 1 ) |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| if( !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| pic_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| if( pps_dep_quant_enabled_idc = = 0 ) |  |
| pic_dep_quant_enabled_flag | u(1) |
| if( !pic_dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

ph_gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. ph_gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When sps_picture_header_enabled_flag is equal to 0, the value of ph_gdr_pic_flag is inferred to be equal to 0.

Table 2 – Proposed common header showing changes relative the picture header in JVET-Q0041-v2_DraftText.docx

| picturecommon_slice_header_rbspstruct( pic_flag ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| if( pic_flag ¦ ¦ (nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ NalUnitType = = GDR_NUT ) ) gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ¦ ¦ (!pic_flag && nal_unit_type = = GDR_NUT ) ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| cph_poc_msb_present_flag | u(1) |
| if( cph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| if( !sps_virtual_boundaries_present_flag ) { |  |
| cph_virtual_boundaries_present_flag | u(1) |
| if( cph_virtual_boundaries_present_flag ) { |  |
| cph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < cph_num_ver_virtual_boundaries; i++ ) |  |
| cph_virtual_boundaries_pos_x[ i ] | u(13) |
| cph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < cph_num_hor_virtual_boundaries; i++ ) |  |
| cph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| chpic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| chpic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| chpic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| chpic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if(chpic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| chpic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| chpic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if(chpic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| chpic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| chpic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag && (pic_flag ¦ ¦ slice_type == I ) ) { |  |
| chpic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| chpic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if(chpic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| chpic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| chpic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| chpic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| chpic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chpic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| chpic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| if( sps_temporal_mvp_enabled_flag && (picFlag ¦ ¦ slice_type != I ) ) |  |
| chpic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| chpic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| chpic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| chpic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| chpic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| chpic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| chpic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| chpic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| chpic_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| chpic_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| if( pps_dep_quant_enabled_idc = = 0 ) |  |
| chpic_dep_quant_enabled_flag | u(1) |
| if( !chpic_dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| chpic_lmcs_enabled_flag | u(1) |
| if( chpic_lmcs_enabled_flag ) { |  |
| chpic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| chpic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| chpic_scaling_list_present_flag | u(1) |
| if( chpic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 3 – Proposed slice header showing changes relative the slice header in JVET-Q0041-v2_DraftText.docx

| slice_header( ) { | Descriptor |
| --- | --- |
| if( !sps_picture_header_enabled_flag ) |  |
| first_vcl_nal_unit_in_picture_flag | u(1) |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( !sps_picture_header_enabled_flag ) { |  |
| slice_pic_parameter_set_id | ue(v) |
| common_slice_header_struct( 0 ) |  |
| } |  |
| if( !pic_rpl_present_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| if( slice_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| slice_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( slice_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| slice_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( pic_rpl_present_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( slice_type != I ) { |  |
| if( slice_type = = B && !pps_mvd_l1_zero_idc ) |  |
| slice_mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( chpic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( sps_sao_enabled_flag && !pic_sao_enabled_present_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if( deblocking_filter_override_enabled_flag && !pic_deblocking_filter_override_present_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

first_vcl_nal_unit_in_picture_flag equal to 1 specifies that the VCL NAL unit containing the slice header is the first VCL NAL unit of the picture. first_vcl_nal_unit_in_picture_flag equal to 0 specifies that the VCL NAL unit containing the slice header is not the first VCL NAL unit of the picture.

slice_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of slice_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] and MvdL1[ x0 ][ y0 ][ cpIdx ][ compIdx ] are set equal to 0 for compIdx = 0..1 and cpIdx = 0..2. slice_mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed. When not present, the value of slice_mvd_l1_zero_flag is inferred to be equal to pps_mvd_l1_zero_idc − 1.

##### 7.4.2.4.3 Order of NAL units and coded pictures and their association to PUs and AUs

This clause specifies the order of NAL units and coded pictures and their association to PUs and AUs for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 9.

A PU consists of one PH NAL unit, one coded picture, which comprises of one or more VCL NAL units, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.4.

An AU consists of zero or one AU delimiter NAL unit and one or more PUs in increasing order of nuh_layer_id.

The first AU in the bitstream starts with the first NAL unit of the bitstream. There shall be at most one AU delimiter NAL unit in an AU.

When sps_picture_header_enabled_flag is equal to 1, Tthe first VCL NAL unit of a picture is the first VCL NAL unit that follows the PH NAL unit in decoding order of the picture. Otherwise (sps_picture_header_enabled_flag is equal to 0), the first VCL NAL unit of a picture is the VCL NAL unit with first_vcl_nal_unit_in_picture_flag equal to 1.

A VCL NAL unit is the first VCL NAL unit of an AU (and consequently the picture containing the first VCL NAL unit is the first picture of the AU) when the VCL NAL unit is the first VCL NAL unit of a picture and one or more the following conditions are true:

–	The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

–	The value of slice_pic_order_cnt_lsb of the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

–	PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

Let firstVclNalUnitInAu be the first VCL NAL unit of an AU. The first of any of the following NAL units preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, specifies the start of a new access unit:

–	AUD NAL unit (when present),

–	DPS NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit ( when present),

–	PPS NAL unit (when present),

–	Prefix APS NAL unit (when present),

–	PH NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL unit with nal_unit_type equal to RSV_NVCL_26 (when present),

–	NAL unit with nal_unit_type in the range of UNSPEC28..UNSPEC29 ( when present).

NOTE – The first NAL unit preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, can only be one of the above-listed NAL units.

The order of the coded pictures and non-VCL NAL units within a PU or an AU shall obey the following constraints:

–	When an AU delimiter NAL unit is present in an AU, it shall be the first NAL unit of the AU.

–	When a PH NAL unit is present in a PU, itThe PH NAL unit in a PU shall precede the first VCL NAL of the PU.

–	When any DPS NAL units, VPS NAL units, SPS NAL units, PPS NAL units, prefix APS NAL units, prefix SEI NAL units, NAL units with nal_unit_type equal to RSV_NVCL_26, or NAL units with nal_unit_type in the range of UNSPEC_28..UNSPEC_29 are present in a PU, they shall not follow the last VCL NAL unit of the PU.

–	When any DPS NAL units, VPS NAL units, SPS NAL units, or PPS NAL units are present in a PU, they shall precede all VCL NAL units in the PU and the PH NAL unit of the PU if it is present.

–	NAL units having nal_unit_type equal to SUFFIX_APS_NUT, SUFFIX_SEI_NUT, FD_NUT, or RSV_NVCL_27, or in the range of UNSPEC_30..UNSPEC_31 in a PU shall not precede the first VCL NAL unit of the PU.

–	When an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units with in the PU other than an EOB NAL unit (when present).

–	When an EOB NAL unit is present in an AU, it shall be the last NAL unit in the AU.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).