| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0443 |
| --- | --- |

| Title: | Independent Subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karl Sharman Steve Keating Adrian Browne | Email: | karl.sharman@sony.com steve.keating@sony.com adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

Subpictures allow regions of a picture to be independently decodable, with care added to the DIS (JVET-P2001-v14) to handle edges of subpictures and describe the subpicture requirements at different levels via a dedicated SEI message.

This contribution proposes that constrains are added to subpictures so that CABAC zero words and MinCR picture level constraints are also applied at a subpicture level. The proposal also highlights some potential errors in the current Subpicture SEI message.

# Introduction

Subpictures allow regions of a picture to be independently decodable, with care added to the DIS to handle edges of subpictures and describe the subpicture requirements at different levels via a dedicated SEI message.

However, two additional aspects have been identified that currently remove the independence of subpictures. It is proposed that these two aspects are modified.

# Proposal

The two aspects proposed to be modified are the CABAC zero word mechanism and the MinCR constraint. They are described in the next two sections.

## CABAC zero words

CABAC zero words (CZW) are added to a stream to constrain the CABAC bin decoding rate.

The current constraint is based upon picture level constraints, and do not constrain subpictures. This means that subpictures are not independent – it would not be possible to extract one subpicture (or combining with other subpictures)and put it into a new stream without risking exceeding the constraint. In addition, without entropy decoding the stream, it is not possible to determine whether the constraint fails.

Therefore it is proposed to place the CABAC zero word constraint on sub-pictures as well, with CABAC zero words added to one of the slices of the sub-picture, as required.

There are two methods to do this – either in the core of the standard, or in the existing Subpicture SEI message.

### CZW Method 1: Changes to the core

The changes are highlighted in the following excerpt of the DIS (JVET-P2001-v14).

7.4.3.13		RBSP slice trailing bits semantics

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNalUnitsOfSubPic be the sum of the values of NumBytesInNalUnit for all VCL NAL units of the j-th coded subpicture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of the j-th coded subpicture.

Let SubPicSizeInMinCbsY[ j ] be derived as follows:

SubPicSizeInMinCbsY[ j ] = (  ( subpic_width_minus1[ j ] + 1 )  / MinCbSizeY ) * ( ( subpic_height_minus1[ j ] + 1  )/ MinCbSizeY )

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY * ( BitDepth + 2 * BitDepth / ( SubWidthC * SubHeightC ) )				(89)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnitsOfSubPic + ( RawMinCuBits * SubPicSizeInMinCbsY[ j ] )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

### CZW Method 2: Changes to the Subpicture SEI message

Alternatively, the changes are highlighted in the following excerpt of the DIS (JVET-P2001-v14).

D.3.8 Subpicture level information SEI message semantics

…

Let NumBytesInVclNalUnitsOfSubPic be the sum of the values of NumBytesInNalUnit for all VCL NAL units of the j-th coded subpicture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of the j-th coded subpicture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY * ( BitDepth + 2 * BitDepth / ( SubWidthC * SubHeightC ) )

…

It is a requirement of bitstream conformance that the bitstreams resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * SubPicSizeY[ i ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

–	The value of Ceil( 256 * ( subpic_width_minus1[ i ] + 1 ) ÷ RefLevelFraction[ i ][ j] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( subpic_height_minus1[ i ] + 1 ) ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of SubPicNumTileCols[ j ] shall be less than or equal to MaxTileCols and of SubPicNumTileRows[ j ] shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

–	The value of BinCountsInNalUnits[ j ] shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnitsOfSubPic + ( RawMinCuBits * SubPicSizeInMinCbsY[ j ] )  32.

## MinCR

MinCR places a constraint on the minimum amount by which a picture must be compressed. This mechanism therefore constrains the number of bytes of a picture, and therefore places a limit on the CPB bandwidth.

The current constraint is on the picture, without concern for subpictures. Again, this means that subpictures are not independent, and that picking one subpicture (or combining it with other subpictures) and placing  in a new stream will not guarantee the MinCR constraint is met.

Again it is proposed that a MinCR constraint is placed at the subpicture-level, either in Annex A (profiles) or in the existing Subpicture SEI message.

### MinCR Method 1: Changes to Annex A

The changes are highlighted in the following excerpt of the DIS (JVET-P2001-v14).

A.4.2	Profile-specific level limits

…

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

For subpicture index j, the variable SubPicSizeInSamplesY[ j ] is set equal to ( subpic_width_minus1[ j ] + 1 ) * ( subpic_height_minus1[ j ] + 1 )

and SubPictureFraction [ j ] is set equal to SubPicSizeInSamplesY[ j ] ÷ PicSizeInSamplesY

…

- The sum of the NumBytesInNalUnit variables for access unit 0  for each subpicture j shall be less than or equal to FormatCapabilityFactor * ( Max( SubPicSizeInSamplesY[ j ], fR * MaxLumaSr * SubPictureFraction[ j ] ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * SubPictureFraction[ j ] ) ÷ MinCr for the value of SubPicSizeInSamplesY of picture 0, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to picture 0.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) for each subpicture j shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) * SubPictureFraction[ j ] ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to picture n.

### MinCR Method 2: Changes to the Subpicture SEI message

Alternatively, the changes are highlighted in the following excerpt of the DIS (JVET-P2001-v14).

D.3.8 Subpicture level information SEI message semantics

…

It is a requirement of bitstream conformance that the bitstreams resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * SubPicSizeY[ i ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

–	The value of Ceil( 256 * ( subpic_width_minus1[ i ] + 1 ) ÷ RefLevelFraction[ i ][ j] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( subpic_height_minus1[ i ] + 1 ) ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of SubPicNumTileCols[ j ] shall be less than or equal to MaxTileCols and of SubPicNumTileRows[ j ] shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

–	The sum of the NumBytesInNalUnit variables for access unit 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(SubPicSizeY[ i ], fR * MaxLumaSr * RefLevelFraction[ i ][ j ] ÷ 256) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * RefLevelFraction[ i ][ j ] ) ÷ (256 * MinCr) for the value of SubPicSizeInSamplesY of picture 0, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to picture 0, at level ref_level_idc[ i ], and MinCr is derived as indicated in A.4.2.

–	The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) * RefLevelFraction[ i ][ j ] ÷ (256 * MinCr), where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to picture n, at level ref_level_idc[ i ] , and MinCr is derived as indicated in A.4.2.

# Errors in Subpicture SEI message

The following are highlighted as potential errors in the DIS (JVET-P2001-v14).

D.3.8 Subpicture level information SEI message semantics

…

It is a requirement of bitstream conformance that the bitstreams resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * SubPicSizeY[ ij ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1 for level ref_level_idc[ i ].

–	The value of Ceil( 256 * ( subpic_width_minus1[ ij ] + 1 ) ÷ RefLevelFraction[ i ][ j] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( subpic_height_minus1[ ij ] + 1 ) ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of SubPicNumTileCols[ j ] shall be less than or equal to MaxTileCols and of SubPicNumTileRows[ j ] shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

# Conclusion

It is proposed that two picture-level constraints, those being on CABAC zero words and on MinCR, are extended so that corresponding sub-picture level constraints are also present.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).