| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0444 |
| --- | --- |

| Title: | Non-CE: Clean-up of High-Level Syntax Related to AMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Michel KERDRANAT Tangi POIRIER Antoine ROBERT Franck GALPIN Fabrice LE LEANNEC | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes some modification to VVC specification to have better control of AVMR. Currently, AMVR is controlled by the General Constraint Information of no_amvr_constraint_flag and the SPS flag of sps_amvr_enabled_flag. However, both these flags cannot completely deactivate AMVR as it can still be enabled with Affine if sps_affine_amvr_enabled_flag is true. This leads to a problem with profiling as AMVR cannot be fully constrained with its general constraint information flag.

# Introduction

In VVC, there exists several constraint flags to disable certain tools at different levels of coding. These flags can exist at decoder parameters set (DPS), Video Parameters Set (VPS) or Sequence Parameter Set (SPS). Among these flag, no_amvr_constraint_flag is identified to have an issue that it cannot disable AMVR completely. In fact, this flag can set sps_amvr_enabled_flag to zero, but not the Affine-AMVR SPS flags (sps_affine_amvr_enabled_flag) .

According to VVC specifications, sps_amvr_enabled_flag and sps_affine_amvr_enabled_flag are coded independently, where the later is coded only affine is enabled. That is, if sps_affine_enabled_flag is true, sps_affine_amvr_enabled_flag can be true regardless of sps_amvr_enabled_flag. The corresponding specification text is shown below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_amvr_enabled_flag | u(1) |
| … |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| … |  |
| } |  |

On the other hand, the general constraint information are utilized to define profiles that contain restricted coding tools. The constraint flag of no_amvr_constraint_flag is meant for profile that does not employ AMVR. However, according to the spec, it can deactivate only the general AMVR, but not Affine-AMVR:

7.4.4.2	General constraint information semantics

…

no_amvr_constraint_flag equal to 1 specifies that sps_amvr_enabled_flag shall be equal to 0. no_amvr_constraint_flag equal to 0 does not impose such a constraint.

…

That is, the problem occurs with the following settings:

- no_amvr_constraint_flag is true

- sps_affine_enabled_flag is true

- sps_affine_amvr_enabled_flag is true.

In this case, AMVR is partially deactivated as it remains active for AMVR. The contribution proposes two methods to solve this problem (Proposal 1 and Proposal 2).

# Proposal 1: Solving at general constraint information level

The Above mentioned problem can be solved by the following modification to the specification (added part is blue):

7.4.4.2	General constraint information semantics

…

no_amvr_constraint_flag equal to 1 specifies that sps_amvr_enabled_flag and sps_affine_amvr_enabled_flag shall be equal to 0. no_amvr_constraint_flag equal to 0 does not impose such a constraint.

…

That is, no_amvr_constraint_flag disables both regular AMVR and Affine-AMVR in this method. The advantage here is to keep the current flexibility of VVC encoder, where it can activate/deactivate AMVR for either regular or Affine blocks (or both). In other words, sps_amvr_enabled_flag and sps_affine_amvr_enabled_flag are kept independent.

# Proposal 2: Solving at SPS level

Instead of modifying the general constraints information syntax, the problem can be solved at SPS level. This is done by enabling the Affine-AMVR only if AMVR is enabled. That is, to code sps_affine_amvr_enabled_flag only if sps_amvr_enabled_flag is true. The corresponding specification modifications are given below (added part in blue):

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_amvr_enabled_flag | u(1) |
| … |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| if (sps_affine_enabled_flag){ |  |
| sps_affine_amvr_enabled_flag | u(1) |
| } |  |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_prof_pic_present_flag | u(1) |
| } |  |
| … |  |
| } |  |

This modification guarantees the deactivation of AMVR through the general constraint information flag since no_amvr_constraint_flag can set sps_amvr_enabled_flag to zero, which in turn sets sps_affine_amvr_enabled_flag to zero.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).