| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0451-v1 |
| --- | --- |

| Title: | On mipSizeId modification for 8x8 blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Haixin Wang, Xinwei Li, Yanzhuo Ma, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn hxwang_2@stu.xidian.edu.cn xinweili@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution change the mipSizeId of 8 × 8 blocks from 1 to 2. For a 8 × 8 block, the predicted samples can be directly derived by matrix multiplification and no upsampling operation is need.

The experimental results are as below:

For AI configuration: -0.03%,-0.01%, and 0.03%, with 100% EncT, 100% DecT;

For RA configuration: 0.00 %, -0.03%, and 0.08%, with 100% EncT, 100% DecT.

Introduction

In VVC, the variable mipSizeId is derived as follows:

- If both nTbW and nTbH are equal to 4, mipSizeId is set equal to 0.

- Otherwise, if either cbWidth or cbHeight is equal to 4, or both cbWidth and cbHeight are equal to 8, mipSizeId is set equal to 1.

- Otherwise, mipSizeId is set equal to 2.

Proposed Method

This contribution proposes to let mipSizeId be 2 for 8 × 8 MIP coded blocks.

The variable mipSizeId is derived as follows:

- If both nTbW and nTbH are equal to 4, mipSizeId is set equal to 0.

- Otherwise, if either cbWidth or cbHeight is equal to 4, or both cbWidth and cbHeight are equal to 8 mipSizeId is set equal to 1.

- Otherwise, mipSizeId is set equal to 2.

By using the proposed method, the predicted sampls of 8x8 blocks is indentical to the matrix-based intra prediction samples predMip[ x ][ y ]. In this way, the upsampling operation can be omitted for 8x8 MIP blocks.

Experimental results

The experiments are performed based on VTM7.0 software [2] under JVET common test condition [3]. Table 1 present a summary of the results.

Table 1. Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.10% | 0.14% | 100% | 100% |
| Class A2 | -0.03% | 0.02% | -0.04% | 99% | 100% |
| Class B | -0.04% | 0.06% | -0.06% | 99% | 100% |
| Class C | -0.02% | -0.05% | 0.02% | 100% | 102% |
| Class E | -0.02% | 0.00% | 0.15% | 99% | 99% |
| Overall | -0.03% | -0.01% | 0.03% | 100% | 100% |
| Class D | -0.04% | 0.08% | 0.05% | 100% | 99% |
| Class F | -0.01% | -0.01% | 0.13% | 99% | 98% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.15% | 0.03% | 100% | 100% |
| Class A2 | -0.04% | 0.06% | 0.06% | 100% | 100% |
| Class B | -0.01% | 0.13% | 0.26% | 100% | 100% |
| Class C | 0.00% | -0.23% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.08% | 100% | 100% |
| Class D | -0.04% | -0.22% | -0.08% | 100% | 99% |
| Class F | -0.02% | 0.06% | -0.01% | 99% | 100% |

Conclusions

In this contribution, the mipSizeId of 8x8 blocks is changed from 1 to 2. Experimental results show a small coding gain for AI configuration. Meanwhile, the upsampling operation can be skipped for 8x8 MIP blocks. We propose to adopt this modification into VVC.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[3] F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document  JVET-N1010, Geneva, Switzerland, March 2019

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).