| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0455-v2 |
| --- | --- |

| Title: | On CU-level BDOF enable condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan, Yi Xue, Yanzhuo Ma, Junyan Huo, Fuzheng Yang Yuanfang Yu Yang Liu | Email: | swan@nwpu.edu.cn xueyi@mail.nwpu.edu.cn yzma@mail.xidian.edu.cn jyhuo@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp. Ltd |  |  |

_____________________________

- Abstract

This contribution proposes a CU-level BDOF enable restriction based on the directions of forward and backward motion vectors. Besides the current BDOF enable condition in VVC,  it is proposed to add an enable restriction as sign(mvL0[ xSbIdx ][ ySbIdx ][0]) * sign(mvL1[ xSbIdx ][ ySbIdx ][0]) is not larger than zero or sign(mvL0[ xSbIdx ][ ySbIdx ][1]) * sign(mvL1[ xSbIdx ][ ySbIdx ][1]) is not larger than zero.

Experimental results report that the BD-rate changes on Y, Cb and Cr are 0.xx%, 0.xx% and 0.xx%, respectively.

Introduction

In the VVC draft 7 [1], a prediction refinement method based on optical flow model, namely bidirectional optical flow (BDOF), is applied to enhance the efficiency of conventional bi-direction motion compensation.

In VVC, the variable bdofFlag is introduced to indicate whether the BDOF is used in a CU level. It is only enabled under several pre-defined conditions.

Proposed method

In this contribution, it is proposed to disable BDOF if the two vectors have same non-zero signs on both horizontal and vertical components, i.e. the following inequations are true at the same time,

sign(mvL0[ xSbIdx ][ ySbIdx ][0]) * sign(mvL1[ xSbIdx ][ ySbIdx ][0]) >0,

sign(mvL0[ xSbIdx ][ ySbIdx ][1]) * sign(mvL1[ xSbIdx ][ ySbIdx ][1]) >0.

The proposed changes on top of JVET-P2001-vE are specified as below, with newly added text highlighted in yellow.

8.5.6 Decoding process for inter blocks

#### 8.5.6.1 General

This process is invoked when decoding a coding unit coded in inter prediction mode.

…

The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - pic_disable_bdof_flag is equal to 0.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic).

      - RefPicList[ 0 ][ refIdxL0 ] is a short-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is a short-term reference picture.

      - sign(mvL0[ xSbIdx ][ ySbIdx ][0]) * sign(mvL1[ xSbIdx ][ ySbIdx ][0]) is not larger than 0 or sign(mvL0[ xSbIdx ][ ySbIdx ][1]) * sign(mvL1[ xSbIdx ][ ySbIdx ][1]) is not larger than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - ciip_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbWidth is greater than or equal to 8.

      - cbHeight is greater than or equal to 8.

      - cbHeight * cbWidth is greater than or equal to 128.

      - RefPicIsScaled[ 0 ][ refIdxL0 ] is equal to 0 and RefPicIsScaled[ 1 ][ refIdxL1 ] is equal to 0.

      - cIdx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

…

Test Result

The proposed methods were tested on top of the VTM-7.0 software, the test results are shown in Table 1.  The overall average bd-rate performance is {%, %, %} for RA configurations.

Table 1. Simulation results of the proposed method

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Class B | 0.00% | 0.02% | 0.03% | 100% | 100.0% |
| Class C | 0.01% | -0.03% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.01% | 0.01% | 100% | 99% |
| Class F | -0.01% | -0.02% | -0.04% | 100% | 100% |

Conclusion

The proposed method adds a BDOF enable restriction by utilizing the sign of forward and backward motion vectors. The proposed method brings a small reduced decoding time. By adding this restriction, no coding efficiency loss is introduced compared to VTM7.0. It is recommended to consider this method into VVC.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] Han Huang, Chun-Chi Chen, Kevin, Joint solution for the reference picture conditions in DMVR and BDOF (JVET-P0089, JVET-P0191, JVET-P0311, JVET-P0415, JVET-P0546, JVET-P0598), 16th Meeting, Oct. 2019

[3] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[4] Xiaoyu Xiu, Yuwen He, Yan Ye, CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO), 12th Meeting, Oct. 2018

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).