| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 Jan. 2020 | Document: JVET-Q0456-v2 |
| --- | --- |

| Title: | Non-CE4: simplified LUT for GEO blending weights generation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma, Qihong Ran, Ruipeng Qiu, Menglin Zhang,, Junyan Huo, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | yzma@mail.xidian.edu.cn qhRan@stu.xidian.edu.cn rpqiu@stu.xidian.edu.cn mlzhang_1@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

In core experiment 4, the blending weights of each block size and partitioning mode are derived from a pre-defined weight mask for the GEO common base. During the process of calculating pre-defined weight mask, a weight look-up table (LUT) is used. In this contribution, it is proposed to simplify the weight LUT without performance loss.

With the proposed method under the CTC, the overall performance is no changed (BD-rate changes on Y, Cb and Cr are 0.00%, 0.00% and 0.00%, respectively) in RA and LDB compared with VTM 7.0_GEO_ common base.

Introduction

In the CE4 common base software, the calculation of weight masks for each GEO coded blocks on-the-fly was cancelled, the information of current block is derived from two sets of pre-defined masks, g_sampleWeightL[] for the blending weights derivation and g_motionMask[] for the masks of motion field storage.

The pre-defined masks g_sampleWeightL[] is calculated by the following equations:

| displacementX=φ | (1) |
| --- | --- |
| displacementY=displancementX+24≫2 % 2 | (2) |
| rho=DisdisplacementX≪8+DisdisplacementY≪8 | (3) |
| offset=1≪8-224≫1 | (4) |
| WeightIdx=(x+offset)≪1+1×DisdisplacementX+(y+offset)≪1+1×DisdisplacementY-rho | (5) |
| WeightIdxAbs=Clip30, 26,absWeightIdx | (6) |
| sampleWeightLxy=WeightIdx≤0 ?WedgeFilterWeightIdxAbs:8-WedgeFilterWeightIdxAbs | (7) |

where (x, y) represents the position of each luma sample in the pre-defined weights mask , Dis[] and WedgeFilter[] are two look-up tables as shown in Table 1 and Table 2, respectively.

The pre-defined masks g_motionMask[] is calculated by the following equations:

| motionOffset=3*DisdisplacementX+3*DisdisplacementY | (8) |
| --- | --- |
| MVoffset=1≪6-56≫1 | (9) |
| motionIdx=(x+MVoffset)≪3+1×DisdisplacementX+(y+MVoffset)≪3+1×DisdisplacementY-rho+motionOffset | (10) |
| motionMask[x][y]= absmotionIdx<32 ?2 : motion≤0 ? 0 :1 | (11) |

where (x, y) represents the position of each 4*4 luma block in the pre-defined motion mask, Dis[] is a look-up tables as shown in Table 1.

Table 1. The look-up table for Dis[]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 8 | 8 | 8 | 4 | 4 | 2 | 0 | -2 | -4 | -4 | -8 | -8 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | -8 | -8 | -8 | -4 | -4 | -2 | -0 | 2 | 4 | 4 | 8 | 8 |

Table 2. The look-up table for GeoFilter[]

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter [idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| Idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter [idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

Proposed methods

In this contribution, it is proposed to reduce or remove the weight LUT without performance loss.

- Reduce the length of the weight LUT (GeoFilter [])

For the pre-defined mask calculation, when weightIdx needs to be mapped to a GeoFilter using the look-up table shown in Table 2, the modification is to reduce the length of weight LUT from 27 to 14. To apply the reduced weight LUT, all the values in the angle LUT Dis[] need to be reduced respectively.

The proposed changes on top of JVET-P2001-vE are specified as below, with deleted text marked in red strike and newly added text highlighted in yellow.

Table 1. Modified look-up table for Dis[]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 4 | 4 | 4 | 2 | 2 | 1 | 0 | -1 | -2 | -2 | -4 | -4 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | -4 | -4 | -4 | -2 | -2 | -1 | -0 | 1 | 2 | 2 | 4 | 4 |

Table 2. The look-up table for GeoFilter[]

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter [idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| Idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter [idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

Table 2. Modified look-up table GeoFilter[]

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter[idx] | 4 | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 8 |

And the calculation of WeightIdxAbs need to modefiy as follow:

| WeightIdxAbs=Clip3(0,2613,absWeightIdx) | (6) |
| --- | --- |

At the same time , the motion mask should also be modified as follow:

| motionMask[x][y]= absmotionIdx<3216 ?2 : motion≤0 ? 0 :1 | (11) |
| --- | --- |

The reduced weight LUT has no impact on performance.

- Remove the weights LUT (GeoFilter [])

Another mathematically identical implementation of this contribution is to replace the reduced (14-length) weight LUT shown in above. By using the proposed conversion equation, the blending weight of all samples in a coding block could be performed on-the-fly in parallel without the process of looking-up table.

In this way, the proposed changes on top of JVET-P2001-vE are specified as below, with deleted text marked in red strike and newly added text highlighted in yellow.

Table 1. Modified look-up table for Dis[]

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 4 | 4 | 4 | 2 | 2 | 1 | 0 | -1 | -2 | -2 | -4 | -4 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | -4 | -4 | -4 | -2 | -2 | -1 | -0 | 1 | 2 | 2 | 4 | 4 |

- Table 2. The look-up table for GeoFilter[]

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter [idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| Idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter [idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

the calculations of pre-defined masks are modified as follows:

| WeightIdxAbs=Clip3(0,2613,absWeightIdx) | (6) |
| --- | --- |
| sampleWeightLxy=WeightIdx≤0 ?WeightIdxAbs+2+(WeightIdxAbs>8)≫2+4:4-WeightIdxAbs+2+(WeightIdxAbs>8)≫2 | (7) |
| motionMask[x][y]= absmotionIdx<3216 ?2 : motion≤0 ? 0 :1 | (11) |

WeightIdxAbs

WeightIdxAbs=Clip3(0,2613,absWeightIdx)

Experimental results

The proposed Simplification is implemented on top of CE4 common base software. The coding performance is provided in Table 5.

Anchor: VTM7.0(CE4) common base

Table 5. Experimental results of Test

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 commone base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |

Conclusion

This contribution presents a simplified method to reduce or remove the weight LUT in GEO blending weights generation. With the proposed method, the memory needed to store weights LUT can be reduced or removed, and the calculation can be implemented in a parallel way when LUT is removed. The change doesn’t introduce any performance change. It is proposed to adopt these simplifications to VVC along the GEO method.

References

- S. Esenlik, H. Gao, E. Alshina, A. M. Kotra, B. Wang, M. Bläser, J. Sauer “CE4 Common Base: Geometric inter prediction”, JVET-Q0079, Brussels, BE, 7–17 January 2020

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).