| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0467 |
| --- | --- |

| Title: | CE5-related: Simplified CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a simplified CCALF filter was proposed to reduce computational complexity and signaling cost. The proposed methods were implemented on top of CE5 anchor software, and simulations were conducted with common test conditions. It is reported that the proposed method 1 has following performance with 25% less multiplications:

- Over VTM-7, BD rate changes are 0.16%, -7.83%, -6.75% for AI, 0.13%, -9.92%, -9.65% for RA, 0.09%，-13.09%，-9.67% for LB, 0.15%, -12.84%, -9.61% for LP.

- Over CE5 Anchor, BD rate changes are -0.01%, 0.54%, 0.37% for AI, -0.01%, 0.29%, 0.25% for RA, -0.03%， -0.08%， -0.11% for LB, 0.01%, 0.10%, -0.30% for LP.

The proposed method 2 has following performance with 37.5% less multiplications:

- Over VTM-7, BD rate changes are 0.14%, -7.37%, -6.32% for AI, 0.14%, -9.59%, -9.13% for RA, 0.12%，-12.31%，-8.88% for LB, 0.13%, -12.28%, -8.76% for LP.

- Over CE5 Anchor, BD rate changes are -0.03%, 1.07%, 0.87% for AI, 0.00%, 0.70%, 0.83% for RA, 0.00%， 0.88%， 0.82% for LB, -0.01%, 0.77%, 0.73% for LP.

# Introduction

The CCALF was previously proposed in JVET-P1008. And it was implemented on top of VTM-7.0 to be used as CE5 anchor.

This method provides a CC-ALF design that has the following properties:

- Filter shape is a 3x4 diamond with 8 unique coefficients

- Filter coefficient dynamic range is limited to 6-bit signed

- Filter selection to performed at the CTU level with support for a maximum of 4 filters

- Symmetric line selection is used at virtual boundary (to better harmonize with ALF)

- Temporal layer coefficient buffers are not used (to better harmonize with the APS concept)

- Residual output is clipped to -2BitDepthC-1 to 2BitDepthC-1-1, inclusive.

An overview of the CC-ALF process is shown in Figure C-1.  As can be seen in the Figure, CC-ALF uses a linear filter to filter luma sample values and generate a residual correction for the chroma channels from the co-located filtered output.  An illustration of the filter shape is provided in Figure C-2.

Figure 1 - Placement of CC-ALF with respect to other loop filters

Figure 2 - Filter support in CE Anchor

# Proposed Methods

The current design of CCALF as in CE5 anchor has 8 unique coefficients with a 3x4 diamond shape. The computational complexity may be further reduced by reducing the number of coefficients. In such case, the signaling cost for the coefficients can also be reduced.

## Simplified CCALF Method 1

In method 1, a simplified version of the CCALF filter is proposed to have 6 unique coefficients with a 3x4 cross shape, as shown in Figure 3. Compared to 3x4 diamond shape, the bottom-left and bottom-right coefficients are removed, and 25% multiplications in CCALF is saved.

Figure 3 - 6-tap cross shape filter

## Simplified CCALF Method 2

Figure 4 - 5-tap cross shape filter

In method 2, a simplified version of the CCALF filter is proposed to have 5 unique coefficients with a 3x3 cross shape, as shown in Figure 4. Compared to 3x4 diamond shape, 37.5% multiplications in CCALF is saved.

# Experimental Results

The proposed methods were implemented based on CE5 anchor software. Simulations were conducted following JVET common test conditions. VTM-7.0 was used as anchor, and CE5 anchor software is used as a second anchor.

## Simulation results of Method 1

As shown in Table 3.1, it was observed that compared with VTM-7.0 anchor, the BD-rate change of the proposed method is 0.16%, -7.83%, -6.75% for AI with 100% Enc. time, 99% Dec. time, 0.13%, -9.92%, -9.65%  for RA with 100% Enc. time, 100% Dec. time, 0.09%，-13.09%，-9.67% for LB with 100% Enc. time, 103% Dec. time.

Using AHG13 YUV metrics, BD-rate of method 1 over VTM-7.0 is: AI -1.33%, RA -1.85%, LB -2.20%.

Compared with CE5 anchor, the BD-rate change of proposed method is are -0.01%, 0.54%, 0.37% for AI with 100% Enc. time, 95% Dec. time, -0.01%, 0.29%, 0.25% for RA with 100% Enc. time, 97% Dec time, -0.03%， -0.08%， -0.11% for LB, with 100% Enc. time, 98% Dec. time.

Using AHG13 YUV metrics, BD-rate of method 1 over CE5 anchor is: AI 0.08%, RA 0.05%, LB -0.05%.

Table 3.1 Simulation Results of simplified CCALF Method 1

|  | AI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | -4.92% | -5.20% | 100% | 102% | 0.00% | 0.04% | 0.12% | 100% | 99% |
| Class A2 | 0.11% | -7.50% | -11.75% | 100% | 102% | 0.00% | 0.50% | 0.60% | 100% | 97% |
| Class B | 0.14% | -12.75% | -10.10% | 101% | 99% | -0.01% | 0.61% | 0.57% | 101% | 96% |
| Class C | 0.16% | -3.54% | -3.22% | 100% | 97% | -0.03% | 0.40% | 0.39% | 101% | 93% |
| Class E | 0.22% | -8.60% | -2.45% | 101% | 97% | -0.02% | 1.13% | 0.00% | 101% | 94% |
| Overall | 0.16% | -7.83% | -6.75% | 100% | 99% | -0.01% | 0.54% | 0.37% | 100% | 95% |
| Class D | 0.08% | -2.93% | -1.74% | 100% | 100% | -0.01% | 0.50% | -0.02% | 100% | 98% |
| Class F | 0.13% | -4.91% | -4.98% | 100% | 100% | -0.03% | 1.07% | 1.19% | 100% | 95% |
|  | RA |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.91% | -6.28% | 100% | 102% | 0.03% | -0.19% | -0.19% | 99% | 99% |
| Class A2 | 0.11% | -10.34% | -15.19% | 100% | 103% | 0.00% | 0.73% | 1.02% | 99% | 98% |
| Class B | 0.17% | -16.97% | -13.50% | 100% | 98% | -0.01% | 0.47% | 0.14% | 100% | 95% |
| Class C | 0.09% | -3.79% | -3.19% | 100% | 99% | -0.04% | 0.11% | 0.15% | 100% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.13% | -9.92% | -9.65% | 100% | 100% | -0.01% | 0.29% | 0.25% | 100% | 97% |
| Class D | 0.08% | -3.65% | -2.64% | 100% | 99% | -0.02% | 0.27% | 0.29% | 100% | 97% |
| Class F | 0.11% | -3.22% | -3.20% | 100% | 100% | -0.05% | 0.75% | 0.68% | 100% | 96% |
|  | LB |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.15% | -19.52% | -17.01% | 100% | 104% | -0.02% | -0.23% | 0.52% | 99% | 100% |
| Class C | 0.09% | -5.12% | -3.86% | 100% | 103% | 0.00% | 0.16% | -0.07% | 100% | 96% |
| Class E | -0.01% | -13.01% | -5.18% | 100% | 101% | -0.11% | -0.17% | -1.20% | 101% | 97% |
| Overall | 0.09% | -13.09% | -9.67% | 100% | 103% | -0.03% | -0.08% | -0.11% | 100% | 98% |
| Class D | 0.07% | -4.85% | -3.45% | 101% | 101% | -0.01% | -0.30% | -0.40% | 100% | 97% |
| Class F | 0.04% | -7.02% | -5.07% | 100% | 102% | -0.05% | 1.21% | 1.57% | 101% | 98% |
|  | LP |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.20% | -18.95% | -17.26% | 100% | 104% | 0.01% | 0.22% | -0.35% | 99% | 98% |
| Class C | 0.15% | -4.76% | -3.86% | 100% | 102% | 0.03% | -0.02% | 0.04% | 100% | 97% |
| Class E | 0.05% | -13.43% | -4.52% | 101% | 101% | -0.02% | 0.05% | -0.66% | 101% | 96% |
| Overall | 0.15% | -12.84% | -9.61% | 100% | 103% | 0.01% | 0.10% | -0.30% | 100% | 97% |
| Class D | 0.09% | -5.40% | -2.38% | 100% | 103% | 0.05% | -1.62% | -0.43% | 100% | 100% |
| Class F | 0.10% | -6.24% | -4.70% | 100% | 102% | -0.04% | 1.36% | 1.55% | 99% | 95% |

## Simulation results of Method 2

As shown in Table 3.2, it was observed that compared with VTM-7.0 anchor, the BD-rate change of the proposed method is 0.14%, -7.37%, -6.32% for AI with 101% Enc. time, 103% Dec. time, 0.14%, -9.59%, -9.13% for RA with 101% Enc. time, 102% Dec. time, 0.12%，-12.31%，-8.88% for LB with 100% Enc. time, 101% Dec. time.

Using AHG13 YUV metrics, BD-rate of method 1 over VTM-7.0 is: AI -1.25%, RA -1.76%, LB -2.02%.

Compared with CE5 anchor, the BD-rate change of proposed method is -0.03%, 1.07%, 0.87% for AI with 101% Enc. time, 99% Dec. time, 0.00%, 0.70%, 0.83% for RA with 100% Enc. time, 99% Dec time, 0.00%，0.88%，0.82% for LB, with 100% Enc. time, 99% Dec. time.

Using AHG13 YUV metrics, BD-rate of method 1 over CE5 anchor is : AI 0.17%, RA 0.15%, LB 0.17%.

Table 3.2 Simulation Results of simplified CCALF Method 2

|  | AI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | -4.55% | -4.79% | 102% | 103% | -0.01% | 0.43% | 0.56% | 102% | 100% |
| Class A2 | 0.11% | -6.87% | -11.35% | 101% | 103% | 0.00% | 1.24% | 1.11% | 101% | 98% |
| Class B | 0.13% | -11.90% | -9.35% | 101% | 102% | -0.02% | 1.61% | 1.48% | 101% | 99% |
| Class C | 0.14% | -3.33% | -2.72% | 100% | 103% | -0.05% | 0.62% | 0.91% | 101% | 99% |
| Class E | 0.21% | -8.52% | -2.55% | 101% | 102% | -0.04% | 1.22% | -0.10% | 100% | 99% |
| Overall | 0.14% | -7.37% | -6.32% | 101% | 103% | -0.03% | 1.07% | 0.87% | 101% | 99% |
| Class D | 0.07% | -2.87% | -1.22% | 100% | 103% | -0.02% | 0.58% | 0.53% | 101% | 102% |
| Class F | 0.12% | -4.75% | -4.89% | 100% | 103% | -0.05% | 1.23% | 1.27% | 100% | 98% |
|  | RA |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | -5.57% | -5.48% | 100% | 102% | 0.02% | 0.16% | 0.67% | 100% | 99% |
| Class A2 | 0.12% | -9.83% | -15.01% | 100% | 103% | 0.00% | 1.37% | 1.24% | 100% | 98% |
| Class B | 0.18% | -16.61% | -12.98% | 101% | 101% | 0.00% | 0.97% | 0.78% | 101% | 99% |
| Class C | 0.12% | -3.63% | -2.64% | 101% | 102% | -0.01% | 0.27% | 0.72% | 101% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | -9.59% | -9.13% | 101% | 102% | 0.00% | 0.70% | 0.83% | 100% | 99% |
| Class D | 0.05% | -3.61% | -2.37% | 101% | 101% | -0.04% | 0.31% | 0.58% | 101% | 98% |
| Class F | 0.11% | -3.08% | -3.07% | 101% | 102% | -0.05% | 0.90% | 0.82% | 101% | 98% |
|  | LB |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.13% | -18.57% | -16.06% | 100% | 101% | -0.04% | 1.04% | 1.76% | 99% | 97% |
| Class C | 0.15% | -4.25% | -3.29% | 101% | 102% | 0.05% | 1.09% | 0.53% | 101% | 95% |
| Class E | 0.08% | -12.60% | -4.36% | 101% | 100% | -0.02% | 0.31% | -0.37% | 101% | 96% |
| Overall | 0.12% | -12.31% | -8.88% | 100% | 101% | 0.00% | 0.88% | 0.82% | 100% | 96% |
| Class D | 0.07% | -5.20% | -3.34% | 101% | 104% | -0.01% | -0.66% | -0.26% | 101% | 100% |
| Class F | 0.02% | -6.17% | -4.38% | 101% | 101% | -0.07% | 2.10% | 2.31% | 101% | 97% |
|  | LP |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.18% | -18.26% | -15.80% | 100% | 100% | -0.01% | 1.12% | 1.50% | 99% | 94% |
| Class C | 0.06% | -3.85% | -3.21% | 101% | 102% | -0.06% | 0.95% | 0.72% | 101% | 96% |
| Class E | 0.13% | -13.54% | -4.42% | 102% | 102% | 0.06% | -0.06% | -0.55% | 102% | 96% |
| Overall | 0.13% | -12.28% | -8.76% | 101% | 101% | -0.01% | 0.77% | 0.73% | 100% | 96% |
| Class D | 0.10% | -5.18% | -2.56% | 101% | 102% | 0.06% | -1.40% | -0.63% | 101% | 99% |
| Class F | 0.15% | -5.70% | -4.12% | 101% | 105% | 0.00% | 1.93% | 2.17% | 101% | 98% |

# Conclusion

It was observed that compared with VTM-7.0 anchor, the BD-rate gain of the CCALF as provided in CE5 common base was mostly maintained by applying proposed method 1 or method 2.

It is asserted that the number of multiplications of the CCALF filter is reduced by 25% and 37.5% respectively, and the signaling cost of the coefficients can be recued similarly.

It is proposed to adopt one of the proposed methods as part of CCALF design.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- C.-Y. Chen, A. Segall, “Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering”, Document of Joint Video Experts Team (JVET), JVET-P2025 Geneva, CH, Oct 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).