| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0470 |
| --- | --- |

| Title: | Conditional signaling of SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to conditionally signal SAO in SPS depending on the ALF usage at sequence level. The proposed methods were implemented on top of VTM-7.0, and simulations were conducted with common test conditions. It is reported that compared with VTM-7.0 anchor, the proposed method has overall performance as following.

- ALF on: AI 0.00%, RA 0.08%; LB 0.09%

- ALF off: AI 0.00%, RA 0.00%, LB 0.00%

# Introduction

Based on the test results on VTM-7.0, it was observed that when ALF is on, the BD-rate improvement brought by SAO is very minor, at 0.00% on AI, 0.08% on RA, and 0.09% on LB. And when ALF is turned off, SAO brings moderate BD-rate impact, at 0.32% on AI, 0.77% on RA, and 0.90% LB. Test results are summarized in Table 1 and Table 2. The results indicate that there’s large performance overlap on the SAO and ALF.

Table 1- SAO tool off result on VTM-7.0

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.22% | 0.12% | 100% | 97% |
| Class A2 | 0.02% | 0.30% | 0.17% | 100% | 97% |
| Class B | 0.02% | 0.05% | 0.17% | 100% | 99% |
| Class C | -0.02% | 0.28% | 0.38% | 100% | 99% |
| Class E | 0.03% | -0.08% | -0.05% | 100% | 99% |
| Overall | 0.00% | 0.15% | 0.17% | 100% | 98% |
| Class D | -0.01% | 0.18% | 0.21% | 99% | 100% |
| Class F | 0.36% | 0.35% | 0.50% | 100% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.13% | 0.22% | 100% | 99% |
| Class A2 | 0.18% | 0.35% | 0.29% | 100% | 98% |
| Class B | 0.11% | 0.21% | 0.52% | 100% | 100% |
| Class C | -0.01% | 0.12% | 0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.20% | 0.33% | 100% | 99% |
| Class D | -0.15% | -0.06% | -0.24% | 100% | 101% |
| Class F | 0.53% | 0.39% | 0.53% | 101% | 101% |
|  |  |  | LB |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | 0.59% | 1.45% | 100% | 99% |
| Class C | 0.06% | 0.23% | 0.83% | 101% | 100% |
| Class E | 0.00% | 0.14% | 0.19% | 100% | 100% |
| Overall | 0.09% | 0.36% | 0.93% | 100% | 99% |
| Class D | -0.10% | 0.50% | 0.29% | 101% | 101% |
| Class F | 0.71% | 0.45% | 1.28% | 101% | 101% |

Table 2 - Results of SAO tool off when ALF is off

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.58% | 1.87% | 99% | 95% |
| Class A2 | 0.48% | 0.58% | 0.45% | 100% | 96% |
| Class B | 0.28% | 0.21% | 0.43% | 99% | 96% |
| Class C | 0.31% | 0.57% | 0.90% | 98% | 90% |
| Class E | 0.38% | -0.05% | 0.01% | 98% | 89% |
| Overall | 0.32% | 0.37% | 0.71% | 99% | 93% |
| Class D | 0.22% | 0.32% | 0.46% | 98% | 93% |
| Class F | 1.00% | 0.77% | 1.04% | 98% | 90% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.35% | 0.39% | 0.50% | 98% | 98% |
| Class A2 | 1.41% | 1.25% | 0.98% | 98% | 97% |
| Class B | 1.00% | 0.90% | 0.77% | 99% | 98% |
| Class C | 0.33% | 0.46% | 0.83% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.77% | 0.75% | 0.77% | 98% | 97% |
| Class D | 0.02% | 0.18% | -0.06% | 99% | 97% |
| Class F | 1.34% | 0.97% | 1.26% | 99% | 96% |
|  |  |  | LB |  |  |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.93% | 1.84% | 3.08% | 100% | 97% |
| Class C | 0.89% | 2.25% | 4.78% | 99% | 100% |
| Class E | 0.87% | 0.83% | 0.46% | 100% | 99% |
| Overall | 0.90% | 1.72% | 2.99% | 100% | 98% |
| Class D | 0.23% | 0.94% | 0.93% | 100% | 98% |
| Class F | 2.31% | 3.46% | 4.14% | 100% | 98% |

# Proposed Method

Considering the performance overlap between SAO and ALF, and in order to reduce the in-loop filtering operations, it is proposed to conditionally enable SAO at sequence level according to the ALF flag in SPS. When ALF is signalled as enabled, the SAO flag in SPS is not signalled but inferred to be 0. When ALF is signalled as disabled, SAO flag in SPS is signalled.

# Experimental Results

The proposed method was implemented based on VTM-7.0. Simulations were conducted following JVET common test conditions. VTM-7.0 was used as anchor.

It was also tested with ALF off on both VTM-7.0 anchor and the proposed test.

As shown in Table 3.1, it was observed that the BD-rate change of proposed method is 0.00% on AI, and 0.08% on RA, 0.09% on LB, compared with VTM-7.0 anchor.

Table 3.1 Simulation Results with ALF on

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.22% | 0.12% | 101% | 97% |
| Class A2 | 0.02% | 0.30% | 0.17% | 100% | 96% |
| Class B | 0.02% | 0.05% | 0.17% | 100% | 98% |
| Class C | -0.03% | 0.27% | 0.37% | 100% | 99% |
| Class E | 0.03% | -0.08% | -0.05% | 101% | 98% |
| Overall | 0.00% | 0.15% | 0.17% | 100% | 98% |
| Class D | -0.01% | 0.18% | 0.21% | 100% | 100% |
| Class F | 0.36% | 0.35% | 0.50% | 100% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.13% | 0.22% | 99% | 98% |
| Class A2 | 0.18% | 0.35% | 0.29% | 99% | 97% |
| Class B | 0.11% | 0.21% | 0.52% | 101% | 99% |
| Class C | -0.01% | 0.12% | 0.19% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.20% | 0.33% | 100% | 99% |
| Class D | -0.15% | -0.06% | -0.24% | 101% | 101% |
| Class F | 0.53% | 0.39% | 0.53% | 101% | 101% |
|  |  |  | LB |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | 0.59% | 1.45% | 101% | 100% |
| Class C | 0.06% | 0.23% | 0.83% | 101% | 102% |
| Class E | 0.00% | 0.14% | 0.19% | 101% | 102% |
| Overall | 0.09% | 0.36% | 0.93% | 101% | 101% |
| Class D | -0.10% | 0.50% | 0.29% | 102% | 105% |
| Class F | 0.71% | 0.45% | 1.28% | 102% | 104% |

As shown in Table 3.2, it was observed that there’s no BD-rate change when ALF is disabled on both proposed software and on VTM-7.0 anchor. This is expected behavior.

Table 3.2 Simulation Results of ALF off

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 111% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 103% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 102% |
|  |  |  | LB |  |  |
|  |  |  | Over VTM-7.0 ALF off |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 103% |

# Spec. Text Change

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alf_enabled_flag | u(1) |
| if (!sps_alf_enabled_flag){ |  |
| sps_sao_enabled_flag | u(1) |
| } |  |
| … | u(1) |

# Conclusion

It was proposed to conditionally signal SAO at sequence level based on ALF usage setting. The performance change under CTC is minor, and it’s asserted that the in-loop filtering process is simplified.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).