Draft text changes for JVET-Q0472 (AHG15: Quantization matrix signalling) aspect #2 “Replacing the flag with a number”, based on JVET-Q0041-v2 (AHG2: Editorial input on VVC draft text), and on top of aspect #1.

_____________________________

[…]

      - 7.3.2.18	Scaling list data syntax

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if( id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| scaling_list_num_coef | ue(v) |
| nextCoef = 0 |  |
| if( id > 13 && scaling_list_num_coef > 0 ) { |  |
| scaling_list_dc_coef[ id − 14 ] | se(v) |
| nextCoef += scaling_list_dc_coef[ id − 14 ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if( !( id > 25 && x >= 4 && y >= 4 ) && i < scaling_list_num_coef ) { |  |
| scaling_list_delta_coef[ id ][ i ] | se(v) |
| nextCoef += scaling_list_delta_coef[ id ][ i ] |  |
| } |  |
| ScalingList[ id ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

[…]

      - 7.4.3.18	Scaling list data semantics

scaling_matrix_for_lfnst_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with LFNST. scaling_matrix_for_lfnst_disabled_flag equal to 0 specifies that the scaling matrices may apply to the blocks coded with LFNST.

scaling_list_pred_id_delta[ id ] specifies the reference scaling list used to derive the predicted scaling matrix ScalingMatrixPred[ id ]. When not present, the value of scaling_list_pred_id_delta[ id ] is inferred to be equal to 0. The value of scaling_list_pred_id_delta[ id ] shall be in the range of 0 to maxIdDelta with maxIdDelta derived depending on id as follows:

maxIdDelta = ( id < 2 ) ? id : ( ( id < 8 ) ? ( id − 2 ) : ( id − 8 ) )	(103)

The variables refId and matrixSize are derived as follows:

refId = 	id − scaling_list_pred_id_delta[ id ]		(104)

matrixSize = ( id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 )	(105)

The (matrixSize)x(matrixSize) array ScalingMatrixPred[ x ][ y ] with x = 0..matrixSize − 1, y = 0..matrixSize − 1 and the variable ScalingMatrixDCPred are derived as follows:

- When scaling_list_pred_id_delta[ id ] is equal to 0, all elements of ScalingMatrixPred are set equal to 16, and ScalingMatrixDCPred is set equal to 16.

- Otherwise, ScalingMatrixPred is set equal to ScalingMatrixRec[ refId ], and the following applies for ScalingMatrixDCPred:

- If refId is greater than 13, ScalingMatrixDCPred is set equal to ScalingMatrixDCRec[ refId − 14 ].

- Otherwise (refId is less than or equal to 13), ScalingMatrixDCPred is set equal to ScalingMatrixPred[ 0 ][ 0 ].

scaling_list_num_coef determines the number of scaling_list_delta_coef[ id ] elements that are present in the syntax. The value of scaling_list_num_coef shall be in the range of 0 to matrixSize * matrixSize, inclusive.

scaling_list_dc_coef[ id − 14 ] is used to derive the value of the variable ScalingMatrixDC[ id − 14 ] when id is greater than 13 as follows:

ScalingMatrixDCRec[ id − 14 ] = ( ScalingMatrixDCPred + scaling_list_dc_coef[ id − 14 ] ) & 256	(106)

When not present, the value of scaling_list_dc_coef[ id − 14 ] is inferred to be equal to 0. The value of scaling_list_dc_coef[ id – 14 ] shall be in the range of −128 to 127, inclusive. The value of ScalingMatrixDCRec[ id − 14 ]  shall be greater than 0.

scaling_list_delta_coef[ id ][ i ] specifies the difference between the current matrix coefficient ScalingList[ id ][ i ] and the previous matrix coefficient ScalingList[ id ][ i − 1 ]. The value of scaling_list_delta_coef[ id ][ i ] shall be in the range of −128 to 127, inclusive. When not present, the value of scaling_list_delta_coef[ id ][ i ] is inferred to be equal to 0.

The (matrixSize)x(matrixSize) array ScalingMatrixRec[ id ] is derived as follows:

ScalingMatrixRec[ id ][ x ][ y ] = ( ScalingMatrixPred[ x ][ y ] + ScalingList[ id ][ k ] ) & 256	(107)
	with k = 0..( matrixSize * matrixSize − 1 ),
		x = DiagScanOrder[ Log2( matrixSize ) ][ Log2( matrixSize ) ][ k ][ 0 ], and
		y = DiagScanOrder[ Log2( matrixSize ) ][ Log2( matrixSize ) ][ k ][ 1 ]

The value of ScalingMatrixRec[ id ][ x ][ y ] shall be greater than 0.

[…]