| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0483-v1 |
| --- | --- |

| Title: | On avoiding out of range motion vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen Andrew Segall | Tel: Email: | fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd, Sharp Labs of America |  |  |

_____________________________

# Abstract

It is asserted that in a corner case involving the use of the regular merge mode and the selection of a temporal merge candidate, a motion vector may exceed the signed 18-bit range. It is proposed to clip the motion vector in such case. This corner case occurs with displacements of more than 8000 samples.  Thus, it is not expected to occur under common test conditions and unlikely to occur in pictures with resolutions of 4K and below.

# Problem statement and proposal

With reference to JVET-Q0041, in section 8.5.2.12, a collocated motion vector is derived. In this derivation process, the motion buffer compression process in section 8.5.2.15 is invoked.

We observe that a motion vector with a value at the extreme of the signaled 18-bit range, i.e. (131071, 0), would be modified to have a value (131072, 0) using this process. When scaling is further applied, the scaled motion vector is then clipped to the −131072..131071 range (signed 18-bit range) in equation (614). However, when no such scaling is applied, the motion vector is used as is (see equation 611). If this motion vector is then selected, for example for a block using the regular merge mode, it is further stored for later spatial motion vector prediction. The resulting value, i.e. (131072, 0), cannot be represented using the signed 18-bit range and thus would require a 19th bit for storage. We believe this is undesirable and inconsistent with previously discussed design goals.

It is thus proposed to add a clipping operation in equation (611) in section 8.5.2.12 as follows (similar to the clipping in equation (614)):

mvLXCol = Clip3(−131072, 131071, mvCol)	(611)

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).