| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0484-v1 |
| --- | --- |

| Title: | AHG9: HLS control of chroma QP offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Brian Heng Peisong Chen Tim Hellman Minhua Zhou | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The cu_chroma_qp_offset_enabled_flag is located in the slice header in the current draft text.  This contribution contends that this syntax element was missing from the slice header during the P-meeting cycle and was only added later as a bug fix (JVET bug ticket #519).  Had it been in the slice header when the picture header concept was adopted at the Geneva October 2019 meeting, it probably would have been moved to either the picture header or the PPS.

This contribution also notes that the PPS syntax element cu_qp_delta_enabled_flag is used to control luma QP, and this PPS-level control is sufficient for luma QP. So for design consistency, this contribution proposes that the chroma QP offsets are designed with an equivalent level of functionality, using a single flag in the PPS.

If this flag were to be moved to the PPS, note that there is already PPS level control with the pps_cu_chroma_qp_offset_list_enabled_flag. This contribution asserts it is unnecessary to have both of these flags in the PPS, and the single pps_cu_chroma_qp_offset_list_enabled_flag would be sufficient for the purpose of enabling CU-level chroma QP offsets.

# Proposal

The cu_chroma_qp_offset_enabled_flag is currently in the slice header of the draft text [1] and specifies whether a chroma QP offset may be present in the transform unit and palette coding syntax.  This contribution contends that it is unnecessary to transmit this flag with every slice header especially when one considers that the similar flag to control the luma delta QP is signaled only in the PPS.

In fact, there is already a PPS syntax element (pps_cu_chroma_qp_offset_list_enabled_flag) which indicates if chroma QP offsets will be enabled. This contribution asserts it is unnecessary to have two flags to separately control this and they should effectively be merged into a single PPS flag.

Specifically, this contribution proposes merging these two syntax elements into the single pps_cu_chroma_qp_offset_list_enabled_flag for ease of understanding compared to the current draft text.  This would then be consistent with the PPS-level cu_qp_delta_enabled_flag syntax element which is used to control luma QP.

The detailed syntax and semantic changes compared to the current draft text [3] are provided as an attachment to this contribution and consist of:

- Removal of cu_chroma_qp_offset_enabled_flag from the slice header

- Replacing cu_chroma_qp_offset_enabled_flag with pps_cu_chroma_qp_offset_list_enabled_flag in the slice data

# References

[1] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).