| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0486-v1 |
| --- | --- |

| Title: | AHG8: Constraints on Scaling Window Offset Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Wade Wan Peisong Chen Brian Heng Minhua Zhou | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

At the Geneva October 2019 meeting, the concept of scaling windows was adopted for deriving the scaling ratios used for reference picture resampling (RPR).  This adoption enables RPR to be used with pictures that are not an exact multiple of Max( 8, MinCbSizeY ) luma samples.  This contribution asserts that the current definition inadvertently allows corner case configurations which can result in significant processing complexity for a decoder and conflicts with the original RPR design principle to add RPR functionality without significantly increasing the complexity burden compared to a decoder that does not support RPR.  Additional constraints on the offset parameters are proposed that would avoid these problematic configurations while still enabling the anticipated functionality of RPR.

# Introduction

The derivation of the scaling ratio for reference picture resampling (RPR) has been an active area of investigation since the issue of how picture sizes affect the derivation was identified in JVET-O0204 [1].  More specifically, the picture width or height may not exactly align to a multiple of Max( 8, MinCbSizeY ) luma samples which was a restriction adopted based on JVET-O0640 [2] to avoid 2x2/2x4/4x2 intra chroma blocks at picture edges.  In these situations, the region to be scaled for RPR is not the same as pic_width_in_luma_samples and pic_height_in_luma_samples.

JVET-P0590 [3] was adopted at the Geneva October 2019 meeting to address this issue by defining a scaling window (using left, right, top and bottom offsets to pic_width_in_luma_samples and pic_height_in_luma_samples) for each picture.  The horizontal and vertical scaling factors can then be computed for each reference picture by comparing the scaling windows of the current and reference pictures.  Figure 1 provides an example of reference picture resampling where:

- The scaling window of the current picture being decoded is the entire picture and of size (W/2)x(H/2)

- The scaling window of the reference picture is the entire picture and of size WxH

- Thus, 2:1 horizontal and vertical downscaling of reference blocks is needed to properly decode pixels in the current picture.  This means a decoder must fetch four times the area from a reference picture when generating a scaled prediction.  So for any given block, there are 4X the processing requirements (e.g. memory bandwidth) to decode that given block.

- However, note that across the entire picture, the processing requirements (e.g. memory bandwidth) to decode the picture in this example is equivalent to a picture of size WxH when reference picture resampling is not invoked because there are ¼ the number of pixels/blocks.

Figure 1: Example where the scaling window is the entire picture

This contribution asserts that the current draft text [4] inadvertently allows the definition of offsets larger than the values necessary to support picture widths or heights that do not exactly align to a multiple of Max( 8, MinCbSizeY ) luma samples.  More specifically, since there are no constraints on the offset values, it is possible to create configurations such as the example shown in Figure 2 where:

- The scaling window of the current picture being decoded is the size (W/2)x(H/2) but the entire picture is size WxH which is different than that the example in Figure 1.

- The scaling window of the reference picture is the entire picture and of size WxH.

- The scaling windows define the scaling ratios needed so 2:1 horizontal and vertical downscaling of reference blocks is also needed in this example to properly decode pixels in the current picture.  This means a decoder must fetch four times the area from a reference picture when generating a scaled prediction.

- The scaling window in the current picture is ¼ the size of the entire picture but it is important to note that the scaling process used for RPR is applied to the entire picture, i.e., processing is still required to be performed by the decoder to properly decode pixels outside the scaling window.  Therefore, decoding this picture would increase by 4X the processing requirements compared to decoding a picture of size WxH that did not use RPR.    This would be problematic for many decoders as memory bandwidth is typically a limited resource in implementations.

Figure 2: Example where the scaling window is a subset (1/4 the pixels) of the entire picture

# Proposal

This contribution contends the offsets should be defined to facilitate padding where the number of padded pixels is smaller than the size of a CU.  This would enable RPR to satisfy the main functionality identified as an issue in earlier meeting, i.e, support for picture sizes that are not exactly a multiple of Max (8, MinCbSizeY) luma samples.  This would also bound the worst case processing requirements and avoid configurations like the example shown above.

The offsets in the scaling window are currently defined in the PPS and coded with ue(v).  This contribution proposes to restrict the offsets to be no greater than Max (8, MinCbSizeY) in the semantics as follows:

Proposed changes in Section 7.4.3.4 (Picture parameter set RBSP semantics):

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. The values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset shall be in the range of 0 to Max (8, MinCbSizeY), inclusive.  When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The value of scaling_win_left_offset + scaling_win_right_offset shall be less than pic_width_in_luma_samples, and the value of scaling_win_top_offset + scaling_win_bottom_offset shall be less than pic_height_in_luma_samples.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −	(74)
			( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −	(75)
			( scaling_win_bottom_offset + scaling_win_top_offset )

# References

[1] J. Samuelsson, S. Deshpande, A. Segall, “AHG8: On Adaptive Resolution Change (ARC) High-Level Syntax (HLS)”, document JVET-O0204, JVET, Gothenburg, Sweden, July 2019.

[2] L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz, “CE3-related: On 2x2/2x4/4x2 chroma blocks at the corner of pictures”, document JVET-O0640, JVET, Gothenburg, Sweden, July 2019.

[3] V. Seregin, A. K. Ramasubramonian, M. Coban, M. Karczewicz, “AHG8: Scaling window for scaling ratio derivation”, document JVET-P0590, JVET, Geneva, Switzerland, October 2019.

[4] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).