| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0487-v2 |
| --- | --- |

| Title: | AHG8: RPR Scaling Window Issues |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Tim Hellman Brian Heng Peisong Chen | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

Contribution JVET-P0590 introduced a scaling window to Reference Picture Resampling (RPR).  This contribution contends that this addition caused four issues: (1) the present specification text disabling Temporal Motion Vector Prediction (TMVP) when RPR is enabled is ambiguous, and (2) the software and specification do not match for certain scaling window configurations, (3) scaling window offsets are not supported in DMVR, and (4) scaling window offsets can introduce unintended half-sample chroma shifts.  Fixes for these purported issues are proposed.

Version 2: After version 1 of this contribution was released, two additional issues were found with the draft text (Items 3 and 4 mentioned above).  This contribution has been modified in an attempt to fix these additional issues as well.

# Background

Four issues relating to RPR and scaling windows are discussed in this contribution:  the TMVP RPR constraint, a spec/code mismatch, lack of support for scaling window offsets in DMVR, and the introduction of unintended half-sample chroma shifts.  All of these issues arise when the scaling windows in the present and reference picture are different, but result in a 1:1 scale factor.

Contribution JVET-P0590 [1] introduced a per-picture scaling window, as shown in Figure 1.  This window is used to compute the scale factor between the current picture and its reference pictures.  Each reference can have its own scaling window, and they can all be different from the current picture’s scaling window.

Figure 1: Scaling Window Definition

When scaling windows are used, there are several ways to create 1:1 scaling factors. For example, in addition to the obvious case (same coded picture width and height, same scaling window offsets), one could have:

- Same coded picture width and height, same scaling window width and height, but different scaling window offsets (as shown in Figure 2).

- Completely different coded picture width and height, but same scaling window width/height.

Figure 2: Scaling factor equal to 1 with misaligned offsets.

## Issue 1: TMVP Constraint

Temporal Motion Vector Prediction (TMVP) needs to be disabled when the co-located reference picture differs in coded size and scale factor from the current picture, otherwise undefined temporal motion vectors could be referenced. Specifically, even if the scale factor is 1:1, if the coded picture sizes are not identical, TMVP can refer to regions outside the reference picture boundaries.

The constraint that controls this restriction is presently written in the semantics of the collocated_ref_idx syntax element in the slice header of the current draft text [2] as:

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture shall be the same and RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx  ] shall be equal to 0.

The term “resolutions” in the highlighted section is ambiguous here, as that term is not defined anywhere.  The important point is that the true (i.e. coded) width and height of the current and co-located pictures must be the same.  The suggested replacement language is:

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture values of pic_width_in_luma_samples and pic_height_in_luma_samples in the current picture and in the reference picture referred to by collocated_ref_idx shall be the same and RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx  ] shall be equal to 0.

## Issue 2: Code/Spec Mismatch when Scale Factor is 1

Since the scaling window in Figure 1 has offsets for each edge, it’s possible to create configurations in which the scaling ratios between the current and reference picture are equal to 1, yet the two scaling windows are different.  For example, in Figure 2 the two pictures have the same width and height, but one has an offset at the top and the other has it at the bottom.  As described earlier, this also results in a 1:1 scale factor.

The specification and code differ in how they handle this situation:

- Specification: In Section 8.5.6.3.1, RPR offsets and scaling is applied to all motion vectors, regardless of whether the scaling factor is 1:1 or not.  The 1:1 scale factor (i.e. 214) is treated the same as any other scale factor. Therefore, during this process any left_offset or top_offset will be applied accordingly, even if the scaling factor is 1:1.

- VTM 7.x Code: In the xPredInterBlkRPR( ) function, all RPR-related logic is only applied if the scaling factor is not 1:1.  Specifically:

const bool scaled = scalingRatio != SCALE_1X;

if( scaled )

{

“RPR prediction”

}

else

{

“standard prediction”

}

The standard inter-prediction process is the same as it was before RPR, and makes no adjustments for RPR left/top scaling window offsets. Therefore, when the scalingRatio is 1:1 above, any left/top offsets will be ignored and motion compensation will fail to match the specification text.

Logically it seems that the specification is correct, as the application of the offset shouldn’t depend on whether the scale factor is 1:1 or not.  Source code can be provided that corrects the source to match the specification.

## Issue 3: Scaling Window Offsets in DMVR

During the first release of this contribution, it was believed that the specification generally supported cases like the example shown in Figure 2 (1:1 scaling ratio with misaligned offsets). Upon further review, it has become apparent that several additional fixes, in both the specification and the software, would be required to support this properly, particularly for DMVR. For example, note that none of the motion vector refinement process (Section 8.5.3) uses the scaling window left or top offsets. Additionally, none of the bounding box derivation (Section 8.5.6.3.1) accounts for these offsets either. Since this use case was never intended and has not been well studied, it is possible that additional issues may be present as well, not just for DMVR.

This contribution asserts that there is no practical reason why this use case would be needed. Scaling windows were added to enable RPR to be used with pictures that are not an exact multiple of Max( 8, MinCbSizeY ) luma samples. With this purpose in mind, an encoder can choose whether it wants to pad from the top, or from the bottom. There is no practical reason to be switching between the two.  Therefore, this contribution asserts that it is not worth the extra complexity that would be required to fix the text/software in order to optimize coding efficiency for a use case that has no practical purpose.

When RPR scaling is in use several tools are currently disabled, either implicitly (DMVR, BDOF, PROF) or via bitstream conformance restrictions (TMVP). This contribution recommends extending this same behavior to the misaligned offset case shown in Figure 2.

Specifically, this contribution proposes to redefine the RefPicIsScaled condition as follows.

Current conditions:

RefPicIsScaled[ i ][ j ] = ( RefPicScale[ i ][ j ][ 0 ]  !=  ( 1   <<  14 ) )  | |
					        ( RefPicScale[ i ][ j ][ 1 ]  !=  ( 1   <<  14 ) )

Proposed change:

RefPicIsScaled[ i ][ j ] = ( pic_width_in_luma_samples != refPicWidth | |

pic_height_in_luma_samples != refPicHeight | |

scaling_win_left_offset != refScalingWinLeftOffset | |

scaling_win_right_offset != refScalingWinRightOffset | |

scaling_win_top_offset != refScalingWinTopOffset | |

scaling_win_bottom_offset != refScalingWinBottomOffset )

## where refPicWidth, refPicHeight, refScalingWinLeftOffset, refScalingWinRightOffset, refScalingWinTopOffset, and refScalingWinBottomOffset, are set equal to the values of pic_width_in_luma_samples,    pic_height_in_luma_samples, scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset, respectively, of the reference picture RefPicList[ i ][ j ].

If this text above is adopted, the proposed text for Issue 1 can also be simplified, because this RefPicIsScaled condition would include the same check for identical pic_width_in_luma_samples and pic_height_in_luma_samples. See attached draft text for details.

## Issue 4: Half-Sample Chroma Offsets

Unlike the conformance window, where the units depend on the chroma format (i.e. offsets are in units of SubWidthC / SubHeightC), the scaling window offsets are specified directly in units of luma samples. Therefore half-sample chroma offsets are possible (i.e. offsets having odd values for 4:2:0 format).

While attempting to fix the issues in the SW, as described above, it was also noted that half-sample chroma offsets for scaling windows leads to some unusual and likely undesirable results.

For example, note in the following text from Section 8.5.6.3.1 that the precision of the current picture’s left scaling offset (red) is treated with less precision than the reference picture’s left scaling offset (blue). Specifically, the current picture’s offset is immediately scaled down by SubWidthC, while the reference picture’s offset is first increased in precision by 10-bits.

“The variable fRefLeftOffset is set equal to ( scaling_win_left_offset  <<  10 ), where scaling_win_left_offset is the scaling_win_left_offset for the reference picture.

refxSbC = ( ( ( xSb − scaling_win_left_offset ) / SubWidthC  <<  5 ) + refMvLX[ 0 ] ) *
		scalingRatio[ 0 ] + addX

refxC = ( ( Sign( refxSbC ) * ( ( Abs( refxSbC ) + 256 )  >>  9 )
		+ xC * ( ( scalingRatio[ 0 ] + 8 )  >>  4 ) )   + fRefLeftOffset / SubWidthC + 16 )  >>  5”

For half-sample chroma left/top offsets, the operations defined above will lead to a half-sample shift in chroma references even when the current and reference picture’s scaling windows are identical.

Since all inter-prediction tools use the process described in the text above, all tools would need to support this unintentional half-sample chroma shift, even if those tools do not support RPR scaling.

This contribution asserts that allowing half-sample chroma left/top offsets is unnecessary for the intended purpose of supporting RPR with pictures that are not an exact multiple of Max( 8, MinCbSizeY ) luma samples. And, as described above, it seems that this usage is not well supported at this time. To avoid complicating the specification further for an unnecessary use case, it is recommended to specify the scaling window using the same units as the conformance window.

Specifically, specifying scaling_win_left_offset and scaling_win_right_offset in units of SubWidthC luma samples, and specifying scaling_win_top_offset and scaling_win_bottom_offset in units of SubHeightC luma samples.

Corresponding changes can be found in the draft text provided with this contribution.

# Conclusion

This proposal contends that four issues were introduced by the addition of the RPR scaling window.  Solutions are provided for all issues, so it therefore recommends that the draft text and source code changes be adopted.

# References

[1] V. Seregin, A Ramasubramonian, et al, “AHG8: Scaling window for scaling ratio derivation”, document JVET-P0590, JVET, Geneva, CH, October 2019.

[2] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).