| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0488-v1 |
| --- | --- |

| Title: | AHG9: Bounding redundant SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The current VVC draft text does not constrain the number of redundant SEI messages.  While it is recognized that a small number of redundant copies may be useful to improve the error resilience characteristics of a bitstream, allowing an unbounded number of redundant copies does not seem to have any practical functionality and increases the burden on decoder implementations.  This contribution proposes a restriction on the number of redundant SEI messages using a bitstream conformance requirement similar to the constraint in the HEVC/H.265 standard.

# Introduction

The current VVC draft text [1] allows for redundant copies of SEI messages with no constraint on the number of these copies. Redundant repetition may be useful for certain applications, but this contribution contends there is questionable functional benefit for allowing a large number of redundant copies.

JCTVC-L0325 [2] raised this issue during HEVC/H.265 standardization and stated that practical decoder implementation may allocate less than 1% of their cycles for processing of SEI messages.  Therefore, not constraining the amount of repetition would put an unnecessary performance burden on decoder implementations to allocate more resources to be fully conformant without a significant increase in functionality.

Bounding the maximum repetition rate to a small practical limit eliminates this problem without compromising on repeating SEI messages for error resilience purposes.  For these reasons, the number of repetitions of SEI messages was limited in the HEVC/H.265 standard [2].  More specifically, it was decided to limit SEI messages on a case-by-case basis with the default being 8 per picture but not having a general statement limiting all SEI messages “to handle potential confusion over scope and allow for unique cases” (as documented in [3]).  This can be seen in section D.3.1 (“General SEI payload semantics”) of the HEVC/H.265 standard which has the following clauses (text highlighted in yellow reflects the specific constraints applicable to this contribution):

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, and 135 to 148, inclusive.

NOTE 3 – SingleLayerSeiList consists of the payloadType values of the SEI messages specified in Annex D excluding 0 (buffering period), 1 (picture timing), 4 (user data registered by Recommendation ITU-T T.35), 5 (user data unregistered), 130 (decoding unit information) and 133 (scalable nesting). VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-nested and contained in an SEI NAL unit, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit. PicUnitRepConSeiList consists of the payloadType values of the SEI messages that are subject to the restriction on 8 repetitions per picture unit.

# Proposal

The table below lists:

- the payloadType and name of the SEI in the draft text

- whether there was the same (or similar) SEI in HEVC and whether that SEI was constrained to 8 per picture

- this contribution’s proposal on whether the VVC SEI should be constrained to 8 repetitions per picture.

| VVC payloadType | VVC SEI | HEVC payloadType | Constrained to 8 per picture in HEVC? | Propose to constrain to 8 per picture in VVC |
| --- | --- | --- | --- | --- |
| 0 | buffering_period | 0 | Yes | Yes |
| 1 | pic_timing | 1 | Yes | Yes |
| 3 | filler_payload | 3 | No | No |
| 4 | user_data_registered_itu_t_t35 | 4 | No | No |
| 5 | user_data_unregistered | 5 | No | No |
| 19 | film_grain_characteristics | 19 | Yes | Yes |
| 45 | frame_packing_arrangement | 45 | Yes | Yes |
| 129 | referenced_parameter_sets | 129* | Yes | Yes |
| 130 | decoding_unit_info | 130 | Yes (8 per decoding unit ) | Yes (8 per decoding unit) |
| 132 | decoded_picture_hash | 132 | Yes | Yes |
| 133 | scalable_nesting | 133 | Yes | Yes |
| 137 | mastering_display_colour_volume | 137 | Yes | Yes |
| 144 | content_light_level_info | 144 | Yes | Yes |
| 145 | dependent_rap_indication | 145 | Yes | Yes |
| 147 | alternative_transfer_characteristics | 147 | Yes | Yes |
| 148 | ambient_viewing_environment | 148 | Yes | Yes |
| 149 | content_colour_volume | NEW in VVC | N/A | Yes |
| 150 | equirectangular_projection | NEW in VVC | N/A | Yes |
| 153 | generalized_cubemap_projection | NEW in VVC | N/A | Yes |
| 154 | sphere_rotation | NEW in VVC | N/A | Yes |
| 155 | regionwise_packing | NEW in VVC | N/A | Yes |
| 156 | omni_viewport | NEW in VVC | N/A | Yes |
| 168 | frame_field_info | 168 | No** | Yes |
| 203 | subpic_level_info | NEW in VVC | N/A | Yes |
| 204 | sample_aspect_ratio_info | NEW in VVC | N/A | Yes |

* The HEVC active_parameter_sets SEI has the same payload type as the VVC reference_parameter_sets SEI and the authors believe it should be treated the same way in terms of repetitions per picture.

** The HEVC frame_field_info SEI was not constrained to 8 repetitions per picture.  The authors of this contribution do not know the background of why this SEI was not constrained or if this was an oversight.  Thus, this contribution suggests that the VVC frame_field_info SEI be constrained unless there is a compelling reason to allow repeating this SEI more than 8 times per picture.

Current draft text in D.3 (“SEI payload semantics”)

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 129, 132, 137, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 168, 203, and 204.

NOTE 4 – VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-scalable-nested and contained in an SEI NAL unit, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit.

Proposed changes to draft text in D.3 (“SEI payload semantics”)

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 129, 132, 137, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 168, 203, and 204.  The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 19, 45, 129, 132, 133, 137, 144, 145, 147, 148, 149, 150, 153, 154, 155, 156, 168, 203, 204.

NOTE 4 – VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-scalable-nested and contained in an SEI NAL unit, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit.  PicUnitRepConSeiList consists of the payloadType values of the SEI messages that are subject to the restriction on 8 repetitions per picture unit.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in SEI NAL units:

–	When an SEI NAL unit contains a non-scalable-nested buffering period SEI message, a non-scalable-nested picture timing SEI message, or a non-scalable-nested decoding unit information SEI message, the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

–	When an SEI NAL unit contains a scalable-nested buffering period SEI message, a scalable-nested picture timing SEI message, or a scalable-nested decoding unit information SEI message, the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0 (buffering period), 1 (picture timing), 130 (decoding unit information) or 133 (scalable nesting).

The following applies on the applicable OLSs or layers of non-scalable-nested SEI messages:

–	For a non-scalable-nested SEI message, when payloadType is equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the non-scalable-nested SEI message applies only to the 0-th OLS.

–	For a non-scalable-nested SEI message, when payloadType is equal to any value among VclAssociatedSeiList, the non-scalable-nested SEI message applies only to the layer for which the VCL NAL units have nuh_layer_id equal to the nuh_layer_id of the SEI NAL unit containing the SEI message.

It is a requirement of bitstream conformance that the following restrictions apply on the value of nuh_layer_id of SEI NAL units:

–	When a non-scalable-nested SEI message has payloadType equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information), the SEI NAL unit containing the non-scalable-nested SEI message shall have nuh_layer_id equal to vps_layer_id[ 0 ].

–	When a non-scalable-nested SEI message has payloadType equal to any value among VclAssociatedSeiList, the SEI NAL unit containing the non-scalable-nested SEI message shall have nuh_layer_id equal to the value of nuh_layer_id of the VCL NAL unit associated with the SEI NAL unit.

–	An SEI NAL unit containing a scalable nesting SEI message shall have nuh_layer_id equal to the lowest value of nuh_layer_id of all layers to which the scalable-nested SEI messages apply (when nesting_ols_flag of the scalable nesting SEI message is equal to 0) or the lowest value of nuh_layer_id of all layers in the OLSs to which the scalable-nested SEI message apply (when nesting_ols_flag of the scalable nesting SEI message is equal to 1).

It is a requirement of bitstream conformance that the following restrictions apply on repetition of SEI messages:

– For each of the payloadType values included in PicUnitRepConSeiList, there shall be less than or equal to 8 identical sei_payload( ) syntax structures within a picture unit.

– There shall be less than or equal to 8 identical sei_payload( ) syntax structures with payloadType equal to 130 within a decoding unit.

# References

[1] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041 JVET, Brussels, Belgium, January 2020.

[2] W. Wan, W. Ahmad, “Bounding redundant SEI messages”, document JCTVC-L0325, JCT-VC, Geneva, Switzerland, January 2013.

[3] G. Sullivan, J.-R. Ohm, “Meeting report of the 12th meeting of the Joint Collaborative Team on Video Coding (JCT-VC)”, document JCTVC-L_Notes_dE, JCT-VC, Geneva, Switzerland, January 2013.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).