| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0491-v2 |
| --- | --- |

| Title: | CE2-related: Palette escape binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Tsung-Chuan Ma, Yi-Wen Chen, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, tsung-chuanma, yiwenchen, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is derived by invoking the fifth order Exp-Golomb binarization process. The proposed method is implemented in VTM7 and evaluated based on CE2 [1] test conditions. Simulation results are:

Dual tree OFF:

Standard QP (22, 27, 32, 37): -0.02% AI, 0.00% RA, -0.03% LB

Low QP (2, 7, 12, 17): -0.62% AI, -0.36% RA, -0.24% LB

Dual tree ON:

Standard QP (22, 27, 32, 37): -0.04% AI, 0.03% RA, 0.02% LB

Low QP (2, 7, 12, 17): -0.34% AI, -0.22% RA, -0.21% LB

# Introduction

In current VVC [3], the palette coding is beneficial when there are limited pixels and the texture is homogeneous in a CU when the coded image is computer generated. If there is a pixel which has different values from other pixels, it is coded not as a regular palette entry, but an escape value. The sample values of each component of the escape value are coded and transmitted after quantized. Each escape sample value is binarized with EG3, which has the bin length from 4 to 18 when the sample value is 0 to (1 << bitdepth) – 1, where the bitdepth in the current VTM is set to 10. Then the binarized symbol is bypass coded. In the worst case, the escape value is coded with 54 bits, whereas it needs only 30 bits when the pixel is coded with FL coding. Therefore, the efficient escape value coding is provided in this proposal.

# Proposal

In this proposal, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is derived by invoking the fifth order Exp-Golomb binarization process.

# Simulation Results

The proposed methods were implemented based on the VTM-7.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CE2 [1] are applied.

Table 1: Results for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.11% | -0.05% | -0.05% | 100% | 100% | -0.07% | 0.18% | 0.15% | 101% | 100% |
| TGM | 0.00% | -0.12% | -0.05% | 101% | 101% | -0.01% | 0.00% | 0.01% | 100% | 99% |
| A | -0.03% | -0.03% | 0.02% | 100% | 100% | 0.00% | 0.05% | 0.01% | 100% | 100% |
| M | -0.03% | 0.02% | 0.00% | 100% | 100% | 0.02% | 0.12% | 0.14% | 100% | 100% |
| CC | 0.00% | -0.01% | 0.01% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.04% | -0.05% | -0.02% | 100% | 100% | -0.02% | 0.08% | 0.07% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | 0.03% | 0.04% | 0.05% | 100% | 100% | -0.06% | 0.33% | 0.27% | 100% | 101% |
| TGM | -0.01% | -0.03% | 0.03% | 100% | 99% | -0.01% | 0.08% | 0.15% | 101% | 97% |
| A | 0.02% | -0.05% | -0.01% | 100% | 101% | 0.04% | 0.05% | 0.08% | 99% | 97% |
| M | 0.10% | 0.28% | 0.38% | 100% | 98% | 0.04% | 0.28% | 0.33% | 103% | 100% |
| CC | 0.00% | 0.03% | 0.06% | 99% | 99% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.03% | 0.05% | 0.10% | 100% | 99% | 0.00% | 0.17% | 0.18% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.01% | -0.04% | -0.03% | 100% | 100% | 0.02% | 0.15% | 0.19% | 99% | 101% |
| TGM | 0.06% | 0.14% | 0.16% | 101% | 101% | -0.27% | -0.02% | 0.01% | 100% | 99% |
| A | 0.05% | 0.08% | -0.09% | 100% | 100% | -0.02% | 0.09% | -0.11% | 102% | 100% |
| M | 0.00% | -0.03% | 0.17% | 99% | 101% | 0.21% | 0.03% | 0.57% | 99% | 99% |
| CC | 0.01% | -0.08% | 0.04% | 99% | 102% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.02% | 0.02% | 0.05% | 100% | 101% | -0.03% | 0.06% | 0.14% | 100% | 100% |

Table 2: Results for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.78% | -0.58% | -0.63% | 100% | 99% | -1.36% | -1.62% | -1.54% | 101% | 99% |
| TGM | -0.18% | -0.19% | -0.22% | 100% | 100% | -0.46% | -0.18% | -0.57% | 100% | 98% |
| A | -0.20% | -0.16% | -0.14% | 98% | 97% | -0.25% | -0.35% | -0.44% | 99% | 97% |
| M | -0.36% | -0.40% | -0.40% | 98% | 100% | -0.64% | -0.71% | -0.70% | 99% | 101% |
| CC | 0.00% | 0.01% | 0.00% | 100% | 100% | 0.00% | 0.00% | -0.01% | 101% | 99% |
| Overall | -0.34% | -0.30% | -0.31% | 99% | 99% | -0.62% | -0.65% | -0.74% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.66% | -0.64% | -0.63% | 100% | 100% | -0.90% | -1.09% | -0.96% | 99% | 98% |
| TGM | -0.04% | 0.02% | 0.00% | 100% | 100% | -0.23% | -0.25% | -0.32% | 99% | 100% |
| A | -0.03% | -0.18% | -0.21% | 100% | 98% | -0.17% | -0.13% | -0.24% | 101% | 100% |
| M | -0.21% | 0.18% | 0.10% | 98% | 98% | -0.21% | -0.13% | -0.07% | 100% | 100% |
| CC | -0.02% | 0.00% | 0.03% | 101% | 100% | -0.01% | 0.01% | 0.00% | 101% | 100% |
| Overall | -0.22% | -0.15% | -0.18% | 100% | 99% | -0.36% | -0.38% | -0.38% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.48% | -0.45% | -0.43% | 98% | 100% | -0.67% | -0.72% | -0.73% | 99% | 99% |
| TGM | -0.08% | -0.04% | 0.00% | 101% | 100% | -0.04% | -0.07% | -0.08% | 102% | 99% |
| A | -0.36% | -0.18% | -0.18% | 101% | 100% | -0.16% | -0.25% | -0.27% | 102% | 102% |
| M | -0.02% | 0.31% | 0.29% | 99% | 101% | -0.17% | -0.07% | -0.07% | 99% | 100% |
| CC | 0.01% | 0.02% | 0.02% | 102% | 100% | 0.01% | 0.00% | 0.01% | 102% | 98% |
| Overall | -0.21% | -0.10% | -0.09% | 100% | 100% | -0.24% | -0.26% | -0.27% | 101% | 99% |

# Working Draft Text

The changes in the VVC working draft [3] is highlight below.

Table 123 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |

…

| palette_coding( ) | palette_predictor_run | EG0 | - |
| --- | --- | --- | --- |
|  | num_signalled_palette_entries | EG0 | - |
|  | new_palette_entries | FL | cMax = ( 1 << BitDepth ) − 1 |
|  | palette_escape_val_present_flag | FL | cMax = 1 |
|  | palette_idx_idc | 9.3.3.13 | MaxPaletteIndex |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | copy_above_palette_indices_flag | FL | cMax = 1 |
|  | run_copy_flag | FL | cMax = 1 |
|  | palette_escape_val | EG35 |  |

# Conclusions

This contribution proposed EG5, instead of the EG3 in the current VVC, as the binarization codeword for palette escape coding. The simulation results show that the proposed method improves coding efficiency, especially for the low QP cases, and has no extra coding complexity. It is recommended the proposed changes be adopted into VVC.

# References

- X. Xu, Y.-H. Chao, Y.-C. Sun and J. Xu, “ Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).