| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0497-v2 |
| --- | --- |

| Title: | Simplification of Rice parameter derivation for RRC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In residual coding of a transform block, for each coefficient, the remaining absolute level is binarized using Golomb-Rice code whose rice parameter is adaptively derived depending on the neighbour coefficient levels and a lookup table. This contribution proposes two methods for deriving the rice parameters without using lookup table. The proposed method 1 reportedly provides 0.01%, 0.00%, and 0.00% luma BD-rate impact for the AI, RA, and LB settings respectively versus the VTM-7.0 anchor under the common test condition (CTC), and proposed method 2 reportedly provides 0.01%, -0.01%, and 0.01% luma BD-rate impact for the AI, RA, and LB settings respectively versus the VTM-7.0 anchor under the common test condition (CTC).

# Introduction

In current VVC [1], each transform block is allowed to consume up to 1.75 regular bins per coefficient for entropy coding transform coefficient levels in order to constrain the wosrt-case bitstream parsing throughput rate. The four syntax elements sig_coeff_flag, abs_level_gt1_flag, par_level_flag and abs_level_gt3_flag in the 1st subblock coding pass are entropy coded in the regular mode. When the remaining regular bin budget is less than 4 right before coding a next coefficient level, the 1st subblock coding pass terminates immediately. The absolute value of each of the remaining uncoded coefficients after the termination of the 1st subblock coding pass is represented in the bypass mode by the syntax element dec_abs_level, which corresponds to a modified absolute level value with the zero level value conditionally being mapped to a nonzero codeword value.

The remainder of the absolute levels are adaptively binarized using the rice parameters derived depending on the sum of neighbour residual levels and lookup table. Given the sum of neighbour residual levels, locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 1.

Table 1. Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

# Proposal

This contribution proposes a simplied method for deriving the rice parameters. In the proposed method, the variable cRiceParam is determined as follows:

cRiceParam = max(0, min(3, (locSumAbs + offset) >> 3))

With the proposed simplification, the lookup table as Table 1 can be removed.

Test 1: the offset is set to 0.

Test 2: the offset is set to 1.

# Simulation Results

The proposed methods were implemented based on the VTM-7.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CTC are applied. The detailed results can be found in the attached excels.

## Test 1

Table 2: Results for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.14% | 0.07% | 100% | 99% |
| Class A2 | 0.00% | -0.02% | -0.03% | 101% | 101% |
| Class B | 0.01% | 0.03% | -0.01% | 100% | 98% |
| Class C | 0.01% | 0.01% | 0.08% | 99% | 100% |
| Class E | 0.01% | -0.09% | -0.02% | 101% | 98% |
| Overall | 0.01% | -0.03% | 0.02% | 100% | 99% |
| Class D | 0.02% | -0.01% | -0.01% | 99% | 100% |
| Class F | 0.01% | 0.03% | 0.03% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.03% | 100% | 100% |
| Class A2 | -0.01% | 0.07% | 0.09% | 99% | 100% |
| Class B | 0.01% | 0.06% | 0.10% | 99% | 97% |
| Class C | 0.01% | 0.09% | -0.09% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.03% | 100% | 99% |
| Class D | 0.03% | -0.21% | -0.05% | 100% | 97% |
| Class F | 0.00% | 0.10% | -0.08% | 100% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.39% | -0.08% | 99% | 98% |
| Class C | 0.03% | -0.20% | -0.05% | 100% | 98% |
| Class E | -0.02% | 0.52% | 0.24% | 101% | 97% |
| Overall | 0.00% | -0.10% | 0.01% | 100% | 98% |
| Class D | 0.02% | 0.31% | -0.39% | 99% | 98% |
| Class F | 0.03% | -0.30% | 0.53% | 97% | 101% |

Table 3: Results for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | -0.01% | 100% | 99% |
| Class A2 | 0.02% | 0.00% | -0.01% | 100% | 100% |
| Class B | 0.08% | 0.07% | 0.09% | 100% | 101% |
| Class C | 0.17% | 0.15% | 0.18% | 100% | 101% |
| Class E | 0.06% | 0.07% | 0.02% | 100% | 99% |
| Overall | 0.08% | 0.07% | 0.06% | 100% | 100% |
| Class D | 0.19% | 0.20% | 0.16% | 100% | 100% |
| Class F | 0.09% | 0.09% | 0.12% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.06% | -0.06% | 100% | 100% |
| Class A2 | -0.11% | -0.18% | -0.17% | 99% | 101% |
| Class B | -0.05% | -0.06% | -0.09% | 100% | 101% |
| Class C | 0.01% | -0.05% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.08% | -0.09% | 100% | 100% |
| Class D | -0.01% | -0.04% | -0.05% | 100% | 100% |
| Class F | 0.01% | 0.15% | 0.11% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.09% | -0.09% | 100% | 100% |
| Class C | -0.04% | -0.07% | -0.10% | 100% | 101% |
| Class E | -0.03% | 0.00% | -0.03% | 100% | 101% |
| Overall | -0.04% | -0.06% | -0.08% | 100% | 100% |
| Class D | -0.05% | -0.03% | -0.05% | 100% | 100% |
| Class F | 0.09% | 0.16% | 0.01% | 100% | 100% |

## Test 2

Table 4: Results for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | 0.06% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | -0.03% | 99% | 99% |
| Class B | 0.01% | 0.02% | -0.11% | 100% | 100% |
| Class C | 0.01% | 0.03% | 0.00% | 100% | 99% |
| Class E | 0.00% | -0.03% | 0.00% | 100% | 99% |
| Overall | 0.01% | -0.02% | -0.03% | 100% | 100% |
| Class D | -0.02% | 0.03% | 0.07% | 99% | 99% |
| Class F | -0.02% | 0.03% | 0.06% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.04% | 100% | 100% |
| Class A2 | -0.04% | -0.06% | -0.04% | 101% | 100% |
| Class B | 0.00% | 0.12% | 0.05% | 100% | 100% |
| Class C | 0.01% | -0.11% | 0.02% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.02% | 100% | 100% |
| Class D | -0.02% | -0.06% | 0.03% | 100% | 100% |
| Class F | -0.04% | 0.09% | 0.03% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.19% | 0.16% | 100% | 101% |
| Class C | 0.01% | -0.51% | -0.16% | 100% | 100% |
| Class E | -0.01% | 0.31% | -1.00% | 100% | 100% |
| Overall | 0.01% | -0.17% | -0.24% | 100% | 100% |
| Class D | -0.05% | -0.07% | -0.51% | 101% | 100% |
| Class F | -0.03% | -0.67% | 0.36% | 100% | 100% |

Table 5: Results for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.01% | 0.02% | 100% | 100% |
| Class B | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class C | 0.03% | 0.03% | 0.03% | 100% | 100% |
| Class E | -0.03% | 0.02% | -0.01% | 100% | 101% |
| Overall | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class D | 0.04% | 0.02% | 0.02% | 100% | 100% |
| Class F | -0.01% | 0.00% | 0.01% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 99% |
| Class C | 0.01% | 0.01% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.06% | 0.04% | 100% | 100% |
| Class F | 0.05% | -0.05% | -0.05% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.02% | 0.03% | 0.02% | 101% | 100% |
| Class E | -0.01% | 0.05% | -0.01% | 100% | 99% |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.16% | 99% | 98% |
| Class F | 0.00% | 0.00% | -0.15% | 100% | 99% |

# Working Draft Text

The changes in the VVC working draft [1] is highlight below.

## Test 1

9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]…

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(1494)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as follows:specified in Error! Reference source not found..

cRiceParam = (locSumAbs >> 3)

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ?  1 : 2 )  <<  cRiceParam

Table 124 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

## Test 2

9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]…

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

locSumAbs = 01
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(1494)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as follows:specified in Error! Reference source not found..

cRiceParam = (locSumAbs >> 3)

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ?  1 : 2 )  <<  cRiceParam

Table 124 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

# References

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).