| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0500 |
| --- | --- |

| Title: | AHG16: On derivation of CCLM predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the current CCLM design, different filters are used to generate the down-sampled luma samples depending on whether the luma samples are available or not, which is not friendly for certain parallel processing such as single instruction multiple data (SIMD) operation. In this proposal, it is proposed to perform repetitive padding for those unavailable luma samples, and apply the same 6-taps filter to generate all the down-sampled luma samples. The results show 0.00% (AI), 0.01%(RA) and 0.00%(LDB) BD-rate impact with negligible run time changes under CTC. It is also asserted that with the proposed method, the VVC spec can be reduced by around 100 lines in specifying the decoding process of CCLM mode.

# Introduction

In the CCLM modes of current VVC, because some luma samples are not available when generating the down-sampled luma samples, different filters are used to generate the down-sampled luma samples. For example, when left reference samples are not available, 2-tap down-sampling filter is used to generate the down-sampled luma samples located at the leftmost position of each row of luma samples. But 6-tap down-sampling filter is used to generate the down-sampled luma samples located at the other locations. Different filters used for down-sampling becomes an obstacle of generating the down-sampled luma samples in parallel such as single instruction multiple data (SIMD) operations.

# Proposal

In the proposed method, samples padding is applied to generate those unavailable samples used by the down-sampling process. With the padding process, the same filter can be used to generate the down-sampled luma samples regardless their position. Moreover, with the proposed simplification, the section to describe the down-sampling process for CCLM modes in VVC spec can be simplified. As shown in the following section, around 100 lines of spec (highlighted in green color) to handle different conditions of unavailable luma samples can be removed with 4 lines added (highlighted in yellow color) to perform the padding process.

Figure 1 Proposed sample padding. The square blocks represent the original luma samples, the black circles represent the down-sampled luma samples and the gray squares represent the padded luma samples.

# Proposed spec changes

##### 8.4.5.12.13 Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

…

…

…

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If both numSampL and numSampT are equal to 0, the following applies:

predSamples[ x ][ y ] = 1  <<  ( BitDepth − 1 )		(354)

- Otherwise, the following ordered steps apply:

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * SubWidthC − 1, y= 0..nTbH * SubHeightC − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - The neighbouring luma samples pY[ x ][ y ] are derived as follows:

    - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..SubHeightC * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..SubWidthC * numSampT − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availT is equal to false, the neighbouring top luma samples pY[ x ][ y ] with x = -1..SubWidthC * numSampT − 1, y = −1, -2, are set equal to the luma samples pY[ x ][ 0 ].

    - When availL is equal to false, the neighbouring left luma samples pY[ x ][ y ] with x = −1, -2, -3, y = -1..SubHeightC * numSampL − 1, are set equal to the luma samples pY[ 0 ][ y ].

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

      - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDstY[ x ][ y ] = pY[ x ][ y ]				(355)

    - Otherwise, the following applies:

      - The one-dimensional filter coefficients array F1 and F2, and the 2-dimensional filter coefficients arrays F3 and F4 are specified as follows.

F1[ 0 ] = 2, F1[ 1 ] = 1				(356)

F2[ 0 ] = 1, F2[ 1 ] = 2, F2[ 2 ] = 1	(357)

F3[ i ][ j ] = F4[ i ][ j ] = 0, with i = 0..2, j = 0..2	(358)

        - If both SubWidthC and SubHeightC are equal to 2, the following applies:

F1[ 0 ] = 1, F1[ 1 ] = 1				(359)

F3[ 0 ][ 1 ] = 1, F3[ 1 ][ 1 ] = 4, F3[ 2 ][ 1 ] = 1, F3[ 1 ][ 0 ] = 1, F3[ 1 ][ 2 ] = 1	(360)

F4[ 0 ][ 1 ]  = 1, F4[ 1 ][ 1 ] = 2, F4[ 2 ][ 1 ] = 1	(361)

F4[ 0 ][ 2 ] = 1,  F4[ 1 ][ 2 ] = 2, F4[ 2 ][ 2 ] = 1	(362)

        - Otherwise, the following applies:

F3[ 1 ][ 1 ] = 8					(363)

F4[ 0 ][ 1 ] = 2, F4[ 1 ][ 1 ] = 4, F4[ 2 ][ 1 ] = 2,	(364)

      - If sps_chroma_vertical_collocated_flag is equal to 1, the following applies:

        - pDsY[ x ][ y ] with x = 01..nTbW − 1, y = 01..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ SubHeightC * y ] +	(365)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F3[ 1 ][ 0 ] * pY[ 0 ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ − 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ SubHeightC * y ] +	(366)
		F3[ 2 ][ 1 ] * pY[ 1 ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ 0 ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (availL is equal to FALSE), pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F2[ 0 ] * pY[ 0 ][ SubHeightC * y − 1 ] +
		F2[ 1 ] * pY[ 0 ][ SubHeightC * y ] +	(367)
		F2[ 2 ] * pY[ 0 ][ SubHeightC * y + 1 ] + 2 ) >> 2

        - If availT is equal to TRUE, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ − 1 ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ 0 ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ 0 ] +	(368)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ 0 ] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ 1 ] + 4 ) >> 3

        - Otherwise (availT is equal to FALSE), pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived:

pDsY[ x ][ 0 ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ 0 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ 0 ] +	(369)
		F2[ 2 ] * pY[ SubWidthC * x + 1 ][ 0 ] + 2 ) >> 2

        - If availL is equal to TRUE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( F3[ 1 ][ 0 ] * pY[ 0 ][ − 1 ] +
		F3[ 0 ][ 1 ] * pY[ − 1 ][ 0 ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ 0 ] +	(370)
		F3[ 2 ][ 1 ] * pY[ 1 ][ 0 ] +
		F3[ 1 ][ 2 ] * pY[ 0 ][ 1 ] + 4 ) >> 3

        - Otherwise, if availL is equal to TRUE and availT is equal to FALSE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( F2[ 0 ] * pY[ − 1 ][ 0 ] + F2[ 1 ] * pY[ 0 ][ 0 ] +
		F2[ 2 ] * pY[ 1 ][ 0 ] + 2 ) >> 2	(371)

        - Otherwise, if availL is equal to FALSE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( F2[ 0 ] * pY[ 0 ][ − 1 ] + F2[ 1 ] * pY[ 0 ][ 0 ] +
		F2[ 2 ] * pY[ 0 ][ 1 ] + 2 ) >> 2	(372)

        - Otherwise (availL is equal to FALSE and availT is equal to FALSE), pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = pY[ 0 ][ 0 ]				(373)

      - Otherwise (sps_chroma_vertical_collocated_flag is equal to 0), the following applies:

        - pDsY[ x ][ y ] with x = 01..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ SubHeightC * y ] +	(374)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1][ SubHeightC * y + 1 ] + 4 ) >> 3

        - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F4[ 0 ][ 1 ] * pY[ − 1 ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ − 1 ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ 0 ][ SubHeightC * y ] +	(375)
		F4[ 1 ][ 2 ] * pY[ 0 ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ 1 ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ 1 ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (availL is equal to FALSE), pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( F1[ 0 ] * pY[ 0 ][ SubHeightC * y ]  +
		F1[ 1 ] * pY[ 0 ][ SubHeightC * y + 1] + 1 ) >> 1	(376)

  - When numSampL is greater than 0, the selected neighbouring left chroma samples pSelC[ idx ] are set equal to p[ −1 ][ pickPosL[ idx ] ] with idx = 0..cntL − 1, and the selected down-sampled neighbouring left luma samples pSelDsY[ idx ] with idx = 0..cntL − 1 are derived as follows:

    - The variable y is set equal to pickPosL[ idx ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ − 1][ y ]			(377)

    - Otherwise the following applies:

      - If sps_chroma_vertical_collocated_flag is equal to 1, the following applies:

        - If y is greater than 0 or availTL is equal to TRUE, pSelDsY[ idx ]  is derived as follows:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ − SubWidthC ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ]  + 	(378)
		F3[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (y is equal to 0), pSelDsY[ idx ]  is derived as follows:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 − SubWidthC ][ 0 ] +
		F2[ 1 ] * pY[ −SubWidthC ][ 0 ] +	(379)
		F2[ 2 ] * pY[ 1 − SubWidthC ][ 0] + 2 ) >> 2

      - Otherwise (sps_chroma_vertical_collocated_flag is equal to 0), the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ −1 − SubWidthC ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ] +	(380)
		F4[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ 1 − SubWidthC][ SubHeightC * y + 1 ] + 4 ) >> 3

  - When numSampT is greater than 0, the selected neighbouring top chroma samples pSelC[ idx ] are set equal to p[ pickPosT[ idx − cntL ] ][ -1 ] with  idx = cntL..cntL + cntT − 1, and the down-sampled neighbouring top luma samples pSelDsY[ idx ] with idx = 0..cntL + cntT − 1 are specified as follows:

    - The variable x is set equal to pickPosT[ idx − cntL ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ x ][ − 1]			(381)

    - Otherwise, the following applies:

      - If sps_chroma_vertical_collocated_flag is equal to 1, the following applies:

        - If x is greater than 0, the following applies:

          - If bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −SubHeightC] +	(382)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −SubHeightC] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ 1 − SubHeightC ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ]  +	(383)
		F2[ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 2 ) >> 2

        - Otherwise (x is equal to 0), the following applies:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ −1 ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ −1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ −SubHeightC ] +	(384)
		F3[ 2 ][ 1 ] * pY[ 1 ][ −SubHeightC ] +
		F3[ 1 ][ 2 ] * pY[ −1 ][ 1 − SubHeightC ] + 4 ) >> 3

          - Otherwise, if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −1 ]  +
		F2[ 2 ] * pY[ 1 ][ −1 ] + 2 ) >> 2	(385)

          - Otherwise, if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ 0 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −2 ] +
		F2[ 2 ] * pY[ 0 ][ −1 ] + 2 ) >> 2	(386)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = pY[ 0 ][ −1 ]		(387)

      - Otherwise (sps_chroma_vertical_collocated_flag is equal to 0), the following applies:

        - If x is greater than 0, the following applies:

          - If bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC x − 1 ][ −2 ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −2 ] +	(388)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ −1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −2 ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ] +	(389)
		F2[ 2 ] * pY[ SubWidthC * x + 1][ −1 ] + 2 ) >> 2

        - Otherwise (x is equal to 0), the following applies:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 ][ −2 ] + F4[ 0 ][ 2 ] * pY[ −1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ 0 ][ −2 ] + F4[ 1 ][ 2 ] * pY[ 0 ][ −1 ] +	(390)		F4[ 2 ][ 1 ] * pY[ 1 ][ −2 ] + F4[ 2 ][ 2 ] * pY[ 1 ][ −1 ] + 4 ) >> 3

          - Otherwise, if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −1 ] +
		F2[ 2 ] * pY[ 1 ][ − 1 ] + 2 ) >> 2	(391)

          - Otherwise, if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F1[ 1 ] * pY[ 0 ][ −2 ] + F1[ 0 ] * pY[ 0 ][ −1 ] + 1 ) >> 1	(392)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = pY[ 0 ][ −1 ]		(393)

…

…

…

# Simulation Results

The proposed method is implemented based on the VTM-7.0 reference software [1] and tested using the common test condition defined in CE3 [2].

Table 1 Results of the proposed first modification

|  | All intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class B | 0.00% | -0.01% | -0.05% | 99% | 99% |
| Class C | -0.01% | 0.05% | -0.07% | 102% | 98% |
| Class E | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class D | -0.02% | 0.04% | 0.03% | 99% | 100% |
| Class F | 0.01% | 0.01% | 0.01% | 99% | 101% |
|  | Random access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.08% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.04% | 0.10% | 100% | 100% |
| Class C | 0.01% | 0.12% | 0.06% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.06% | 100% | 100% |
| Class D | 0.05% | -0.29% | -0.03% | 98% | 101% |
| Class F | 0.01% | 0.06% | 0.00% | 100% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.21% | -0.01% | 100% | 100% |
| Class C | 0.02% | -0.43% | -0.10% | 99% | 99% |
| Class E | -0.01% | 0.26% | 0.00% | 101% | 100% |
| Overall | 0.00% | -0.17% | -0.04% | 100% | 100% |
| Class D | -0.08% | 0.18% | 0.22% | 100% | 98% |
| Class F | -0.01% | -0.70% | 0.15% | 101% | 100% |

# References

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011, 16th meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).