| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0508-v2 |
| --- | --- |

| Title: | CE4-related: Combined tests of JVET-Q0268 and JVET-Q0123 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kevin Reuzé, Han Huang, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz (Qualcomm) Jingya Li, Chong Soon Lim (Panasonic) | Tel: Email: | +65 6552 5423 kreuz@qti.qualcomm.com |
| Source: | Qualcomm Inc, Panasonic Corporation. |  |  |

_____________________________

# Abstract

This contribution proposes a quality scalable GEO design built on the top of JVET-Q0268 and JVET-Q0123. JVET-Q0123 uses 14 GEO modes per PU shape which are a subset of 32 modes per PU shape supported by JVET-Q0268. To combine the two designs, the GEO mode index signalling method proposed in JVET-Q0507 is employed, in which a SPS flag is added with flag equal to 0 and 1 meaning 14 and 32 modes in use, respectively. The combined design provides one-level quality scalability and allows a less capable encoder to achieve decent gain when only 14 GEO modes are enabled on the encoder side.

Compared to the VTM7.0 anchor, the summary results are as follows:

| Test # | Setting | RA (Y, U, V) | LDB (Y, U, V) |
| --- | --- | --- | --- |
| Test 1) | scalable number of modes (14 modes) | -0.14%, -0.11%, -0.12% | -0.**%, -0.**%, -0.**% |
| Test 2) | scalable number of modes (32 modes) | -0.28%, -0.35%, -0.33% | -0.66%, -0.67%, -0.75% |
| Test 3) | scalable number of modes (14 modes, encoder only change) | -0.01%, 0.00%, -0.01% | 0.**%, 0.**% ,0.**% |
| Test 4) | Scalable number of modes (2 modes) | 0.15%, 0.24%, 0.30% | 0.18%, 0.44%, 0.31% |

For 14 GEO modes, the proposed algorithm provides an average gain of -0.13%, -0.11%, -0.11% in RA and -0.**%, -0.**%, -0.**% in LDB when compared to the encoder only method (test 1 vs. test 3)

The results of test 2) also reveals that the full benefit of 32 mode GEO design proposed in JVET-Q0268 is retained in this combined solution.

Additional results are provided for scalable number of modes set to 2 modes. In this case, a switch between 2 modes and 32 modes are proposed.

# Introduction

This contribution proposes a quality scalable GEO design built on the top of JVET-Q0268 and JVET-Q0123. Purpose of this proposal is to address a concern raised by the AHG16 on the JVET email reflector, which claimed that the proposed GEO design under consideration is not quality scalable on the encoder side, a less capable encoder supporting a subset of GEO modes may incur loss instead of gain.

# Proposed combination

JVET-Q0123 and JVET-Q268 support 14 and 32 GEO modes, respectively, wherein 14 modes and 32 modes are PU shape dependent. Specifically, the GEO modes tables are different for each PU shape with (1:8, 1:4, 1:2, 1:1 width/height or height/width ratios). The 32 GEO modes supported by JVET-Q0268 are strictly a superset of 14 modes used in JVET-Q0123, which makes it possible to build quality scalability without introducing new mode tables. Figure 1 and Figure 2 depict an example in which the 14 modes used in JVET-Q0123 are a subset of 32 modes used in JVET-Q0268 for square PU shape. Details are explained in JVET-Q0123 and JVET-Q0268.

Figure 1: 14 modes (diagonal and inverse diagonal angles)

Figure 2:32 modes (CU with width/height=1)

To combine the two designs and to provide one level encoder quality scalability, the GEO index signaling method proposed in JVET-Q0507 is employed. If the GEO is enabled, a flag is signaled at SPS level to switch between 14 and 32 modes.

- If 14 mode is selected, only the first 14 modes of a GEO mode table of a certain coding block shape are valid.

- If 32 mode is selected, all 32 modes of a GEO mode table of a certain coding block shape are valid.

Syntax change is illustrated in Table 1 as below:

Table 1: proposed SPS syntax change to support ‘scalable’ GEO

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| sps_geo_enabled_flag | u(1) |
| if(sps_geo_enabled_flag) |  |
| sps_geo_mode_set_id | u(1) |
| … |  |

sps_geo_mode_set_id specifies whether there are 14 or 32 used modes for GEO inter prediction. sps_geo_mode_set_id equals to 0 specifies that there are 14 used modes and sps_geo_mode_set_id equals to 1 specifies that there are 32 GEO modes in use. When sps_geo_mode_set_id is not present, it is inferred to be equal to 1.

| merge_geo_partition_idx[ ][ ] | TB | cMax = sps_geo_mode_set_id == 0 ? 14 : 32 |
| --- | --- | --- |

To verify the benefit of the proposed signaling, the following three tests are conducted:

- In test 1), the JVET-Q0268 software is used with only the first 14 GEO modes are enabled on the encoder side, the proposed signalling is used.

- In test 2), the JVET-Q0268 software is used with all 32 GEO modes are enabled on the encoder side, the proposed signalling is used.

- In test 3), the JVET-Q0268 software is used with only the first 14 GEO modes are enabled on the encoder side, the proposed signalling is not used (encoder only method).

- In test 4), the JVET-Q0268 software is used with only the first 2 GEO modes (diagonal and invert diagonal angle with distance index = 0) are enabled on the encoder side, the proposed signalling is used. In this case:

  - If 2 mode is selected, only the first 2 modes of a GEO mode table of a certain coding block shape are valid.

  - If 32 mode is selected, all 32 modes of a GEO mode table of a certain coding block shape are valid.

# Experimental results

Coding performance is reported as below. The CTC (common test conditions) defined in [1] was followed.

Table 2: 14 modes

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 Common Software |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.04% | -0.08% | 101% | 101% |
| Class A2 | -0.11% | 0.03% | 0.05% | 96% | 95% |
| Class B | -0.08% | -0.01% | -0.04% | 100% | 100% |
| Class C | -0.28% | -0.39% | -0.36% | 98% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.11% | -0.12% | 99% | 97% |
| Class D | -0.18% | -0.24% | -0.09% | 97% | 92% |
| Class F (optional) | -0.09% | -0.11% | -0.05% | 98% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4 Common Software |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.38% | -0.50% | -0.50% | 96% | 91% |
| Class E | -0.53% | 0.34% | -0.72% | 93% | 88% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.34% | 0.07% | -0.94% | 95% | 92% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 3: 32 modes

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.17% | -0.21% | 103% | 103% |
| Class A2 | -0.24% | -0.21% | -0.12% | 99% | 98% |
| Class B | -0.16% | -0.13% | -0.16% | 103% | 102% |
| Class C | -0.58% | -0.86% | -0.79% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.35% | -0.33% | 101% | 100% |
| Class D | -0.41% | -0.56% | -0.49% | 101% | 99% |
| Class F | -0.29% | -0.37% | -0.33% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.50% | -0.42% | 105% | 103% |
| Class C | -0.80% | -1.21% | -0.91% | 100% | 100% |
| Class E | -0.99% | -0.22% | -1.10% | 96% | 95% |
| Overall | -0.66% | -0.67% | -0.75% | 101% | 100% |
| Class D | -0.68% | -1.06% | -0.76% | 102% | 99% |
| Class F | -0.53% | -0.58% | -0.16% | 101% | 102% |

Table 4: 14 modes (encoder only change)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 Common Software |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.08% | 0.14% | 97% | 96% |
| Class B | 0.03% | 0.13% | 0.00% | 99% | 99% |
| Class C | -0.09% | -0.21% | -0.15% | 97% | 90% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.01% | 98% | 96% |
| Class D | -0.06% | 0.05% | -0.02% | 97% | 92% |
| Class F (optional) | -0.03% | -0.02% | 0.01% | 98% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4 Common Software |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.06% | -0.15% | 0.06% | 96% | 92% |
| Class E | -0.20% | 0.28% | -0.25% | 94% | 89% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.20% | 0.08% | 96% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 5: 2 modes

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.14% | 0.20% | 99% | 99% |
| Class A2 | 0.13% | 0.28% | 0.23% | 99% | 99% |
| Class B | 0.11% | 0.19% | 0.32% | 99% | 99% |
| Class C | 0.25% | 0.33% | 0.39% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.24% | 0.30% | 99% | 99% |
| Class D | 0.12% | 0.20% | 0.28% | 99% | 99% |
| Class F | 0.09% | 0.14% | 0.20% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | 0.15% | 0.03% | 98% | 99% |
| Class C | 0.28% | 0.32% | 0.58% | 98% | 99% |
| Class E | 0.08% | 1.08% | 0.41% | 97% | 99% |
| Overall | 0.18% | 0.44% | 0.31% | 98% | 99% |
| Class D | 0.24% | 0.88% | -0.07% | 98% | 99% |
| Class F | 0.13% | 0.20% | 0.48% | 98% | 98% |

# Discussions

The GEO design should consider a wide range of encoder implementations and be able to provide coding gain even if an encoder is not capable of searching full modes. This combined solution not only reduces the number of GEO modes to 32 per PU shape but also enables one-level quality scalability on the encoder side. Therefore, it has addressed concerns regarding the number of GEO modes and the GEO mode index signalling raised by the AHG16 on the JVET email reflector.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[2] J. Li, C.-S. Lim, “CE4-related: Modifications of GEO”, JVET-Q0123, 17th Meeting: Brussels, BE, 7–17 January 2020

[3] K. Reuzé, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz, “CE4-related: GEO with 32 modes”, JVET-Q0268, 17th Meeting: Brussels, BE, 7–17 January 2020

[4] K. Reuzé, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz, J. Li, C.-S. Lim, “CE4-related: Quality scalable GEO”, JVET-Q0507, 17th Meeting: Brussels, BE, 7–17 January 2020

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).