Proposed spec changes

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_act_qp_offsets_present_flag | u(1) |
| if (pps_act_qp_offsets_present_flag) { |  |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_minus1 | se(v) |
| pps_act_cr_qp_offset_minus3 | se(v) |
| } |  |
| …… |  |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

……

pps_act_qp_offsets_present_flag equal to 1 specifies that pps_act_y_qp_offset_plus5  and pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3 are present in the PPS syntax structure. pps_act_qp_offsets_present_flag equal to 0 specifies that pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, and pps_act_cr_qp_offset_minus3 are not present in the PPS RBSP syntax structure. When sps_act_enabled_flag is equal to 0, the value of pps_act_qp_offsets_present_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1, pps_act_cr_qp_offset_minus3, are used to determine the offsets that are applied to the quantization parameter values qP for the luma, Cb, Cr components respectively, when cu_act_enabled_flag is equal to 1. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_minus1 and pps_act_cr_qp_offset_minus3 are inferred to be equal to 0.

The variable ppsActQpOffsetY is set equal to pps_act_y_qp_offset_plus5 - 5.

The variable ppsActQpOffsetCb is set equal to pps_act_cb_qp_offset_minus1 + 1.

The variable ppsActQpOffsetCr is set equal to pps_act_cr_qp_offset_minus3 + 3.

ppsActQpOffsetY, ppsActQpOffsetCb and ppsActQpOffsetCr shall be in the range of -12 to 12, inclusive.

……

### 8.7.3 Scaling process for transform coefficients

……

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 )                                                               (1133)

qP = Clip3(0, 63+ QpBdOffset, qP + ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx = = 0 ? ppsActQpOffsetY : (cIdx == 1 ? ppsActQpOffsetCb : ppsActQpOffsetCr)) : 0 ) )                            (1133)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 )  ?  1 : 0                                  (1134)

bdShift = BitDepth + rectNonTsFlag +	                                                                                                 (1135)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ]  ?  5  :  0 )                       (1136)

qP = Clip3( QpPrimeTsMin, 63+ QpBdOffset, qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx = = 0 ? ppsActQpOffsetY : (cIdx == 1 ? ppsActQpOffsetCb : ppsActQpOffsetCr)): 0 ) )                             (1136)

……

### 8.7.4.6 Residual modification process for blocks using colour space conversion

……

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

tmp = rY[ x ][ y ] − rCb[ x ][ y ]	(1191)

rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]	(1192)

rCb[ x ][ y ] = tmp − rCr[ x ][ y ]	(1193)

rCr[ x ][ y ] = tmp + rCr[ x ][ y ]	(1194)

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )	(1191)

rY[ x ][ y ] = tmp + rCb[ x ][ y ]	(1192)

rCb[ x ][ y ] = tmp − ( rCr[ x ][ y ]  >>  1 )	(1193)

rCr[ x ][ y ]  +=  rCb[ x ][ y ]	(1194)

……