| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0512 |
| --- | --- |

| Title: | Enabling transform skip and BDPCM for chroma in ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

At the 16th JVET meeting, adaptive color transform (ACT) was adopted into VVC draft 7 for removing the redundancy between three color components in 444 videos. At the same meeting, transform skip (TS) and block differential pulse coded modulation (BDPCM) were extended to coding chroma residuals. Due to separate adoptions, in VVC draft 7 and VTM-7.1 software, the interactions of the three tools are not properly handled. Specifically, though chroma TS mode is supported for the ACT mode in VVC draft 7, it is not checked for the ACT mode at VTM-7.1 encoder. Additionally, when the ACT is applied to one intra coding unit (CU), the BDPCM mode is always disabled for chroma components in both VVC draft 7 and VTM-7.1.

In this contribution, to improve the coding efficiency of 444 videos, changes are proposed to enable the TS and BDPCM modes for the chroma residual coding of the ACT mode. The simulations were conducted for RGB and YCbCr444 videos with CTC QPs and lower QPs. Compared to the VTM-7.1 ACT anchors, the average BD-rate impacts of the proposed changes are summarized as

With proposed encoder modification of enabling chroma TS for the ACT:

- Average {G, B, R} BD-rate differences {-0.13%, -0.11%, -0.07%}, {-0.06%, -0.11%, -0.06%} and {-0.34%, -0.17%, -0.15%} for AI, RA and LD, respectively, for RGB sequences.

With the method one of normatively enabling chroma BDPCM for the ACT:

- Average {G, B, R} BD-rate differences {-0.21%, -0.13%, -0.13%}, {-0.10%, -0.08%, -0.08%} and {-0.35%, -0.19%, -0.21%} for AI, RA and LD, respectively, for RGB sequences.

With the method two of normatively enabling chroma BDPCM for the ACT:

- Average {G, B, R} BD-rate differences {xx%, xx%, xx%}, {xx%, xx%, xx%} and {xx%, xx%, xx%} for AI, RA and LD, respectively, for RGB sequences.

# Introduction

At the 16th JVET meeting, adaptive color transform (ACT) [1] was adopted into VVC draft 7 [2] to enhance the efficiency of 444 video coding, as summarized as:

- Convert prediction residuals from one color space to YCgCo space.

- One ACT flag is signaled at CU-level.

- QP offsets are applied to adjust the dynamic range of the residual signals in the transformed domain.

- The ACT is implicitly disabled for separate-tree partition and intra sub-partition prediction (ISP).

At the same meeting, transform skip (TS) [3] and block differential pulse coded modulation (BDPCM) [4], which were previously only supported for luma component, were extended to coding chroma residuals, with the following modifications:

- Signal transform_skip_flag for chroma components.

- Signal intra_bdpcm_chroma_flag and intra_bdpcm_dir_chroma when BDPCM is enabled for chroma components.

- When chroma BDPCM is enabled for one intra CU, the flag transform_skip_flag is not signaled for chroma but inferred to be one.

- TS residual coding is applied to chroma TS with context-coded bin (CCB) limit of 1.75 per coefficient.

Due to separate adoptions, the interactions of the three tools are not properly handled in VVC draft 7 [2] and VTM-7.1[5], as described as follows:

- Although chroma TS mode is supported for the ACT mode in VVC draft 7, the rate-distortion (R-D) checking of the chroma TS is not conducted for the ACT mode in VTM-7.1.

- In VVC draft 7 [2], BDPCM mode is always disabled for chroma components of one ACT intra CU, i.e., the flag intra_bdpcm_chroma_flag is not signaled but always inferred to be zero when one intra CU is coded with the ACT being applied.

# Proposal

To improve the coding efficiency of the ACT, the following changes are proposed in this contribution.

- Encoder-only modification of enabling chroma TS for the ACT

Encoder-only modifications are provided to enable the R-D checking of the chroma TS mode for the CUs that apply the ACT, for both joint coding of chroma residuals (JCCR) mode and non-JCCR modes. Additionally, to reduce the encoding complexity, the following fast encoder logics are applied when enabling the chroma TS and BDPCM for the ACT mode.

- The same encoder method, which is used for the fast MTS selection in VTM7.1, is reused to reduce the number of transform candidates for R-D checking.

- The R-D checking of chroma TS mode is skipped if the coefficients of chroma transform mode are all zeros.

- Normative modifications of enabling chroma BDPCM for the ACT

In the contribution, two methods are proposed to enable the chroma BDPCM mode when the ACT is applied.

- Method One: when the ACT is applied to one intra CU, it is proposed to signal intra_bdpcm_chroma_flag to indicate whether BDPCM mode applied to chroma components or not. When the flag is one, intra_bdpcm_dir_chroma is further signaled to indicate the BDPCM direction used for both chorma components.

- Method Two: when the ACT is applied, the presence of the flag intra_bdpcm_chroma_flag is conditioned on the presence of the luma BDPCM flag, i.e., intra_bdpcm_flag. If the flag intra_bdpcm_flag is equal to one, the flag intra_bdpcm_chroma_flag is signaled; otherwise, the flag intra_bdpcm_chroma_flag is inferred to be 0. When the flag intra_bdpcm_chroma_flag is equal to one, the applied chroma BDPCM direction is always set equal to the luma BDPCM direction, i.e., the flag intra_bdpcm_chroma_dir is set equal to the value of intra_bdpcm_dir.

# Simulation results

The proposed changes were implemented on top of VTM-7.1 [5] and tested following the test conditions used by the core experiment (CE) 2 [6]. In the simulation, ACT, IBC, PLT, BDPCM (both for luma and chroma, i.e., BDPCM = 2) and inter HashME are enabled for both anchor and test. Results at both CTC QPs (22, 27, 32, 37) and lower QPs (5, 10, 15, 17) when the separate-tree is disabled are provided in the below.

- Performance of enabling the chroma TS for the ACT

Table 1 The BD-rate performance of enabling chroma TS for ACT, CTC QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.12% | -0.08% | -0.02% | 100% | 102% |
| RGB, TGM 720p | -0.12% | -0.17% | -0.12% | 101% | 99% |
| RGB, Animation | -0.08% | -0.11% | -0.14% | 100% | 100% |
| RGB, Mixed content | -0.15% | -0.12% | -0.07% | 103% | 100% |
| RGB, Camera-Captured content | -0.15% | -0.03% | -0.06% | 96% | 98% |
| YUV, TGM 1080p | -0.16% | -0.11% | -0.13% | 101% | 99% |
| YUV, TGM 720p | -0.22% | -0.08% | -0.07% | 99% | 101% |
| YUV, Animation | -0.03% | -0.08% | -0.11% | 100% | 101% |
| YUV, Mixed content | -0.25% | -0.05% | -0.22% | 99% | 100% |
| YUV, Camera-Captured content | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Overall (RGB) | -0.13% | -0.11% | -0.07% | 100% | 100% |
| Overall (YUV) | -0.15% | -0.08% | -0.11% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.10% | -0.14% | -0.04% | 97% | 96% |
| RGB, TGM 720p | -0.06% | -0.13% | -0.12% | 102% | 101% |
| RGB, Animation | 0.05% | -0.12% | -0.09% | 98% | 101% |
| RGB, Mixed content | -0.15% | -0.11% | -0.02% | 100% | 100% |
| RGB, Camera-Captured content | 0.06% | -0.02% | -0.05% | 99% | 96% |
| YUV, TGM 1080p | -0.08% | -0.06% | -0.03% | 101% | 105% |
| YUV, TGM 720p | -0.15% | -0.10% | -0.24% | 105% | 98% |
| YUV, Animation | -0.09% | -0.21% | -0.10% | 100% | 100% |
| YUV, Mixed content | -0.26% | -0.11% | -0.20% | 101% | 99% |
| YUV, Camera-Captured content | 0.01% | -0.04% | -0.04% | 100% | 101% |
| Overall (RGB) | -0.06% | -0.11% | -0.06% | 100% | 99% |
| Overall (YUV) | -0.12% | -0.10% | -0.13% | 102% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.08% | -0.10% | -0.05% | 98% | 101% |
| RGB, TGM 720p | -0.38% | -0.08% | -0.22% | 104% | 100% |
| RGB, Animation | -0.46% | -0.30% | -0.20% | 104% | 99% |
| RGB, Mixed content | -0.72% | -0.42% | -0.27% | 100% | 100% |
| RGB, Camera-Captured content | -0.12% | -0.03% | -0.03% | 103% | 101% |
| YUV, TGM 1080p | -0.19% | -0.20% | -0.25% | 102% | 101% |
| YUV, TGM 720p | -0.16% | -0.31% | -0.05% | 103% | 100% |
| YUV, Animation | 0.00% | -0.16% | -0.05% | 101% | 100% |
| YUV, Mixed content | -0.14% | 0.18% | 0.30% | 100% | 100% |
| YUV, Camera-Captured content | 0.07% | -0.02% | 0.09% | 99% | 100% |
| Overall (RGB) | -0.34% | -0.17% | -0.15% | 101% | 101% |
| Overall (YUV) | -0.11% | -0.13% | -0.02% | 101% | 100% |

Table 2 The BD-rate performance of enabling chroma TS for ACT, lower QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.07% | -0.02% | -0.02% | 100% | 103% |
| RGB, TGM 720p | -0.12% | -0.03% | 0.06% | 97% | 98% |
| RGB, Animation | -0.07% | -0.03% | -0.06% | 99% | 100% |
| RGB, Mixed content | -0.15% | 0.00% | 0.02% | 99% | 102% |
| RGB, Camera-Captured content | 0.00% | 0.00% | -0.01% | 97% | 95% |
| YUV, TGM 1080p | -0.23% | -0.12% | -0.12% | 102% | 104% |
| YUV, TGM 720p | -0.14% | -0.03% | 0.01% | 102% | 101% |
| YUV, Animation | -0.14% | -0.11% | -0.12% | 99% | 97% |
| YUV, Mixed content | -0.22% | 0.05% | 0.03% | 101% | 98% |
| YUV, Camera-Captured content | 0.00% | -0.01% | -0.02% | 97% | 98% |
| Overall (RGB) | -0.09% | -0.02% | 0.01% | 99% | 101% |
| Overall (YUV) | -0.16% | -0.05% | -0.05% | 101% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.41% | -0.33% | -0.28% | 101% | 98% |
| RGB, TGM 720p | -0.16% | -0.03% | 0.13% | 103% | 100% |
| RGB, Animation | -0.50% | -0.41% | -0.32% | 101% | 97% |
| RGB, Mixed content | -0.17% | -0.10% | 0.02% | 99% | 95% |
| RGB, Camera-Captured content | -0.10% | 0.01% | -0.02% | 104% | 104% |
| YUV, TGM 1080p | -0.55% | -0.41% | -0.34% | 105% | 102% |
| YUV, TGM 720p | -0.15% | 0.10% | 0.05% | 99% | 100% |
| YUV, Animation | -0.65% | -0.61% | -0.48% | 99% | 99% |
| YUV, Mixed content | -0.37% | -0.15% | -0.11% | 100% | 100% |
| YUV, Camera-Captured content | -0.03% | -0.03% | -0.08% | 102% | 101% |
| Overall (RGB) | -0.25% | -0.15% | -0.06% | 101% | 98% |
| Overall (YUV) | -0.37% | -0.22% | -0.19% | 101% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.34% | -0.32% | -0.33% | 103% | 99% |
| RGB, TGM 720p | -0.43% | -0.26% | -0.20% | 98% | 101% |
| RGB, Animation | -0.43% | -0.30% | -0.28% | 96% | 99% |
| RGB, Mixed content | -0.46% | -0.37% | -0.14% | 101% | 103% |
| RGB, Camera-Captured content | -0.22% | 0.06% | -0.03% | 96% | 94% |
| YUV, TGM 1080p | -0.43% | -0.24% | -0.21% | 104% | 101% |
| YUV, TGM 720p | -0.08% | -0.03% | -0.22% | 102% | 99% |
| YUV, Animation | -0.64% | -0.46% | -0.41% | 99% | 96% |
| YUV, Mixed content | -0.07% | 0.07% | -0.06% | 106% | 103% |
| YUV, Camera-Captured content | -0.04% | 0.02% | -0.06% | 98% | 101% |
| Overall (RGB) | -0.38% | -0.26% | -0.20% | 100% | 100% |
| Overall (YUV) | -0.26% | -0.14% | -0.20% | 102% | 100% |

- Performance of Method One for enabling the chroma BDPCM for the ACT

Table 3 The BD-rate performance of enabling chroma BDPCM for ACT (Method One), CTC QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.25% | -0.12% | -0.10% | 99% | 99% |
| RGB, TGM 720p | -0.24% | -0.18% | -0.14% | 103% | 103% |
| RGB, Animation | -0.06% | -0.11% | -0.11% | 100% | 100% |
| RGB, Mixed content | -0.24% | -0.15% | -0.19% | 100% | 96% |
| RGB, Camera-Captured content | -0.12% | -0.04% | -0.05% | 99% | 100% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 104% | 100% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 102% | 101% |
| YUV, Animation | 0.00% | 0.00% | -0.01% | 100% | 99% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall (RGB) | -0.21% | -0.13% | -0.13% | 101% | 100% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 101% | 101% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.14% | -0.05% | -0.06% | 100% | 101% |
| RGB, TGM 720p | -0.02% | -0.09% | -0.10% | 101% | 100% |
| RGB, Animation | -0.02% | -0.01% | 0.05% | 101% | 100% |
| RGB, Mixed content | -0.26% | -0.14% | -0.15% | 101% | 98% |
| RGB, Camera-Captured content | 0.03% | -0.06% | -0.01% | 101% | 100% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 98% |
| YUV, TGM 720p | -0.02% | -0.02% | -0.01% | 103% | 100% |
| YUV, Animation | -0.04% | -0.05% | -0.03% | 100% | 100% |
| YUV, Mixed content | -0.03% | -0.02% | -0.01% | 101% | 99% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall (RGB) | -0.10% | -0.08% | -0.08% | 101% | 100% |
| Overall (YUV) | -0.02% | -0.02% | -0.01% | 101% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | 0.02% | 0.05% | 0.02% | 99% | 100% |
| RGB, TGM 720p | -0.50% | -0.24% | -0.46% | 99% | 95% |
| RGB, Animation | -0.33% | -0.20% | -0.19% | 100% | 100% |
| RGB, Mixed content | -0.79% | -0.54% | -0.28% | 100% | 98% |
| RGB, Camera-Captured content | -0.16% | -0.02% | -0.09% | 97% | 99% |
| YUV, TGM 1080p | -0.02% | 0.01% | -0.05% | 100% | 101% |
| YUV, TGM 720p | 0.02% | -0.01% | -0.25% | 99% | 96% |
| YUV, Animation | 0.02% | 0.00% | -0.03% | 101% | 100% |
| YUV, Mixed content | -0.13% | -0.25% | -0.33% | 96% | 97% |
| YUV, Camera-Captured content | 0.06% | -0.02% | 0.15% | 99% | 97% |
| Overall (RGB) | -0.35% | -0.19% | -0.21% | 99% | 98% |
| Overall (YUV) | -0.02% | -0.05% | -0.12% | 99% | 98% |

Table 4 The BD-rate performance of enabling chroma BDPCM for ACT (Method One), lower QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.16% | 0.03% | -0.04% | 92% | 102% |
| RGB, TGM 720p | -0.25% | -0.12% | -0.12% | 94% | 98% |
| RGB, Animation | -0.16% | -0.07% | -0.07% | 101% | 101% |
| RGB, Mixed content | -0.44% | -0.23% | -0.22% | 100% | 102% |
| RGB, Camera-Captured content | -0.01% | 0.00% | 0.00% | 102% | 102% |
| YUV, TGM 1080p | 0.00% | 0.00% | 0.00% | 103% | 102% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 98% |
| YUV, Animation | 0.00% | 0.00% | 0.00% | 104% | 103% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 103% | 100% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 96% |
| Overall (RGB) | -0.22% | -0.08% | -0.10% | 96% | 101% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 102% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.09% | 0.10% | 0.14% | 101% | 100% |
| RGB, TGM 720p | -0.23% | -0.13% | -0.03% | 100% | 99% |
| RGB, Animation | -0.02% | -0.06% | -0.02% | 101% | 98% |
| RGB, Mixed content | -0.23% | -0.05% | 0.00% | 100% | 94% |
| RGB, Camera-Captured content | -0.03% | 0.00% | -0.01% | 102% | 102% |
| YUV, TGM 1080p | -0.01% | -0.02% | -0.02% | 102% | 100% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.01% | 99% | 100% |
| YUV, Animation | -0.01% | -0.02% | -0.01% | 97% | 97% |
| YUV, Mixed content | 0.00% | 0.00% | 0.00% | 98% | 95% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall (RGB) | -0.15% | -0.02% | 0.03% | 101% | 98% |
| Overall (YUV) | 0.00% | -0.01% | 0.00% | 99% | 98% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.14% | -0.01% | -0.08% | 99% | 97% |
| RGB, TGM 720p | -0.42% | -0.20% | -0.12% | 99% | 100% |
| RGB, Animation | -0.09% | -0.07% | -0.03% | 97% | 103% |
| RGB, Mixed content | -0.87% | -0.36% | -0.26% | 99% | 100% |
| RGB, Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| YUV, TGM 1080p | -0.03% | -0.01% | -0.01% | 102% | 105% |
| YUV, TGM 720p | -0.01% | -0.01% | -0.09% | 103% | 101% |
| YUV, Animation | 0.01% | -0.01% | 0.01% | 99% | 97% |
| YUV, Mixed content | 0.06% | 0.01% | 0.02% | 101% | 102% |
| YUV, Camera-Captured content | 0.00% | 0.01% | 0.01% | 98% | 98% |
| Overall (RGB) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall (YUV) | 0.00% | 0.00% | -0.02% | 101% | 101% |

- Performance of Method Two for enabling the chroma BDPCM for the ACT

Table 5 The BD-rate performance of enabling chroma BDPCM for ACT (Method Two), CTC QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.12% | -0.08% | -0.02% | 100% | 102% |
| RGB, TGM 720p | -0.12% | -0.17% | -0.12% | 101% | 99% |
| RGB, Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| RGB, Mixed content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| RGB, Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, TGM 720p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, Mixed content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall (RGB) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall (YUV) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6 The BD-rate performance of enabling chroma BDPCM for ACT (Method Two), low QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 ACT ON |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| RGB, TGM 1080p | -0.07% | -0.02% | -0.02% | 100% | 103% |
| RGB, TGM 720p | -0.12% | -0.03% | 0.06% | 97% | 98% |
| RGB, Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| RGB, Mixed content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| RGB, Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, TGM 720p | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, Mixed content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| YUV, Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall (RGB) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall (YUV) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Proposed spec changes

- Spec changes of Method One for enabling BDPCM for ACT

| coding_unit ( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( !cu_act_enabled_flag ) { |  |
| if( cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && sps_bdpcm_chroma_enabled_flag ) { |  |
| intra_bdpcm_chroma_flag | ae(v) |
| if( intra_bdpcm_chroma_flag ) |  |
| intra_bdpcm_chroma_dir_flag | ae(v) |
| } else { |  |
| if( !cu_act_enabled_flag ) { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |
| …… |  |
| } |  |

- Spec changes of Method Two for enabling BDPCM for ACT

| coding_unit ( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( !cu_act_enabled_flag ) { |  |
| if( cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && (!cu_act_enabled_flag ¦¦ intra_bdpcm_flag) && sps_bdpcm_chroma_enabled_flag ) { |  |
| intra_bdpcm_chroma_flag | ae(v) |
| if( intra_bdpcm_chroma_flag && !cu_act_enabled_flag ) |  |
| intra_bdpcm_chroma_dir_flag | ae(v) |
| } else { |  |
| if( !cu_act_enabled_flag ) { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |
| …… |  |
| } |  |

intra_bdpcm_chroma_dir_flag equal to 0 specifies that the BDPCM prediction direction is horizontal. intra_bdpcm_chroma_dir_flag equal to 1 specifies that the BDPCM prediction direction is vertical.

When intra_bdpcm_chroma_dir_flag is not present, it is inferred to be equal to be equal to intra_bdpcm_luma_dir_flag when cu_act_enabled_flag is equal to one; otherwise, it is inferred to be 0.

# Reference

X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, “Support of adaptive color transform for 444 video coding in VVC”, JVET-P0517, Oct. 2019, Geneva, Switzerland.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding”, JVET-P0058, Oct. 2019, Geneva, Switzerland.

F. Henry, et al, “CE8-4.1: BDPCM and Transform skip for Chroma”, JVET-P0059, Oct. 2019, Geneva, Switzerland.

VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.1.

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode”, JVET-P2022, Oct. 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).