| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0514_r1 |
| --- | --- |

| Title: | AHG11: Encoder improvements on JCCR with chroma transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

At the 16th JVET meeting, transform skip (TS) mode was extended to coding chroma residuals. In VVC draft 7, the syntax design allows the chroma TS mode to be jointly applied with the joint coding of chroma residuals (JCCR) mode. However, the rate-distortion (R-D) checking of the TS mode in VTM-7.0 is never performed when the chroma residuals of one coding unit (CU) are coded by the JCCR mode. In this contribution, one encoder implementation is provided to properly enable the chroma TS mode for the chroma residual coding of the JCCR mode in VTM-7.0. The simulations were conducted on YCbCr444 videos with CTC QPs and lower QPs. Compared to the VTM-7.0 anchors, the performance of the proposed encoder improvement is summarized as follows:

For the CTC QPs:

- Average {Y, Cb, Cr} BD-rate differences {-0.13%, -0.09%, -0.18%%}, {-0.11%, -0.09%, -0.13%} and {-0.10%, 0.00%, 0.09%%} for AI, RA and LDB, respectively.

For the lower QPs:

- Average {Y, Cb, Cr} BD-rate differences {-0.15%, -0.01%, -0.01%%}, {-0.41%, -0.23%, -0.23%} and {-0.36%, -0.22%, -0.21%%} for AI, RA and LDB, respectively.

In all the tests, the impacts of the proposed encoder modifications on both encoding and decoding time are negligible.

# Proposal

At the 16th JVET meeting, transform skip (TS) [1], which were previously only supported for luma component, were extended to coding chroma residuals, with the following modifications:

- Signal transform_skip_flag for chroma components.

- TS residual coding is applied to chroma TS with context-coded bin (CCB) limit of 1.75 per coefficient.

Although chroma TS mode can be applied to joint coding of chroma residuals (JCCR) mode in VVC draft 7 [2], the rate-distortion (R-D) cost of the chroma TS is never checked when the JCCR mode is enabled for the chroma residual coding of one coding unit (CU) in VTM-7.0 [3].

To further improve the coding efficiency, encoder modifications are proposed in this contribution to enable the R-D checking of the chroma TS mode when the JCCR mode is applied. Additionally, to avoid the encoding complexity increase, the following fast encoder methods are applied.

- The same encoder method, which is used for the fast MTS selection in VTM7.0, is reused to reduce the number of transform candidates for R-D checking.

- The R-D checking of chroma TS mode is skipped if the coefficients of chroma transform mode are all zeros.

# Simulation results

The proposed changes were implemented on top of VTM-7.0 [3] and tested following the test conditions used by the core experiment (CE) 2 [4]. In the simulation, IBC, PLT, BDPCM (both for luma and chroma, i.e., BDPCM = 2) and inter HashME are enabled for both anchor and test. Results at both CTC QPs (22, 27, 32, 37) and lower QPs (2, 7, 12, 17) when the dual-tree is disabled and enabled are provided in the below.

2.1 Simulation results for dual-tree OFF

Table 1 The BD-rate performance of the proposed changes for CTC QPs and dual-tree OFF

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.14% | -0.10% | -0.10% | 100% | 99% |
| TGM 720p | -0.15% | -0.03% | -0.18% | 100% | 99% |
| Animation | -0.05% | -0.20% | -0.28% | 100% | 100% |
| Mixed content | -0.28% | -0.06% | -0.30% | 103% | 103% |
| Camera-Captured content | 0.00% | -0.08% | -0.02% | 103% | 101% |
| Overall | -0.13% | -0.09% | -0.18% | 101% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.09% | -0.12% | -0.12% | 100% | 100% |
| TGM 720p | -0.13% | 0.01% | -0.12% | 100% | 102% |
| Animation | -0.05% | -0.34% | -0.34% | 101% | 100% |
| Mixed content | -0.23% | 0.04% | 0.01% | 100% | 101% |
| Camera-Captured content | -0.01% | -0.02% | -0.10% | 99% | 101% |
| Overall | -0.11% | -0.09% | -0.13% | 100% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.16% | -0.14% | -0.17% | 100% | 105% |
| TGM 720p | -0.06% | 0.17% | 0.25% | 100% | 101% |
| Animation | -0.17% | -0.23% | -0.17% | 102% | 95% |
| Mixed content | -0.08% | 0.17% | 0.58% | 101% | 100% |
| Camera-Captured content | -0.03% | 0.06% | -0.09% | 99% | 100% |
| Overall | -0.10% | 0.00% | 0.09% | 101% | 101% |

Table 2 The BD-rate performance of the proposed changes for lower QPs and dual-tree OFF

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.20% | -0.09% | -0.11% | 104% | 99% |
| TGM 720p | -0.13% | 0.01% | 0.04% | 99% | 95% |
| Animation | -0.16% | 0.10% | 0.10% | 100% | 98% |
| Mixed content | -0.18% | -0.04% | -0.04% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | -0.02% | 101% | 103% |
| Overall | -0.15% | -0.01% | -0.01% | 101% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.68% | -0.45% | -0.34% | 101% | 99% |
| TGM 720p | -0.10% | -0.06% | -0.08% | 99% | 100% |
| Animation | -0.83% | -0.53% | -0.49% | 98% | 100% |
| Mixed content | -0.30% | -0.08% | -0.15% | 102% | 104% |
| Camera-Captured content | -0.03% | 0.06% | -0.03% | 100% | 100% |
| Overall | -0.41% | -0.23% | -0.23% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.58% | -0.34% | -0.36% | 101% | 101% |
| TGM 720p | -0.14% | -0.15% | 0.01% | 103% | 102% |
| Animation | -0.80% | -0.56% | -0.52% | 97% | 99% |
| Mixed content | -0.15% | 0.01% | -0.10% | 103% | 100% |
| Camera-Captured content | -0.03% | 0.00% | -0.07% | 101% | 98% |
| Overall | -0.36% | -0.22% | -0.21% | 101% | 100% |

2.2 Simulation results for dual-tree ON

Table 3 The BD-rate performance of the proposed changes for CTC QPs and dual-tree ON

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.22% | -0.18% | -0.25% | 101% | 101% |
| TGM 720p | -0.05% | -0.07% | -0.16% | 98% | 96% |
| Animation | -0.20% | -0.40% | -0.35% | 100% | 103% |
| Mixed content | -0.14% | -0.32% | -0.09% | 102% | 97% |
| Camera-Captured content | 0.00% | -0.10% | -0.01% | 101% | 99% |
| Overall | -0.13% | -0.21% | -0.19% | 100% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.15% | -0.13% | -0.20% | 102% | 100% |
| TGM 720p | -0.10% | -0.09% | -0.27% | 102% | 100% |
| Animation | -0.01% | -0.29% | -0.29% | 101% | 99% |
| Mixed content | -0.12% | -0.53% | -0.25% | 100% | 100% |
| Camera-Captured content | -0.01% | 0.04% | 0.00% | 99% | 100% |
| Overall | -0.09% | -0.20% | -0.22% | 101% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | -0.07% | -0.09% | 101% | 102% |
| TGM 720p | -0.15% | -0.11% | -0.10% | 99% | 100% |
| Animation | -0.05% | -0.19% | -0.27% | 99% | 100% |
| Mixed content | -0.11% | -0.31% | 0.02% | 101% | 103% |
| Camera-Captured content | 0.01% | -0.02% | -0.16% | 97% | 99% |
| Overall | -0.09% | -0.14% | -0.11% | 100% | 101% |

Table 4 The BD-rate performance of the proposed changes for lower QPs and dual-tree ON

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.09% | -0.09% | -0.08% | 102% | 101% |
| TGM 720p | -0.07% | -0.08% | -0.07% | 100% | 96% |
| Animation | -0.16% | -0.18% | -0.13% | 106% | 105% |
| Mixed content | -0.08% | -0.05% | -0.03% | 100% | 100% |
| Camera-Captured content | -0.01% | 0.00% | 0.00% | 96% | 95% |
| Overall | -0.09% | -0.09% | -0.07% | 101% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.66% | -0.43% | -0.40% | 102% | 99% |
| TGM 720p | 0.01% | -0.09% | -0.12% | 105% | 101% |
| Animation | -0.80% | -0.66% | -0.52% | 101% | 100% |
| Mixed content | -0.28% | -0.07% | -0.17% | 98% | 100% |
| Camera-Captured content | -0.03% | 0.01% | -0.06% | 100% | 105% |
| Overall | -0.37% | -0.27% | -0.26% | 101% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.56% | -0.38% | -0.36% | 99% | 99% |
| TGM 720p | -0.15% | -0.02% | -0.06% | 103% | 101% |
| Animation | -0.82% | -0.53% | -0.46% | 101% | 96% |
| Mixed content | -0.28% | -0.16% | -0.31% | 100% | 103% |
| Camera-Captured content | -0.03% | 0.04% | -0.05% | 102% | 98% |
| Overall | -0.39% | -0.23% | -0.26% | 101% | 100% |

# Reference

T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding”, JVET-P0058, Oct. 2019, Geneva, Switzerland.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode”, JVET-P2022, Oct. 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).