| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0515 |
| --- | --- |

| Title: | Disabling chroma transform skip mode for ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

It is reported that with the adoption of chroma transform skip mode in VVC draft 7, the transform skip mode is treated differently for luma and chroma components in the intra sub-partition (ISP) mode. Specifically, for the ISP mode, the transform skip mode is always disabled for luma component but allowed for chroma components. In the current VVC, this is also the only prediction mode under which transform skip is enabled differently for different video components.

To make the design more consistent, this contribution proposes to disable the transform skip for both luma and chroma components when the ISP mode is applied. Simulation results reportedly show that for YCbCr444 videos, the proposed method has average {Y, Cb, Cr} BD-rate differences of {0.00%, 0.03%, 0.01%} and {-0.05%, -0.02%, 0.02%} for RA and LD configurations, when dual-tree is enabled. When dual-tree is disabled, the corresponding average {Y, Cb, Cr} BD-rate differences are {0.00%, 0.02%, 0.02%}, {-0.02%, 0.01%, 0.06%} and {-0.05%, 0.07%, 0.09%} for AI, RA and LD configurations, respectively.

# Introduction

At the 16th JVET meeting, transform skip (TS) [1], which were previously only supported for luma component, were extended to coding chroma residuals, with the following modifications:

- Signal transform_skip_flag for chroma components.

- TS residual coding is applied to chroma TS with context-coded bin (CCB) limit of 1.75 per coefficient.

In VVC draft 7 [2], intra sub-partition (ISP) is one coding tool where one coding unit (CU) is further partitioned into multiple transform units (TUs) and the implicit selection between DCT-2 and DST-7 transforms is applied to code luma residuals.

It is reported that with enabling chroma TS mode, the use of the transform skip mode is inconsistent between the luma and chroma components of the ISP mode. Specifically, the transform skip mode is always disabled for luma component but allowed to be used for chroma components. Such design looks unreasonable given that luma channel usually contains more abundant residual information than chroma channels.

# Proposal

In addition to luma component, this contribution proposes to also disable the transform skip mode for chroma components of the ISP mode such that the use of the transform skip mode is consistent between luma and chroma components. The proposed text specification changes are as follows.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && (IntraSubPartitionsSplitType[ x0 ][ y0 ] == ISP_NO_SPLIT) && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && (IntraSubPartitionsSplitType[ x0 ][ y0 ] == ISP_NO_SPLIT) && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

# Simulation results

The proposed change was implemented on top of VTM-7.0 [1] and tested following the test conditions used by the core experiment (CE) 2 [3]. In the simulation, IBC, PLT, BDPCM (both for luma and chroma, i.e., BDPCM = 2) and inter HashME are enabled for both anchor and test.

- Simulation results for dual-tree ON

Table 1 The BD-rate performance of the proposed method when dual-tree is ON

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.03% | 0.02% | 100% | 98% |
| TGM 720p | -0.01% | -0.01% | -0.03% | 99% | 99% |
| Animation | -0.02% | 0.01% | -0.03% | 101% | 98% |
| Mixed content | 0.04% | 0.13% | 0.10% | 97% | 99% |
| Camera-Captured content | 0.02% | 0.03% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.03% | 0.01% | 99% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.09% | -0.11% | -0.03% | 98% | 97% |
| TGM 720p | -0.04% | 0.03% | 0.24% | 100% | 102% |
| Animation | 0.02% | 0.16% | -0.02% | 99% | 99% |
| Mixed content | -0.12% | -0.17% | -0.14% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.01% | 0.02% | 101% | 101% |
| Overall | -0.05% | -0.02% | 0.02% | 100% | 100% |

- Simulation results for dual-tree OFF

Table 2 The BD-rate performance of Method Two when dual-tree is OFF

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.02% | 0.02% | 98% | 100% |
| TGM 720p | 0.01% | -0.01% | 0.05% | 101% | 101% |
| Animation | -0.02% | 0.07% | 0.02% | 104% | 94% |
| Mixed content | 0.00% | 0.00% | 0.01% | 103% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.01% | 96% | 100% |
| Overall | 0.00% | 0.02% | 0.02% | 100% | 99% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | -0.01% | 0.02% | 100% | 95% |
| TGM 720p | -0.09% | 0.02% | 0.12% | 100% | 97% |
| Animation | 0.03% | 0.01% | 0.09% | 97% | 102% |
| Mixed content | -0.02% | 0.07% | 0.02% | 98% | 100% |
| Camera-Captured content | 0.01% | -0.06% | 0.05% | 102% | 99% |
| Overall | -0.02% | 0.01% | 0.06% | 99% | 98% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.02% | 0.02% | 100% | 98% |
| TGM 720p | -0.20% | -0.04% | 0.12% | 100% | 101% |
| Animation | 0.00% | 0.17% | 0.04% | 100% | 101% |
| Mixed content | -0.01% | 0.22% | 0.18% | 101% | 100% |
| Camera-Captured content | 0.02% | 0.02% | 0.10% | 101% | 99% |
| Overall | -0.05% | 0.07% | 0.09% | 100% | 100% |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019, Geneva, Switzerland.

VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode”, JVET-P2022, Oct. 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).