| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0521-v2 |
| --- | --- |

| Title: | Alignment of BDPCM for ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang Hyeongmun Jang, Junghak Nam, Seunghwan Kim, Jaehyun Lim | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com xiaoyuxiu@kwai.com hm.jang@lge.com |
| Source: | Bytedance Inc., Kwai Inc. and LG Electronics Inc. |  |  |

_____________________________

# Abstract

The design of ACT assumes that the prediction of three-colour components follows the same process, which is not the case for BDPCM. This contribution proposes to align prediction process for BDPCM among three colour components for an ACT coded block. Simulation results are summarized as follows.

Overall bitrate saving on top of JVET-QXXX under lossless test condition:

RGB: AI: -2.14%, RA: -1.18%, LDB: -1.12%

YUV: AI: 0.00%, RA: 0.00%, LDB: 0.00%

Overall BD bitrate on top of JVET-QXXX saving under lossy test condition:

RGB: AI: -0.21%, RA: -0.15%, LDB: -0.10%

YUV: AI: 0.00%, RA:0.02%, LDB: -0.02%

# Introduction

Adaptive colour transform (ACT) was adopted at the last meeting. In the current draft [1], when ACT is enabled, BDPCM for the first colour component (luma component) can be used while that for the 2nd and 3rd components (chroma components) are always disabled. This leads to unalignment of prediction process among three colour components, which is not consistent with the ACT design philosophy.

# Proposed Method

This document proposes to align the prediction process for ACT coded blocks when the luma BDPCM mode is enabled. In the proposed method, when the luma BDPCM mode and ACT are both enabled for one block, the usage flag and prediction direction of chroma BDPCM mode are directly inherited from those of the luma BDPCM mode. Accordingly, the flag of chroma BDPCM mode is not needed to be signalled when ACT is enabled.

# Simulation Results

The proposed method is implemented on top of the method in [2]. The anchor is the method proposed in [2]. The simulation follows the test condition used in [2]. The summarized results are given in Table 1-6. Detailed results could be found in the attached spreadsheets.

Table 1 Lossy coding performance of the proposed method compared to [2] under AI condition

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RGB, TGM 1080p | -0.30% | -0.24% | -0.22% | 91% | 96% |
| RGB, TGM 720p | -0.17% | -0.16% | -0.11% | 99% | 98% |
| RGB, Animation | -0.03% | -0.04% | -0.02% | 100% | 98% |
| RGB, Mixed content | -0.29% | -0.20% | -0.22% | 98% | 99% |
| RGB, Camera-Captured content | -0.11% | -0.01% | -0.04% | 104% | 102% |
| YUV, TGM 1080p | -0.01% | -0.01% | -0.01% | 100% | 100% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 101% | 104% |
| YUV, Animation | 0.01% | 0.01% | 0.01% | 100% | 100% |
| YUV, Mixed content | 0.00% | 0.00% | -0.01% | 100% | 100% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 93% | 96% |
| Overall (RGB) | -0.21% | -0.16% | -0.15% | 97% | 98% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 99% | 101% |

Table 2 Lossy coding performance of the proposed method compared to [2] under RA condition

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RGB, TGM 1080p | -0.17% | -0.11% | -0.11% | 99% | 100% |
| RGB, TGM 720p | -0.19% | -0.11% | -0.09% | 99% | 100% |
| RGB, Animation | -0.02% | -0.01% | -0.04% | 102% | 97% |
| RGB, Mixed content | -0.21% | -0.16% | -0.17% | 101% | 102% |
| RGB, Camera-Captured content | -0.03% | 0.03% | -0.01% | 100% | 100% |
| YUV, TGM 1080p | 0.00% | -0.01% | 0.01% | 101% | 101% |
| YUV, TGM 720p | 0.02% | 0.03% | -0.01% | 100% | 99% |
| YUV, Animation | 0.03% | 0.05% | 0.02% | 101% | 101% |
| YUV, Mixed content | 0.05% | 0.02% | 0.06% | 102% | 100% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall (RGB) | -0.15% | -0.09% | -0.10% | 100% | 100% |
| Overall (YUV) | 0.02% | 0.02% | 0.02% | 101% | 100% |

Table 3 Lossy coding performance of the proposed method compared to [2] under LDB condition

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RGB, TGM 1080p | -0.10% | -0.03% | -0.06% | 97% | 98% |
| RGB, TGM 720p | -0.13% | -0.14% | 0.00% | 102% | 101% |
| RGB, Animation | -0.03% | 0.09% | 0.05% | 103% | 100% |
| RGB, Mixed content | -0.15% | -0.07% | -0.17% | 100% | 99% |
| RGB, Camera-Captured content | -0.03% | -0.03% | 0.04% | 101% | 98% |
| YUV, TGM 1080p | 0.01% | 0.02% | 0.00% | 106% | 103% |
| YUV, TGM 720p | 0.06% | 0.01% | -0.29% | 103% | 100% |
| YUV, Animation | 0.00% | 0.00% | 0.02% | 100% | 99% |
| YUV, Mixed content | -0.20% | -0.23% | -0.18% | 99% | 98% |
| YUV, Camera-Captured content | 0.02% | 0.02% | 0.00% | 100% | 101% |
| Overall (RGB) | -0.10% | -0.06% | -0.05% | 100% | 99% |
| Overall (YUV) | -0.02% | -0.03% | -0.10% | 102% | 100% |

Table 4 Lossless coding performance of the proposed method compared to [2] under AI condition

|  | ratio |  | bit-rate savings |
| --- | --- | --- | --- |
|  | Anchor | Proposed |  |
| RGB, TGM 1080p | 26.5 | 26.7 | -0.85% |
| RGB, TGM 720p | 11.4 | 11.9 | -3.34% |
| RGB, Animation | 2.4 | 2.5 | -2.61% |
| RGB, Mixed content | 6.4 | 6.6 | -3.12% |
| RGB, Camera-Captured content | 1.6 | 1.6 | -0.60% |
| YUV, TGM 1080p | 27.6 | 27.6 | 0.00% |
| YUV, TGM 720p | 14.4 | 14.4 | 0.00% |
| YUV, Animation | 6.5 | 6.5 | 0.00% |
| YUV, Mixed content | 8.4 | 8.4 | 0.00% |
| YUV, Camera-Captured content | 1.9 | 1.9 | 0.00% |
| All (RGB) | 12.6 | 12.9 | -2.14% |
| All (YUV) | 13.5 | 13.5 | 0.00% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 95% |  |  |

Table 5 Lossless coding performance of the proposed method compared to [2] under RA condition

|  | ratio |  | bit-rate savings |
| --- | --- | --- | --- |
|  | Anchor | Proposed |  |
| RGB, TGM 1080p | 423.0 | 426.4 | -0.48% |
| RGB, TGM 720p | 111.3 | 115.2 | -2.07% |
| RGB, Animation | 4.0 | 4.1 | -0.41% |
| RGB, Mixed content | 38.0 | 38.6 | -1.84% |
| RGB, Camera-Captured content | 1.3 | 1.3 | -0.19% |
| YUV, TGM 1080p | 440.9 | 440.9 | 0.00% |
| YUV, TGM 720p | 141.8 | 141.8 | 0.00% |
| YUV, Animation | 14.4 | 14.4 | 0.00% |
| YUV, Mixed content | 52.0 | 52.0 | 0.00% |
| YUV, Camera-Captured content | 2.0 | 2.0 | 0.00% |
| All (RGB) | 161.3 | 163.4 | -1.18% |
| All (YUV) | 158.4 | 158.4 | 0.00% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |

Table 6 Lossless coding performance of the proposed method compared to [2] under LDB condition

|  | ratio |  | bit-rate savings |
| --- | --- | --- | --- |
|  | Anchor | Proposed |  |
| RGB, TGM 1080p | 593.9 | 598.1 | -0.43% |
| RGB, TGM 720p | 183.6 | 191.8 | -2.07% |
| RGB, Animation | 4.0 | 4.0 | -0.25% |
| RGB, Mixed content | 41.7 | 42.5 | -1.69% |
| RGB, Camera-Captured content | 1.3 | 1.3 | -0.17% |
| YUV, TGM 1080p | 615.7 | 615.7 | 0.00% |
| YUV, TGM 720p | 237.3 | 237.3 | 0.00% |
| YUV, Animation | 14.0 | 14.0 | 0.00% |
| YUV, Mixed content | 57.2 | 57.2 | 0.00% |
| YUV, Camera-Captured content | 2.0 | 2.0 | 0.00% |
| All (RGB) | 231.6 | 235.3 | -1.12% |
| All (YUV) | 226.8 | 226.8 | 0.00% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

# Text changes

The texts changes are on top of the text proposed in JVET-Q0510 [2]. The newly added texts are highlight by grey. The deleted texts are marked by strikethrough.

intra_bdpcm_chroma_flag equal to 1 specifies that BDPCM is applied to the current chroma coding blocks at the location ( x0, y0 ), i.e. the transform is skipped, the intra chroma prediction mode is specified by intra_bdpcm_chroma_dir_flag. intra_bdpcm_chroma_flag equal to 0 specifies that BDPCM is not applied to the current chroma coding blocks at the location ( x0, y0 ).

When intra_bdpcm_chroma_flag is not present it is inferred to be equal to 0. it is inferred to be equal to sps_bdpcm_chroma_enabled_flag && cu_act_enabled_flag && intra_bdpcm_luma_flag.

The variable BdpcmFlag[ x ][ y ][ cIdx ] is set equal to intra_bdpcm_chroma_flag for x = x0..x0 + cbWidth − 1, y = y0..y0 + cbHeight − 1 and cIdx = 1..2.

intra_bdpcm_chroma_dir_flag equal to 0 specifies that the BDPCM prediction direction is horizontal. intra_bdpcm_chroma_dir_flag equal to 1 specifies that the BDPCM prediction direction is vertical.

When intra_bdpcm_chroma_dir_flag is not present, it is inferred to be equal to ( cu_act_enabled_flag ?  intra_bdpcm_luma_dir_flag : 0 ).

The variable BdpcmDir[ x ][ y ][ cIdx ] is set equal to intra_bdpcm_chroma_dir_flag for x = x0..x0 + cbWidth − 1, y = y0..y0 + cbHeight − 1 and cIdx = 1..2.

# Conclusion

This document proposed that the usage flag and prediction direction of the chroma BDPCM mode for an ACT coded block are inherited from those the luma BDPCM mode of the block directly. It is recommended to adopt the proposed changes to the next version of VVC draft.

# References

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019

- X. Xiao, Y-W Chen, T-C Ma, H-J Jhu, X. Wang, J. Zhao, Hendry, S. Paluri, S-H Kim, W. Zhu, J. Xu, L. Zhang, “ACT Colour Conversion for Both Lossless and Lossy Coding”, JVET-Q0510, Jan. 2020.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).