| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0523 |
| --- | --- |

| Title: | Redistribution of chroma information for improved HDR color representation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | informatio |  |  |
| Author(s) or Contact(s): | Maryam Azimi University of Cambridge Cambridge, UK Mahsa T. Pourazad Telus Communications Inc. Vanocuver, Canada Panos Nasiopoulos University of British Columbia Vancouver, Canada | Email: | maryam.azimih@gmail.com mahsa.pourazad@gmail.com panos@ece.ubc.ca |
| Source: | University of British Columbia (UBC) and TELUS Communications Inc. |  |  |

_____________________________

# Abstract

We propose a chroma processing method that adds to the existing NCL YCbCr 4:2:0 and ICtCp color encoding to reduce their visible color errors for encoding HDR color pixels. We identify the colors that are represented with visible color shifts using 10-bit YCbCr 4:2:0 and ICtCp 4:2:0 in terms of CIE DE2000 and DEITP. We then redistribute code-words for their chroma channels using a non-linear function such that those colors that bear larger color differences due to bit-depth quantization are given more code-words. The quantized new YCb*Cr* and ICt*Cp* (the star script to differentiate them from their originals) signals are then compressed with VTM xxx to evaluate the compression efficiency of the proposed method.

# Proposed chroma redistribution function

In this document we propose a chroma processing for YCbCr and ICtCp color to assign more code-words during the bit-depth quantization to those colors our visual system is most sensitive to. We first identify how different colors are distorted due to bit-depth quantization and whether these distortions are visible to our eyes. We use the CIE DE2000 and DEITP (BT.2124) [1] perception-based color difference metric to calculate the color differences between the original RGB color pixels and the ones represented by 10-bit YCbCr and ICtCp.

The original and encoded colors are constructed based on the process explained in [2]. Figs. 1 and 2 show the color differences between quantized YCbCr and ICtCp and the original linear RGB colors in terms of DE2000 at selected sampled luminance levels and bit-depths using a heatmap for errors (the bar shown in Figs. 1 and 2).

The first observation from Fig. 1 and Fig. 2 is that the color differences due to quantization are mainly visible around the white point as the chroma bit-depths are reduced. Encoded saturated colors at the border of the gamut are still visually intact at 8 bits per chroma channel. This implies that colors around white point require more code-words than the ones further away. In other words, saturated colors can be quantized with fewer bits and still be visually similar to the original color. This suggests a non-linear behavior in chroma channels in terms of required code-words. In [2], we proposed such a function for YCbCr which we will use in this document for ICtCp as well.  Using the function in [2], most of the available code-words are allocated to colors around the white point with chroma values between [-0.2 and 0.2].

The second observation from Fig. 1 and Fig. 2 is that the color errors are more intense along the vertical axis (v’), from color yellow to blue. This observation implies a different behavior for Cr and Cb (or Ct and Cp), indicating that a different code-words redistribution function may be needed for representing each chroma channel. However, in this work we focus on using a common transfer function for both. A final observation from Fig. 1 and Fig 2 is that ICtCp requires one fewer bit compared to YCbCr for its chroma channels while showing the same performance in terms of both DE2000.

Fig. 3 and Fig. 4 show the same color differences in terms of DEITP. By comparing Figs. 1 and 2 and Figs. 3 and 4, we notice that DE2000 and DEITP  behave differently in predicting color differences. In this work, we only consider DE2000 results.

# Results

Using the function in [2], we transfer the chroma channels of YCbCr and ICtCp before quantization. Fig. 1 (d) and Fig. 2 (d) show how the proposed transfer function reduces the visible color errors in terms of DE2000 compared to the original case. Table I reports how the number of pixels (in %) with visible color errors, as well as the average error over the entire frame is reduced when the proposed method is used compared to the original 10-bit YCbCr and ICtCp, in 4:4:4 and 4:2:0 formats for three of the HDR videos.

Table II provides the BD-rate results of the proposed 10-bit YCb*Cr* and ICt*Cp*  in terms of PSNR L100, Y-wPSNR, U-wPSNR, V-wPSNR, and DE100 when compressed with the original 10-bit NCL YCbCr and ICtCp as the anchor.

# References

ITU BT.2124, “Objective metric for the assessment of the potential visibility of colour differences in television”

Fig. 1. Color errors generated due to bit-depth quantization for (a) 10-bit YCbCr, (b) Y10CbCr9, and (c) Y10CbCr8, over the BT.2020 gamut using the error bars on the right, in terms of DE2000. Numbers indicate the used bit-depth.

JCTVC AK0029, “Perceptual redistribution of chroma information for improved HDR color representation”.

Fig. 3. Color errors generated due to bit-depth quantization for (a) 10-bit YCbCr, (b) Y10CbCr9, and (c) Y10CbCr8, (d) over the BT.2020 gamut using the error bars on the right, in terms of DEITP. Numbers indicate the used bit-depth.

Fig. 2. Color errors generated due to bit-depth quantization for (a) 10-bit ICtCp, (b) I10CtCp9, and (c) I10CtCp8 over the BT.2020 gamut using the error bars on the right, in terms of DE2000. Numbers indicate the used bit-depth.

Fig. 2. Color errors generated due to bit-depth quantization for (a) 10-bit ICtCp, (b) I10CtCp9, and (c) I10CtCp8 over the BT.2020 gamut using the error bars on the right, in terms of DE2000. Numbers indicate the used bit-depth.

Fig. 4. Color errors generated due to bit-depth quantization for (a) 10-bit ICtCp, (b) I10CtCp9, and (c) I10CtCp8 over the BT.2020 gamut using the error bars on the right, in terms of DEITP. Numbers indicate the used bit-depth.

Fig. 4. Color errors generated due to bit-depth quantization for (a) 10-bit ICtCp, (b) I10CtCp9, and (c) I10CtCp8 over the BT.2020 gamut using the error bars on the right, in terms of DEITP. Numbers indicate the used bit-depth.

Fig. 5. Color errors generated due to bit-depth quantization for (a) proposed Y10Cb*Cr*10, (b) proposed ICt*Cp *10 over the BT.2020 gamut using the error bars on the right, in terms of DE2000. Numbers indicate the used bit-depth.

Fig. 5. Color errors generated due to bit-depth quantization for (a) proposed Y10Cb*Cr*10, (b) proposed ICt*Cp *10 over the BT.2020 gamut using the error bars on the right, in terms of DE2000. Numbers indicate the used bit-depth.

Table 1 Average DE2000 and percentage of pixels with DE2000 values greater than one (visible error) represented with the original YCbCr and ICtCp and the proposed YCb*Cr* and ICt*Cp*with 4:4:4 and 4:2:0 chroma, compared to the originals (first frame)

| Sequence | Method | % of pixels with DE > 1 |  | Average DE of frame |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4:4:4 | 4:2:0 | 4:4:4 | 4:2:0 |
| SunRise | YCbCr10 | 62.38 | 88.94 | 1.80 | 5.26 |
|  | Proposed YCb*Cr*10 | 29.67 | 85.91 | 0.8128 | 4.86 |
|  | ICtCp10 | 25.8517 | 85.6161 | 0.7404 | 4.7387 |
|  | Proposed ICt*Cp*10 | 3.14 | 84.5524 | 0.3451 | 4.6147 |
| Market | YCbCr10 | 42.35 | 86.52 | 1.2062 | 6.07 |
|  | Proposed YCb*Cr*10 | 10.98 | 84.81 | 0.4792 | 5.8067 |
|  | ICtCp10 | 11.0317 | 84.5210 | 0.4946 | 5.7889 |
|  | Proposed ICt*Cp*10 | 1.04 | 84.0899 | 0.2423 | 5.7389 |
| BalloonFestival | YCbCr10 | 3.79 | 33.51 | 0.425 | 2.7661 |
|  | Proposed YCb*Cr*10 | 0.18 | 29.60 | 0.1819 | 2.6294 |
|  | ICtCp10 | 0.2410 | 29.5615 | 0.2089 | 2.6432 |
|  | Proposed ICt*Cp*10 | 0.01 | 29.2524 | 0.1219 | 2.6009 |

Table 2 Average Objective Results of Proposed 10-bit YCb*Cr* Compared to the Anchor of 10-bit YCbCr (VTM 7.0)

| Sequence | Metrics |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPSNRY | wPSNRU | wPSNRV |
| Hurdles | % | % | % | % | % |
| Market | % | % | % | % | % |
| BalloonFestival | % | % | % | % | % |
| ShowGirl | % | % | % | % | % |
| Starting | % | % | % | % | % |
| SunRise | % | % | % | % | % |
| Overall | % | % | % | % | % |

Patent rights declaration(s)

University of British Columbia and TELUS Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TABLE II. Average Objective Results of Proposed 10-bit YCb*Cr* Compared to the Anchor of 10-bit YCbCr
(Negative Values Indicate an Improvement Over the Anchor, While Positive Values Indicate a Loss)

| Sequence | Metrics |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | tPSNR X | tPSNR Y | tPSNR Z | tPSNR XYZ | tOSNR XYZ |
| FireEater2 | -1.3% | 10.6% | -21.9% | -3.6% | -19.7% |
| Market3 | 10.4% | 13.5% | -6.1% | 4.9% | 0.0% |
| BalloonFestival | 6.3% | 18.6% | -23.5% | -3.0% | -20.3% |
| Overall | 5.2% | 14.2% | -17.1% | -0.6% | -13.3% |