| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0524 |
| --- | --- |

| Title: | Non-CE4: DC balancing in DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou Maryam Homayouni | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Decoder-side motion vector refinement (DMVR) refines the accuracy of motion vectors of merge mode by minimizing the sum of absolute difference (SAD) between the prediction blocks in reference picture lists L0 and L1. The refined MV might not be accurate in the case that two prediction blocks are not in the same level of brightness, for example in the case of fading scenes or lightening condition change in different frames. It is proposed to balance the DC component of two prediction blocks for SAD calculation. For this purpose, DC value of prediction blocks of L0 and L1 is calculated, and then delta DC value is subtracted from sample differences during SAD calculation. This modification stabilizes the functionality of DMVR in different situations. In the case of RA, the average BD-rate gain is reported to be 0.10% (0.17% gain for class A1 sequences).

# Introduction/Problem statement

In the latest VVC draft, DMVR can be applied for the CUs which are coded in a specific situation, for example with following modes and features:

- CU level merge mode with bi-prediction MV

- One reference picture is in the past and another reference picture is in the future with respect to the current picture

- The distances (i.e. POC difference) from both reference pictures to the current picture are same

- CU has more than 128 luma samples

- Both CU height and CU width are larger than or equal to 8 luma samples

- BCW (Bi-prediction with CU weights) weight index indicates equal weight

- WP is not enabled for the current block

In DVMR, prediction block is partitioned to 16x16 subblocks, and MV refinement is performed for each 16x16 subblocks, separately. MV refinement (i.e. MVoffset) obeys the following rule, where MV0 and MV1 are the initial merge motion vectors, and MV0' and MV1' are the refined motion vectors.

MV0'=MV0+MVoffset

MV1'=MV1-MVoffset

MVoffset search is performed in two steps, including integer offset search, and fractional offset calculation. In integer offset search, 25 search points (i.e. 5 in horizontal and 5 in vertical directions) are searched. Search points in each direction are {-2, -1, 0, 1, +2} in full-pel Luma precision. For each search point, prediction blocks (i.e. Pred0' and Pred1') are generated using the refined MV (i.e. MV0' and MV1') and SAD is calculated (only even rows are used).

SAD(MVoffset)=j=0,j+=2H-1i=0W-1absPred0'i,j-Pred1'i,j

If the SAD of the initial MV pair (i.e. MVoffset=(0,0)) is smaller than a threshold, the integer sample stage of DMVR is terminated. Otherwise SADs of the 25 search points (i.e. the initial point and remaining 24 points) are calculated, and the point with the smallest SAD is selected as the output of integer sample offset search stage. The initial MV pair is prioritized by multiplying its SAD value to a factor of 0.75. The integer sample search is followed by fractional sample refinement, that calculates the fractional MVoffset based on the SAD values of the search point around the minimum integer point.

During integer sample search, as the MVoffset is an integer number in full-pel precision, most of the prediction samples which are involved in SAD calculation are shared. So, initial prediction blocks are generated with blocks size of (Width+2*DMVR_SR)*(Height+2*DMVR_SR) in each reference frame (DMVR_SR=2). During integer offset search, samples of prediction blocks (of size width*height) for each search point are directly extracted from this initial prediction blocks. To reduce computational complexity and memory access, these initial prediction blocks are generated using bi-linear interpolation filter.

# Proposal

The main idea of the proposal is to balance DC difference (i.e., DeltaDC) of two prediction blocks in the SAD calculation. So, the SAD value for each search point is calculated using the following equation:

SAD(MVoffset)=j=0,j+=2H-1i=0W-1absPred0'i,j-Pred1'i,j-DeltaDCMVoffset

To calculate DeltaDC values, two methods have been test as described below.

## Method1: DetlaDC calculation

In this method, DeltaDC is calcualtion for each search point separately. For this purpose, DC is calculated for both each prediction blocks (i.e., Pred0' and Pred1') using (only even rows are used)

DCL(MVoffset)=j=0, j+=2H-1i=0W-1PredL'i,j

Then DeltaDC is calculated as the difference between DC values of reference list L0 and L1.

DDC= DC0MVoffset-DC1MVoffset

num_shift=floorLog2W+floorLog2W-1

offset=1≪num_shift -1

DDC=DDC>0 ? DDC+offset≫num_shift:--DDC+offset≫num_shift

DeltaDCMVoffset= DDC

Method1 is more suitable to be implemented before SAD calculation.

## Method2: DeltaDC interpolation

In this method, first DC values are calculated for four corner search points as mentioned above. Four corner search points are MVoffset = {(–2, –2), (+2, –2), (–2, +2), (+2 , +2)}.

And then DC values for other 21 remaining search points of MVoffset=(x,y) are estimated based on these four corner DC values using bi-linear interpolations as below:

DCLMVoffset=DC-2,-2×4-x×4-y+DC+2,-2×x×4-y+DC-2,+2×4-x×y+DC+2,+2×x×y≫4

Then DeltaDC values is calculated as described above for method 1.

Method2 is more suitable to be implemented after generating the initial prediction blocks (i.e., after initial motion compensation function). in VTM implementation, right after initial motion compensation, DC values are calculated for 4x4 subblocks, and corner DC values are calculating by accumulating DC values of the corresponding 4x4 blocks.

# Objective results

Results according to the JVET CTC are provided in the attached spreadsheet for two cases. Summary of the results is provided below. As shown in the following table, applying the proposal contributes to 0.10%  average BD-rate gain in RA test case, and 0.17% gain for class A1 sequences.

Table 1. Method 1: DC calculation

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16 % | -0.11 % | -0.08 % | 103 % | 99 % |
| Class A2 | -0.09 % | -0.12 % | -0.08 % | 103 % | 100 % |
| Class B | -0.09 % | -0.03 % | -0.06 % | 104 % | 101 % |
| Class C | -0.04 % | 0.04 % | 0.00 % | 106 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.09 % | -0.05 % | -0.05 % | 104 % | 100 % |
| Class D | -0.02 % | 0.07 % | 0.02 % | 106 % | 101 % |
| Class F | -0.07 % | -0.03 % | -0.03 % | 104 % | 101 % |

Table 2. Method 1: DC interpolation

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17 % | -0.07 % | -0.14 % | 101 % | 100 % |
| Class A2 | -0.09 % | -0.09 % | -0.10 % | 101 % | 103 % |
| Class B | -0.10 % | -0.06 % | -0.06 % | 101 % | 102 % |
| Class C | -0.07 % | 0.00 % | 0.03 % | 99 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.10 % | -0.05 % | -0.06 % | 101 % | 102 % |
| Class D | -0.03 % | -0.03 % | 0.07 % | 99 % | 100 % |
| Class F | -0.05 % | -0.06 % | -0.06 % | 100 % | 101 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).