| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0561-v1 |
| --- | --- |

| Title: | CE3-related: Transquant Bypass Mode for Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Marta Karczewicz, Hongtao Wang, Hilmi E. Egilmez, Muhammed Coban, 5775 Morehouse Dr. San Diego, CA, 92121, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to bring back HEVC like transquant bypass mode into VVC for lossless coding. Currently, lossless coding is possible via existing syntax with TS residual coding. However, TS residual coding has lesser bit-rate savings as compared to regular coefficient coding as observed in the 16th Geneva meeting. As a result, CE3 has many tests for signaling either a high-level flag or a low-level flag for switching between TS residual coding and regular residual coding. In the case of transquant bypass, transform and quantization steps are simply bypassed using a coding unit (CU) level flag. This flag does not only provide a CU level control for lossless coded blocks but is also more flexible for doing mixed lossy and lossless coding.

- The simulation results show overall bit-rate savings of -5.32% AI, -5.62% RA, and -5.00% LDB with Class F: -3.08% AI, -3.73% RA, and -3.54% LDB and Class TGM: -3.19% AI, -2.59% RA, and -2.84% LDB.

- The simulation results with Luma BDPCM enabled show overall bit-rate savings of  X% AI, X% RA, and -X% LDB with Class F: X% AI, X% RA, and X% LDB and Class TGM: X% AI, X% RA, and X% LDB.

- The simulation results with Luma and Chroma BDPCM enabled show overall bit-rate savings of  X% AI, X% RA, and -X% LDB with Class F: X% AI, X% RA, and X% LDB and Class TGM: X% AI, X% RA, and X% LDB.

# Introduction

In VTM-7.0, lossless coding is possible via existing syntax with TS enabled both for luma and chroma [1]. This form of lossless coding can only use transform skip residual coding (TSRC) and is unable to switch between TSRC and regular residual coding (RRC). In the 16th JVET Geneva meeting it was observed that RRC had more coding-gains as compared to doing TSRC [2]. CE3 was established to investigate both bit-rate savings for both residual coding methods for lossless coding as well as whether low-level signaling is necessary for lossless coding.

# Proposed Method

In this document, it is proposed to bring back transquant bypass mode of HEVC into VVC, where a CU-level transquant bypass flag is for each CU that indicates transform and quantization steps are bypassed. Note that the proposed implementation replicates the code of JVET-P0082 for transquant bypass mode with fixes related to BDPCM.

# Results

For the anchor VTM-7.0 with “lossless.cfg” config file on top of the CTC configurations. For the proposed tests “TransquantBypassEnable=1” and “CUTransquantBypassFlagForce =1” are added on top of the provided “lossless.cfg” config file. BDPCM is disabled for both camera captured and screen content. As outlined in CE3 description, the experiments were run for 100 frames only for AI, RA and LDB configurations.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Proposal |  | Anchor | Proposal |  | Anchor | Proposal |  |
| Class A1 | 2.2 | 2.4 | -5.56% | 2.2 | 2.4 | -6.21% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.86% | 1.7 | 1.9 | -8.09% |  |  |  |
| Class B | 2.2 | 2.3 | -4.08% | 2.3 | 2.4 | -3.87% | 2.3 | 2.4 | -3.88% |
| Class C | 1.9 | 2.0 | -5.07% | 2.4 | 2.6 | -5.50% | 2.4 | 2.6 | -5.42% |
| Class D | 1.9 | 2.0 | -4.68% | 2.8 | 2.9 | -5.38% | 2.7 | 2.9 | -5.36% |
| Class E | 2.8 | 2.9 | -3.91% |  |  |  | 3.1 | 3.3 | -6.31% |
| Class F | 5.3 | 5.5 | -3.08% | 33.7 | 34.7 | -3.73% | 50.7 | 52.1 | -3.54% |
| TGM | 11.8 | 12.2 | -3.19% | 107.1 | 109.4 | -2.59% | 124.9 | 128.1 | -2.84% |
| Overall | 2.1 | 2.2 | -5.32% | 2.2 | 2.3 | -5.62% | 2.6 | 2.7 | -5.00% |
| Enc Time[%] | 91% |  |  | 95% |  |  | 94% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  | 109% |  |  |

Table 1. Lossless Coding with Transquant Bypass Mode (No BDPCM)

(pending results)

Table 2. Lossless Coding with Transquant Bypass Mode (Luma BDPCM)

(pending results)

Table 3. Lossless Coding with Transquant Bypass Mode (Luma and Chroma BDPCM)

# Recommendation

It is recommended to adopt transquant bypass mode for lossless coding in VVC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz “AHG18: BDPCM for Lossless,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P0082, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).