| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0565 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0458: CE4-Related: On-the-fly Weighting Index to Sample Blending Weight Conversion without Table Look-Up on Top of Combined Test in Q0061 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao | Tel: Email: | han.gao@huawei.com |
| Source: | Huawei |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking of JVET-Q0458. One method is proposed in JVET-Q0458. It’s reported that the WD and software are matching with the descripted methods. Simulation results of this report are matched with the results provided by the proponent of JVET-Q0458

# Introduction

In JVET-Q0458, the following method is proposed

## On-the-fly Weighting Index to Weight Calculation

In order to reduce the implementation cost of look-up table and ease weighting derivation in parallel, the blending weight of samples could be derived with the following equations.

| wIdxL | =1≪idx2wShiftBit+2+partIdx ? wIdx : -wIdx | (1) |
| --- | --- | --- |

| weight | =Clip30, 8,wIdxL+1≪idx2wShiftBit-1≫idx2wShiftBit | (2) |
| --- | --- | --- |

Where idx2wShiftBit is the conversion factor of the weighting index to weight, and its typical value in Eq. (2) is 3.

It’s verified by the crosschecker that both the WD and software are matching the descripted methods. Simulations results for the proposed is matched with the results provided by the proponent of JVET-Q0458 based on the simulation results in the following Table

The test is on top of JVET-Q0061 combined test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.23% | -0.24% | 102% | 100% |
| Class A2 | -0.17% | -0.14% | -0.13% | 102% | 100% |
| Class B | -0.14% | -0.10% | -0.18% | 102% | 100% |
| Class C | -0.53% | -0.86% | -0.89% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.34% | -0.37% | 102% | 100% |
| Class D | -0.33% | -0.56% | -0.49% | 102% | 100% |
| Class F (optional) | -0.23% | -0.29% | -0.25% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.58% | -0.45% | 102% | 98% |
| Class C | -0.60% | -1.15% | -0.76% | 103% | 99% |
| Class E | -0.87% | 0.13% | -0.71% | 100% | 100% |
| Overall | -0.52% | -0.59% | -0.62% | 102% | 99% |
| Class D | -0.45% | -0.83% | -0.66% | 102% | 99% |
| Class F (optional) | -0.44% | -1.01% | 0.03% | 101% | 99% |

# Patent rights declaration(s)

Huawei does not have any current or pending patent rights relating to the technology described in this contribution.