| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0584 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0521 (Alignment of BDPCM for ACT) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Huan Dou, Lidong Xu, Yi-jen Chiu 8F Raycom Infotech Park A, Haidian District, Beijing, 100029, China | Tel: Email: | +86-10-82611515 huan.dou@intel.com lidong.xu@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for JVET-Q0521 (Alignment of BDPCM for ACT) which proposes to infer the chroma mode to be BDPCM when the luma BDPCM mode and ACT are both enabled for a block. The crosschecker verified that the software implementation matches the description and the simulation BD-rate results reportedly match those provided by the proponents.

# Verification

The proposal JVET-Q0521 proposes to infer the chroma mode to be BDPCM when the luma BDPCM mode and ACT are both enabled for a block. It was verified that the implementation in the provided software matches the description, and the BD rates reported in the proposal were reproduced.

## Performance in lossy coding

Table 1-3 showed the performance data of AI, RA, and LDB, respectively, under lossy coding configuration. The EncT and DecT data are not accurate.

Table 1. Performance under lossy coding AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RGB, TGM 1080p | -0.30% | -0.24% | -0.22% | 92% | 94% |
| RGB, TGM 720p | -0.17% | -0.16% | -0.11% | 98% | 99% |
| RGB, Animation | -0.03% | -0.04% | -0.02% | 100% | 96% |
| RGB, Mixed content | -0.29% | -0.20% | -0.22% | 100% | 101% |
| RGB, Camera-Captured content | -0.11% | -0.01% | -0.04% | 100% | 100% |
| YUV, TGM 1080p | -0.01% | -0.01% | -0.01% | 100% | 103% |
| YUV, TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| YUV, Animation | 0.01% | 0.01% | 0.01% | 102% | 102% |
| YUV, Mixed content | 0.00% | 0.00% | -0.01% | 101% | 104% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Overall (RGB) | -0.21% | -0.16% | -0.15% | 97% | 98% |
| Overall (YUV) | 0.00% | 0.00% | 0.00% | 100% | 101% |

Table 2. Performance under lossy coding RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RGB, TGM 1080p | -0.17% | -0.11% | -0.11% | 97% | 99% |
| RGB, TGM 720p | -0.19% | -0.11% | -0.09% | 98% | 100% |
| RGB, Animation | -0.02% | -0.01% | -0.04% | 102% | 100% |
| RGB, Mixed content | -0.21% | -0.15% | -0.17% | 98% | 99% |
| RGB, Camera-Captured content | -0.03% | 0.03% | -0.01% | 99% | 97% |
| YUV, TGM 1080p | 0.00% | -0.01% | 0.01% | 100% | 100% |
| YUV, TGM 720p | 0.02% | 0.03% | -0.01% | 104% | 105% |
| YUV, Animation | 0.03% | 0.05% | 0.02% | 107% | 106% |
| YUV, Mixed content | 0.05% | 0.02% | 0.06% | 104% | 103% |
| YUV, Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Overall (RGB) | -0.15% | -0.09% | -0.10% | 99% | 99% |
| Overall (YUV) | 0.02% | 0.02% | 0.02% | 103% | 103% |

Table 3. Performance under lossy coding LDB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RGB, TGM 1080p | -0.10% | -0.03% | -0.06% | 100% | 103% |
| RGB, TGM 720p | -0.13% | -0.14% | 0.00% | 99% | 96% |
| RGB, Animation | -0.03% | 0.09% | 0.05% | 99% | 98% |
| RGB, Mixed content | -0.15% | -0.07% | -0.17% | 100% | 99% |
| RGB, Camera-Captured content | -0.03% | -0.03% | 0.04% | 98% | 101% |
| YUV, TGM 1080p | 0.01% | 0.02% | 0.00% | 101% | 104% |
| YUV, TGM 720p | 0.06% | 0.01% | -0.29% | 100% | 100% |
| YUV, Animation | 0.00% | 0.00% | 0.02% | 102% | 101% |
| YUV, Mixed content | -0.20% | -0.23% | -0.18% | 99% | 103% |
| YUV, Camera-Captured content | 0.02% | 0.02% | 0.00% | 100% | 102% |
| Overall (RGB) | -0.10% | -0.06% | -0.05% | 99% | 100% |
| Overall (YUV) | -0.02% | -0.03% | -0.10% | 100% | 102% |

## Performance in lossless coding

Table 4-6 showed the performance data of AI, RA, and LDB, respectively, under lossless coding configuration. The EncT and DecT data are not accurate.

Table 4. Performance under lossless coding AI configuration

|  | ratio |  | bit-rate savings |
| --- | --- | --- | --- |
|  | Anchor | Proposed |  |
| RGB, TGM 1080p | 26.5 | 26.7 | -0.85% |
| RGB, TGM 720p | 11.4 | 11.9 | -3.34% |
| RGB, Animation | 2.4 | 2.5 | -2.61% |
| RGB, Mixed content | 6.4 | 6.6 | -3.12% |
| RGB, Camera-Captured content | 1.6 | 1.6 | -0.60% |
| YUV, TGM 1080p | 27.6 | 27.6 | 0.00% |
| YUV, TGM 720p | 14.4 | 14.4 | 0.00% |
| YUV, Animation | 6.5 | 6.5 | 0.00% |
| YUV, Mixed content | 8.4 | 8.4 | 0.00% |
| YUV, Camera-Captured content | 1.9 | 1.9 | 0.00% |
| All (RGB) | 12.6 | 12.9 | -2.14% |
| All (YUV) | 13.5 | 13.5 | 0.00% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 97% |  |  |

Table 5. Performance under lossless coding RA configuration

|  | ratio |  | bit-rate savings |
| --- | --- | --- | --- |
|  | Anchor | Proposed |  |
| RGB, TGM 1080p | 423.0 | 426.4 | -0.48% |
| RGB, TGM 720p | 111.3 | 115.2 | -2.07% |
| RGB, Animation | 4.0 | 4.1 | -0.41% |
| RGB, Mixed content | 38.0 | 38.6 | -1.84% |
| RGB, Camera-Captured content | 1.3 | 1.3 | -0.20% |
| YUV, TGM 1080p | 440.9 | 440.9 | 0.00% |
| YUV, TGM 720p | 141.8 | 141.8 | 0.00% |
| YUV, Animation | 13.7 | 13.7 | 0.00% |
| YUV, Mixed content | 52.0 | 52.0 | 0.00% |
| YUV, Camera-Captured content | 2.0 | 2.0 | 0.00% |
| All (RGB) | 161.3 | 163.4 | -1.18% |
| All (YUV) | 158.2 | 158.2 | 0.00% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 96% |  |  |

Table 6. Performance under lossless coding LDB configuration

|  | ratio |  | bit-rate savings |
| --- | --- | --- | --- |
|  | Anchor | Proposed |  |
| RGB, TGM 1080p | 593.9 | 598.1 | -0.43% |
| RGB, TGM 720p | 183.6 | 191.8 | -2.07% |
| RGB, Animation | 4.0 | 4.0 | -0.25% |
| RGB, Mixed content | 41.7 | 42.5 | -1.69% |
| RGB, Camera-Captured content | 1.3 | 1.3 | -0.17% |
| YUV, TGM 1080p | 615.7 | 615.7 | 0.00% |
| YUV, TGM 720p | 237.3 | 237.3 | 0.00% |
| YUV, Animation | 14.0 | 14.0 | 0.00% |
| YUV, Mixed content | 57.2 | 57.2 | 0.00% |
| YUV, Camera-Captured content | 2.0 | 2.0 | 0.00% |
| All (RGB) | 231.6 | 235.3 | -1.12% |
| All (YUV) | 226.8 | 226.8 | 0.00% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 101% |  |  |