| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0590-v1 |
| --- | --- |

| Title: | AHG12: A summary of proposals on subpicture ID signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on subpicture ID signalling submitted to this JVET meeting. It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on subpicture ID signalling

This category of proposals is listed in the agenda item 6.20.1.3, including 9 contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0119 AHG12: Cleanups on signalling of subpictures, tiles, and rectangular slices [Y.-K. Wang (Bytedance)]

Items 2-5 of this contribution belong to this category.

2) Condition sps_subpic_id_present_flag on "if( subpics_present_flag )", i.e., the syntax element sps_subpic_id_present_flag is not signalled when subpics_present_flag is equal to 0, and infer the value of sps_subpic_id_present_flag to be equal to 0 when it is not present. For easier understanding, the names of subpics_present_flag and sps_subpic_id_present_flag are suggested to be changed to subpic_info_present_flag and subpic_ids_explicitly_signalled_flag, respectively.

3) Remove the signalling of subpicture IDs from the PH syntax.

4) When subpicture IDs are indicated to be explicitly signalled, they are signalled either in the SPS or in the PPS, cannot be in both, and cannot be in neither. For easier understanding, the names of sps_subpic_id_signalling_present_flag and pps_subpic_id_signalling_present_flag are suggested to be changed to subpic_ids_in_sps_flag and subpic_ids_in_pps_flag, respectively.

5) When subpics_present_flag is equal to 1, the length of subpicture IDs is explicitly signalled in the SPS regardless of whether subpicture IDs are explicitly signalled, and the length of slice_subpic_id is always specified by that length.

JVET-Q0290 AHG9/AHG12: Modifications related to subpicture signalling and RPR [S.-T. Hsiang, C.-Y. Chen, T.-D. Chuang, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Item 3 of this contribution belongs to this category.

3. When subpics_present_flag is equal to 0, sps_subpic_id_present_flag is inferred to be equal to 0.

JVET-Q0152 AHG9/AHG12: Miscellaneous HLS topics [M. Coban, V. Seregin, Y.-J. Chang, M. Karczewicz (Qualcomm)]

Item 1 of this contribution belongs to this category.

1) Restrict explicit subpicture ID signalling to the case where all signalled subpictures are treated as pictures.

JVET-Q0213 [AHG9/AGH12] On subpicture ID mapping signaling [Y. He, A. Hamza (InterDigital)]

This contribution proposes two options to modify subpicture signaling to reduce subpicture ID mapping redundancy.

1. Option 1 proposes to signal subpicture sets in SPS, each subpicture set includes a number of subpictures applying to one CLVS of the CVS referring to the SPS.

2. Option 2 proposes to signal the subpicture ID mapping for those subpictures in a coded picture associated with the PPS and/or PH.

Justifications: A single SPS may be referred by multiple CLVSs, each with different subpicture boundaries, of the CVS, a subset of these subpictures signalled in SPS may apply to a CLVS. As a result, it is redundant to signal all subpicture ID mapping in PPS and/or PH that is referred by a CLVS.

JVET-Q0222 AHG12: On Subpicture Signalling [S. Deshpande (Sharp)]

Item 3 and 5 of this contribution belong to this category.

Proposal 3: Condition slice_subpic_id on "if( subpics_present_flag  | |  sps_subpic_id_present_flag )", or alternatively, skip signalling of all the subpicture ID syntax elements in the SPS when subpics_present_flag is equal to 0.

Proposal 5: Add the following constraint: When sps_subpic_id_present_flag is equal to 1, at least one of sps_subpic_id_signalling_present_flag, ph_subpic_id_signalling_present_flag and pps_subpic_id_signalling_present_flag shall be equal to 1.

JVET-Q0235 AHG12 Simplifying the nesting condition of subpicIdList[i] [M. Hirabayashi, M. Katsumata, T. Suzuki (Sony)]

This contribution proposes to replace the sps_subpic_id_present_flag with a new flag indicating whether to describe subpicture ID mapping in SPS. This new flag could simplify the nesting condition of the list subpicIdList[ i ].

JVET-Q0285 AHG9: On adaptation parameter set ID [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Item 1 of this contribution belongs to this category.

Proposal 1: Method for identification of APSs associated with a subpicture for bitstream extraction and merging, by signaling the list of associated subpicture IDs in each APS.

JVET-Q0375 AHG9: A cleanup on the signalling of subpicture IDs [D. Kim, G. Ko, J. Jung, J. Son, J. Kwak (WILUS)]

This contribution proposes adding a condition to the semantics of pps_subpic_id_signalling_present_flag or sps_subpic_id_present_flag to signal the subpicture IDs in at least one of SPS, PPS, and PH when sps_subpic_id_present_flag in SPS is equal to 1.

JVET-Q0412 AHG9/AHG12: A syntax and semantics fix for subpicture ID mapping [J. Chen, R.-L. Liao, Y. Ye, J. Luo (Alibaba)]

In VVC draft 7, the subpictue ID may be signaled in either SPS, PPS or PH, or not signalled in the bitstream at all. According to current syntax, when sps_subpic_id_present_flag is equal to 1, the subpicture ID may not be signalled in SPS, which conflicts with semantics of sps_subpic_id_present_flag. Moreover, in the case of sps_subpic_id_present_flag equal to 1 but subpicture ID not being signalled in SPS, PPS or PH, the subpicture ID list derived in decoder side becomes undefined, according to the current subpicture ID list derivation rules. Thus, two methods are proposed in this contribution to solve these two problems.

In method 1, when sps_subpic_id_present_flag is equal to 1 and subpicture ID is not signalled in SPS or PPS, it is mandatory for subpicture ID to be signaled in PH in order to avoid the case of having undefined subpicture ID list. And the semantics of sps_subpic_id_present_flag is changed to specify the signaling of subpicture ID in the SPS, PPS or PH.

In method 2, an inferred value is assigned to pps_subpic_id[ i ], so that the inferred ID will be used in the case when sps_subpic_id_present_flag is equal to 1 but subpicture ID is not signalled in SPS, PPS or PH. And the semantics of sps_subpic_id_present_flag is changed to specify the possibility of signaling of subpicture ID in the SPS, PPS or PH.

# List of design questions

- When subpics_present_flag is equal to 0, i.e., there is no subpicture layerout informaton signalled and no subpicture ID signalled in SHs, do we allow subpicture ID mapping to be explicitly signalled?

  - Yes.

    - Condition slice_subpic_id on "if( subpics_present_flag  | |  sps_subpic_id_present_flag )". (JVET-Q0222)

  - No. (JVET-Q0119, JVET-Q0290, JVET-Q0222)

    - Condition sps_subpic_id_present_flag on "if( subpics_present_flag )" and infer the value of sps_subpic_id_present_flag to be equal to 0 when it is not present. (JVET-Q0119, JVET-Q0290, JVET-Q0222)

    - Constrain the value of sps_subpic_id_present_flag to be equal to 0 when subpics_present_flag is equal to 0.

- Remove signalling of the subpicture ID mapping from PH?

  - Yes, otherwise subpicture based extraction would have to change PHs. (JVET-P0119)

  - No.

- When the subpicure ID mapping is indicated to be explicitly signalled, it has to be explicitly signalled in at least one place?

  - Yes. (JVET-P0119, JVET-P0222, JVET-P0375, JVET-P0412)

  - No.

    - An inferred value is assigned to pps_subpic_id[ i ] when sps_subpic_id_present_flag is equal to 1 but the subpicture ID mapping is not signalled in SPS, PPS or PH. (JVET-P0412)

- When subpics_present_flag is equal to 1, the subpicture ID length is explicitly signalled in the SPS regardless of whether subpicture IDs are explicitly signalled, and the lenght of slice_subpic_id is always specified by that length.

  - Yes (JVET-P0119)

  - No.

- Restrict explicit subpicture ID signalling to the case where all signalled subpictures are treated as pictures.

  - Yes. (JVET-Q0152)

  - No.

- Do something such that different CLVSs referring to the same SPS can have different subpicture information, incuding subpicture ID mappings, such that subpicture ID mapping redunancy can be avoided/reduced?

  - Yes.

    - Signal subpicture sets in SPS, each subpicture set afincludes a number of subpictures applying to one CLVS of the CVS referring to the SPS. (JVET-Q0213)

    - Signal the subpicture ID mapping for those subpictures in a coded picture associated with the PPS and/or PH. (JVET-Q0213)

  - No.

- Replace the sps_subpic_id_present_flag with a new flag indicating whether to describe subpicture ID mapping in SPS?

  - Yes. (JVET-Q0235)

  - No.

- Signal the list of the associated subpicture IDs in each APS?

  - Yes, for identification of APSs associated with a subpicture for bitstream extraction and merging. (JVET-Q0285)

  - No.