| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0592-v1 |
| --- | --- |

| Title: | AHG12: A summary of proposals on general and misc. subpicture aspects |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on general and misc. subpicture aspects submitted to this JVET meeting. It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on general and misc. subpicture aspects

This category of proposals is listed in the agenda item 6.20.1.1, including 10 contributions. These contributions and summaryies of the related proposals are listed below.

JVET-Q0044 AHG9/AHG12: Simplification of slice index with subpictures [T. Nishi, V. Drugeon (Panasonic)]

It is suggested to simplify the indexing of slices by aligning the picture level slice index to the subpicture level slice index, thereby removing the need to map the subpicture level slice index to the picture level slice index.

That means, disallow the picture-level slice index values allocated as shown in in Fig. 1 below. For the same picture partitioning (into subpictures, slices and tiles), use the picture-level slice index values as in Fig. 2.

In other words, picture-level slice index values for slices within a subpicture are required to be continuous. In other words, for any two slices sliceA and sliceB belonging to different subpictures subpicA and subpicB, respectively, when the subpicture index of subpicA is less than that of subpicB, the picture-level slice index value of sliceA shall be less than that sliceB.

Figure 1: Picture divided in 12 tiles, 8 slices and 2 subpictures

Figure 2: Picture divided in 12 tiles, 8 slices and 2 subpictures with slices indexed according to their decoding order

JVET-Q0113 AHG9/AHG8/AHG12: Some general HLS syntax clean-ups [Y.-K. Wang (Bytedance)]

Item 2 of this contribution belongs to this category.

- Text changes for the blank clause for specifying the order of VCL NAL units and association to coded pictures and related. Besides some editorial changes for reorganization of the clauses and addition of a clause on order of PUs and their association to AUs, the proposed text changes include the following aspects:

  - The terms picture-level slice index and subpicture-level slice index are defined, as the order of the slices within a picture and subpicture, respectively, being signalled in the PPS when rect_slice_flag is equal to 1. Note that the term slice index and picture-level slice index are currently used but not defined.

  - When rect_slice_flag is equal to 1, the decoding order of VCL NAL units within a subpicture is specified to be in increasing order of their subpicture-level slice index values, i.e., the slice_address values.

  - It is required that the values of subpicture IDs are increasing in increasing order of the subpicture indices.

  - When slice_subpic_id is not present, the value of slice_subpic_id is inferred to be equal to 0.

With these changes, the decoding order of VCL NAL units within a coded picture can be dertermined by the values of slice_subpic_id and slice_address in the slice headers.

JVET-Q0119 AHG12: Cleanups on signalling of subpictures, tiles, and rectangular slices [Y.-K. Wang (Bytedance)]

Items 1 of this contribution belongs to this category.

- Change the coding of sps_num_subpics_minus1 from u(8) to ue(v), and the value of sps_num_subpics_minus1 is restricted to be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive.

JVET-Q0222 AHG12: On Subpicture Signalling [S. Deshpande (Sharp)]

Item 4 of this contribution belongs to this category.

Proposal 4: Either reserve or allow the value 255 for sps_num_subpics_minus1.

JVET-Q0157 AHG12: On motion compensation for sub-pictures [V. Seregin, Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

1. To fix the decoding problem of the reordered sub-pictures by considering boundaries of a reference sub-picture in the motion compensation process and TMVP derivation.

2. Additionally, it is proposed to replace a current picture size with a reference picture size in the clipping (motion padding) since the reference picture may have different size than the current picture when reference picture resampling is enabled.

JVET-Q0169 AHG9/AHG12: Bitstream conformance requirements on subpicture ID [C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Proposes two constraints:

1) The maximum value of subpicture ID derived by the signalled subpicture ID length shall be greater than or equal to the number of subpicture IDs.

2) For each signalled subpicture ID, it shall be different from all other subpicture IDs in the same picture.

JVET-Q0210 [AHG9]: Miscellaneous HLS clean-ups [Hendry, S. Paluri (LGE)]

Items 1, 3, and 4 of this contribution belong to this category.

1. Prohibit rectangular slices in a tile belonging to different subpictures.

3. Specify a constraint such that when subpicture signalling is present, there shall be at least one subpicture that is an independently coded subpicture.

4. Specify a constraint such that when subpicture ID is signalled in PPS or picture header, all subpictures are independently coded subpictures.

JVET-Q0271 AHG9: On Subpicture Ordering [M. Damghanian, R. Sjöberg, M. Pettersson (Ericsson)]

This contribution proposes that the following changes in VVC are adopted:

- To define an independent subpicture as a subpicture for which the subpicture boundaries are treated as picture boundaries in the decoding process and no loop filtering across the subpicture boundaries is done. (Editorial)

- To add a syntax element sps_independent_subpics_flag in the SPS. When equal to 1 it specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are signalled only when sps_independent_subpics_flag is equal to 0. (Signalling efficiency)

- To specify that the subpicture availability rule applies only when sps_independent_subpics_flag is equal to 0. (Functionality)

JVET-Q0406 AHG12: On CABAC zero words for subpictures [R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)]

This contribution asserts that subpicture extraction could result in violating the bin-to-bit ratio constraint as CABAC zero words are not associated to subpictures. Therefore, it is proposed to add a bitstream constraint for the VCL NAL units of subpictures to fulfil the bin-to-bit ratio constraint individually in addition to the picture unit level constraint when the following conditions apply:

– a subpicture has subpic_treated_as_pic_flag[ ] equal to 1.

– a subpicture level information SEI message is present in the CLVS.

JVET-Q0443 Independent Subpictures [K. Sharman, S. Keating, A. Browne (Sony)]

Item 1 (the aspect on CABAC zero words and MinCR constraints) of this contribution belongs to this category.

Subpictures allow regions of a picture to be independently decodable, with care added to the DIS (JVET-P2001-v14) to handle edges of subpictures and describe the subpicture requirements at different levels via a dedicated SEI message.

1) This contribution proposes that constrains are added to subpictures so that CABAC zero words and MinCR picture level constraints are also applied at a subpicture level.

# List of design questions

Only one design question/choice is listed below, the only overlapping aspects. All other proposals can be individually discussed based on the above summaries, and, when needed, open the contribution documents themselves.

- On sps_num_subpics_minus1, do one of the following?

  - Change the coding o from u(8) to ue(v), and specify that the value shall be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. (JVET-Q-0119)

  - Specify the the value 255 is reserved. (JVET-Q0222)

  - Remove the value range of 0..254 such that the value range is 0..255. (JVET-Q0222)