| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0608 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0167 (Non-CE5: Multiplication simplification for ALF and CC-ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-Q0167 (Non-CE5: Multiplication simplification for ALF and CC-ALF). The simulations were carried out based on the common test condition (CTC). Based on the results, the test results are matched.

# Introduction

In JVET-Q0167, a restriction of ALF coefficients is proposed. Instead of allowing all coefficient values from -128 to 127, only values that are pure powers of two +/-{0, 1, 2, 4, 8, …} and values that are 3 times pure powers of two +/-{3, 6, 12, 24, …}(test 1) are proposed to be allowed. It is claimed that this gives a substantial complexity reduction in terms of surface area of hardware implementations of ALF since every per-sample multiplication can be then replaced by an addition and shifts.

A variant (test 2) which, in addition to allowing the values of test 1, also allows coefficient values that are 5 times a pure power of two +/-{5, 10, 20, 40, …} is also presented, and it is claimed that this can also be implemented in hardware with the same number of additions as test 1.

# Simulation Results

## Test 1 – allowing 3 times power of two

In test 1, an ALF coefficient can be either a pure power of two, or 3 times a pure power of two. This means that the following coefficients are allowed: f  {0, 1, 2, 3, 4, 6 8, 12, 16, 24, 32, 48, 64}.

Summarized results with VTM-7.0 as anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 98% | 102% |
| Class A2 | 0.04% | 0.03% | 0.03% | 101% | 104% |
| Class B | 0.02% | -0.07% | -0.06% | 100% | 102% |
| Class C | -0.02% | -0.07% | -0.07% | 99% | 102% |
| Class E | 0.05% | -0.05% | -0.05% | 100% | 101% |
| Overall | 0.02% | -0.04% | -0.03% | 100% | 102% |
| Class D | -0.02% | 0.09% | 0.05% | 100% | 102% |
| Class F | -0.02% | -0.09% | -0.08% | 101% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.16% | -0.03% | 99% | 99% |
| Class A2 | 0.05% | -0.01% | 0.01% | 102% | 102% |
| Class B | 0.04% | -0.06% | -0.06% | 102% | 101% |
| Class C | 0.00% | -0.20% | -0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.11% | -0.04% | 101% | 101% |
| Class D | -0.03% | -0.30% | -0.06% | 100% | 101% |
| Class F | -0.06% | -0.19% | -0.10% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.53% | -0.24% | 101% | 101% |
| Class C | -0.03% | -0.34% | -0.60% | 101% | 101% |
| Class E | -0.01% | 0.17% | -0.38% | 101% | 100% |
| Overall | 0.00% | -0.29% | -0.39% | 101% | 101% |
| Class D | -0.13% | 0.31% | 0.11% | 100% | 101% |
| Class F | -0.13% | -0.48% | -0.18% | 99% | 101% |

## Test 2 – allowing also 5 times power of two

In test 2, in addition to the allowed coefficient values in test 1, an ALF coefficient can also be 5 times a pure power of two. This means that the following coefficients are allowed: f  {0, 1, 2, 3, 4, 5, 6 8, 10, 12, 16, 20, 24, 32, 40, 48, 64}.

Summarized results with VTM-7.0 as anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class A2 | 0.02% | 0.01% | 0.00% | 102% | 105% |
| Class B | 0.00% | -0.08% | -0.07% | 100% | 103% |
| Class C | -0.03% | -0.09% | -0.09% | 99% | 101% |
| Class E | 0.00% | -0.06% | -0.06% | 98% | 100% |
| Overall | 0.00% | -0.05% | -0.05% | 100% | 102% |
| Class D | -0.02% | 0.04% | 0.00% | 99% | 101% |
| Class F | -0.03% | -0.09% | -0.12% | 99% | 103% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 102% | 101% |
| Class A2 | 0.01% | 0.05% | 0.05% | 99% | 103% |
| Class B | 0.01% | -0.16% | -0.02% | 100% | 101% |
| Class C | -0.01% | -0.27% | -0.23% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.12% | -0.06% | 101% | 101% |
| Class D | -0.05% | -0.32% | -0.09% | 99% | 100% |
| Class F | -0.01% | -0.06% | -0.09% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.24% | -0.03% | 102% | 102% |
| Class C | 0.01% | -0.36% | -0.15% | 101% | 100% |
| Class E | -0.13% | 0.05% | -0.86% | 100% | 102% |
| Overall | -0.04% | -0.21% | -0.28% | 101% | 101% |
| Class D | 0.01% | -0.25% | -0.44% | 100% | 100% |
| Class F | -0.14% | -0.25% | -0.03% | 100% | 103% |

## Test 3 – applying test 1 to both ALF and CC-ALF

In test 3, the ALF part is equivalent to test 1, but it also enables CC-ALF. In order to guarantee that the DC-filter response is zero (i.e., that the coefficients sum to zero), the DC-neutral formulation presented in JVET-Q0165 is used, which guarantees no change in chroma if luma is flat. The same restriction as in test 1 is applied to the coefficients of both ALF and CC-ALF, i.e., the following are allowed: {0, 1, 2, 3, 4, 6, 8, 12, 16, 24, 32, 48, 64}.

Summarized results with CE5 common base as anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.13% | 101% | 101% |
| Class A2 | 0.04% | -0.55% | -0.40% | 100% | 99% |
| Class B | 0.01% | -0.04% | -0.12% | 101% | 100% |
| Class C | -0.06% | -0.17% | -0.25% | 100% | 101% |
| Class E | 0.03% | 0.06% | -0.72% | 99% | 101% |
| Overall | 0.00% | -0.13% | -0.29% | 100% | 100% |
| Class D | -0.02% | -0.13% | -0.44% | 100% | 99% |
| Class F | -0.06% | -0.12% | -0.16% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.25% | -0.31% | 99% | 101% |
| Class A2 | 0.08% | -0.27% | -0.33% | 101% | 101% |
| Class B | 0.04% | -0.37% | -0.48% | 101% | 100% |
| Class C | -0.05% | -0.42% | -0.44% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.34% | -0.41% | 100% | 101% |
| Class D | -0.06% | -0.55% | -0.50% | 100% | 99% |
| Class F | -0.05% | -0.31% | -0.34% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.96% | -0.49% | 99% | 100% |
| Class C | 0.00% | -0.48% | -0.79% | 99% | 101% |
| Class E | 0.03% | -0.81% | -1.62% | 100% | 100% |
| Overall | 0.01% | -0.76% | -0.87% | 99% | 100% |
| Class D | -0.05% | -1.36% | -0.69% | 99% | 99% |
| Class F | -0.07% | -0.50% | 0.17% | 100% | 99% |

## Test 4 – further restricting CC-ALF coefficients

In test 4, while the ALF part is equivalent to test 1, it places further restrictions on the CC-ALF coefficients, allowing only pure powers of two {0, 1, 2, 4, 8, 16, 32, 64}. This makes the CC-ALF part equivalent to test 2 of JVET-Q0165, which is a DC-neutral version of CE5-2.1.

Summarized results with CE5 common base as anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.05% | 100% | 99% |
| Class A2 | 0.04% | -0.42% | -0.29% | 99% | 99% |
| Class B | 0.00% | 0.06% | -0.01% | 98% | 100% |
| Class C | -0.07% | -0.15% | -0.22% | 101% | 100% |
| Class E | 0.01% | 0.25% | -0.67% | 99% | 100% |
| Overall | -0.01% | -0.04% | -0.22% | 99% | 100% |
| Class D | -0.02% | -0.15% | -0.48% | 100% | 100% |
| Class F | -0.07% | -0.02% | -0.04% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.26% | -0.27% | 100% | 100% |
| Class A2 | 0.08% | -0.20% | -0.24% | 99% | 99% |
| Class B | 0.05% | -0.36% | -0.36% | 100% | 100% |
| Class C | -0.04% | -0.45% | -0.47% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.33% | -0.35% | 100% | 100% |
| Class D | -0.06% | -0.54% | -0.42% | 99% | 101% |
| Class F | -0.03% | -0.22% | -0.33% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.82% | -0.26% | 100% | 100% |
| Class C | 0.00% | -0.43% | -0.74% | 99% | 101% |
| Class E | 0.04% | -1.38% | -1.47% | 101% | 99% |
| Overall | 0.01% | -0.83% | -0.72% | 100% | 100% |
| Class D | -0.05% | -1.01% | -0.54% | 100% | 99% |
| Class F | -0.10% | -0.35% | -0.37% | 101% | 100% |

## Test 5

In test 5, the ALF part is equivalent to test 2, allowing the coefficients to assume the values of {0, 1, 2, 3, 4, 6 8, 12, 16, 24, 32, 48, 64}. For the CC-ALF coefficients, the restriction is the same as for test 4, i.e., allowing only pure powers of two {0, 1, 2, 4, 8, 16, 32, 64}. This makes the CC-ALF part equivalent to test 2 of JVET-Q0165, which is a DC-neutral version of CE5-2.1.

Summarized results with CE5 common base as anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.07% | 99% | 101% |
| Class A2 | 0.02% | -0.46% | -0.29% | 100% | 100% |
| Class B | -0.02% | 0.05% | -0.04% | 101% | 99% |
| Class C | -0.08% | -0.18% | -0.25% | 100% | 100% |
| Class E | -0.04% | 0.23% | -0.69% | 100% | 100% |
| Overall | -0.03% | -0.06% | -0.24% | 100% | 100% |
| Class D | -0.02% | -0.19% | -0.53% | 100% | 100% |
| Class F | -0.08% | -0.06% | -0.06% | 99% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.24% | -0.17% | 101% | 100% |
| Class A2 | 0.01% | -0.23% | -0.31% | 101% | 100% |
| Class B | 0.00% | -0.35% | -0.52% | 99% | 99% |
| Class C | -0.03% | -0.50% | -0.46% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.34% | -0.39% | 100% | 100% |
| Class D | -0.05% | -0.55% | -0.31% | 98% | 102% |
| Class F | -0.04% | -0.19% | -0.32% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.83% | -0.58% | 100% | 100% |
| Class C | 0.02% | -0.57% | -0.79% | 100% | 99% |
| Class E | -0.01% | -1.57% | -1.65% | 102% | 100% |
| Overall | -0.01% | -0.93% | -0.92% | 101% | 100% |
| Class D | -0.08% | -1.69% | -1.17% | 100% | 100% |
| Class F | -0.13% | -0.53% | -0.24% | 101% | 100% |