| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0612-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0327:On IBC/ATMVP candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen | Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of swapping the first two IBC merge candidate and IBC inter candidate and changing the derivation process of motion shift in ATMVP to only check the top block of the current block which are proposed in JVET-Q0327[1]. It is reported that the coding results are identical with those reported by the proponent.

- Introduction

JVET-Q0327 proposes to swap the first two IBC merge candidate and IBC inter candidate and change the derivation process of motion shift in ATMVP to only check the top block of the current block. we tests are conducted in this contribution. It is reportedly shown the coding results are identical with those reported in JVET-Q0327.

Simulation results

- Test1: Modification of IBC merge/IBC inter candidate list construction.

Table 1~Table 3. Experimental results of JVET-Q0327 Test1: IBC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.07% | 0.18% | 165% | 100% |
| Class A2 | -0.79% | -1.01% | -0.95% | 176% | 100% |
| Class B | -0.79% | -0.65% | -0.88% | 190% | 102% |
| Class C | -0.30% | -0.24% | -0.36% | 186% | 100% |
| Class E | -1.28% | -1.31% | -1.25% | 191% | 101% |
| Overall | -0.64% | -0.63% | -0.66% | 182% | 101% |
| Class D | -0.62% | -0.66% | -0.58% | 176% | 101% |
| Class F | 0.00% | -0.04% | 0.14% | 100% | 100% |
| Class SCC | -0.06% | -0.02% | 0.00% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.01% | 0.25% | 107% | 100% |
| Class A2 | -0.10% | -0.12% | -0.07% | 108% | 100% |
| Class B | -0.10% | -0.19% | -0.36% | 111% | 101% |
| Class C | 0.21% | 0.04% | 0.11% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.07% | -0.05% | 111% | 100% |
| Class D | 0.11% | -0.10% | -0.13% | 114% | 101% |
| Class F | -0.03% | -0.06% | -0.09% | 100% | 101% |
| Class SCC | 0.05% | 0.06% | 0.08% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% |  | -0.13% |  | 0.06% | 117% |  | 98% |  |
| Class C | 0.09% |  | -0.32% |  | 0.16% | 123% |  | 104% |  |
| Class E | -0.06% |  | 0.55% |  | 1.14% | 112% |  | 102% |  |
| Overall | 0.02% |  | -0.03% |  | 0.37% | 117% |  | 101% |  |
| Class D | 0.02% |  | 0.20% |  | -0.09% | 122% |  | 102% |  |
| Class F | -0.11% |  | -0.38% |  | 0.26% | 101% |  | 102% |  |
| Class SCC | -0.05% |  | -0.12% |  | -0.11% | 99% |  | 95% |  |

- Test2: Modification of ATMVP

Table 4~Table 5. Experimental result of the proposed method (Test2: ATMVP)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.04% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.02% | 101% | 100% |
| Class B | 0.00% | 0.05% | 0.02% | 100% | 101% |
| Class C | 0.01% | 0.02% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 100% | 101% |
| Class D | -0.02% | -0.03% | -0.04% | 101% | 100% |
| Class F | 0.01% | -0.04% | 0.00% | 101% | 101% |
| Class SCC | -0.03% | -0.03% | -0.03% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | 0.10% | 100% | 99% |
| Class C | -0.03% | -0.14% | -0.06% | 101% | 104% |
| Class E | 0.01% | 0.16% | -0.72% | 101% | 103% |
| Overall | -0.01% | -0.07% | -0.16% | 101% | 102% |
| Class D | -0.07% | 0.78% | 0.25% | 100% | 98% |
| Class F | 0.04% | -0.37% | -0.12% | 100% | 102% |
| Class SCC | 0.03% | 0.02% | -0.01% | 99% | 95% |

- Test3: Combination of Test1 and Test2.

Table 6~Table 8. Experimental result of the proposed method (Test3: IBC + ATMVP)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.04% |  | -0.07% |  | 0.18% | 164% |  | 100% |  |
| Class A2 | -0.79% |  | -1.01% |  | -0.95% | 176% |  | 100% |  |
| Class B | -0.79% |  | -0.65% |  | -0.88% | 190% |  | 101% |  |
| Class C | -0.30% |  | -0.24% |  | -0.36% | 186% |  | 100% |  |
| Class E | -1.28% |  | -1.31% |  | -1.25% | 190% |  | 100% |  |
| Overall | -0.64% |  | -0.63% |  | -0.66% | 182% |  | 100% |  |
| Class D | -0.62% |  | -0.66% |  | -0.58% | 176% |  | 101% |  |
| Class F | 0.00% |  | -0.04% |  | 0.14% | 100% |  | 100% |  |
| Class SCC | -0.06% |  | -0.02% |  | 0.00% | 100% |  | 100% |  |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | -0.01% | 0.17% | 107% | 100% |
| Class A2 | -0.11% | -0.04% | -0.05% | 108% | 100% |
| Class B | -0.10% | -0.15% | -0.28% | 111% | 101% |
| Class C | 0.25% | 0.09% | 0.07% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.03% | -0.05% | 111% | 100% |
| Class D | 0.12% | -0.09% | -0.11% | 115% | 101% |
| Class F | -0.02% | -0.09% | -0.09% | 100% | 101% |
| Class SCC | 0.03% | 0.03% | 0.10% | 99% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.08% | -0.34% | 117% | 98% |
| Class C | 0.07% | -0.13% | -0.02% | 123% | 104% |
| Class E | -0.11% | 0.25% | 0.78% | 112% | 102% |
| Overall | -0.02% | -0.01% | 0.05% | 117% | 101% |
| Class D | 0.05% | -0.11% | 0.15% | 122% | 103% |
| Class F | 0.02% | -0.27% | 0.53% | 100% | 101% |
| Class SCC | -0.10% | -0.08% | -0.14% | 99% | 94% |

References

[1] S. Wan, Y. Xue, J.-Y. Huo, Y.-Z. Ma, F.-Z. Yang, Y.-F. Yu, Y. Liu, on CU-level BDOF enable condition, JVET-Q0455, Brussels, Janunary, 2020.