| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0627 |
| --- | --- |

| Title: | On Dependent Quantization: combination of JVET-Q0243 and JVET-Q0227 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. de Lagrange, F. Hiron, F. Le Léannec, E. François (InterDigital) | Tel: Email: | Fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution relates to Dependent Quantization and combines the proposals from JVET-Q0243 on using 8 quantization states instead of 4 states in the current VTM, and JVET-Q0227 on using an qP-dependent offset when applying the scaling process for transform coefficients.

The reported psnr-Y, U, V BD-rate variations compared to VTM7.0 when using 8 quantization states are:

AI:		–0.47%,     0.42%,     0.43%  (Y, Cb, Cr)  at 109% encoding time and  100% decoding time
	RA:	–0.38%,   –0.05%,   –0.12%  (Y, Cb, Cr)  at 106% encoding time and  101% decoding time
	LB:		–x.xx%,   –x.xx %,  –x.xx %  (Y, Cb, Cr)  at xxx% encoding time and xxx% decoding time

# Introduction

JVET-Q0243 proposes to generalize the Dependent Quantization process by enabling the usage of 8 states, in addition to the 4 states in the current VTM. The number of states is indicated in the picture header or the picture parameter set, using a syntax element pic_dep_quant_enabled_idc indicating if DepQ is not used (0), or if it used with 4 states (1) or 8 states (2)..

JVET-Q0227 proposes a modification of the scaling factors ls[ x ][ y ] derivation formula, in case of using Dependent Quantization, by using a qP-dependent offset, specified as follows:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + qPOffset) % 6 ] ) << ( (qP + qPOffset) / 6 )

where qPOffset is defined as follows:

- If qP > (30 + QpBdOffset), qPOffset = 2

- Otherwise if qP > (23 + QpBdOffset), qPOffset = 1

- Otherwise, qPOffset = 0

These 2 proposals are combined in the current contribution.

Investigations have been made on using alternate offset values, resulting in the current proposal.

# Tests results

SDR CTCs are used, as defined in JVET-N1010. BD-rate variations relative to the VTM7.0 are reported in the tables below, for the 4-states case and for the 8-states case. Full results are also attached to the contribution.

## Using 4 states

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.24% | 102% | 102% |
| Class A2 | -0.32% | 0.35% | 0.26% | 103% | 101% |
| Class B | -0.06% | 0.33% | 0.18% | 101% | 97% |
| Class C | 0.01% | 0.44% | 0.38% | 102% | 101% |
| Class E | -0.03% | 0.30% | 0.41% | 101% | 101% |
| Overall | -0.07% | 0.29% | 0.28% | 101% | 100% |
| Class D | 0.00% | 0.50% | 0.49% | 101% | 100% |
| Class F | -0.07% | 0.28% | 0.34% | 102% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.22% | -0.25% | 100% | 102% |
| Class A2 | -0.20% | -0.34% | -0.46% | 102% | 99% |
| Class B | -0.08% | -0.33% | -0.29% | 100% | 99% |
| Class C | -0.02% | -0.35% | -0.35% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.23% | -0.33% | 101% | 100% |
| Class D | -0.04% | -0.38% | -0.42% | 99% | 100% |
| Class F | -0.07% | -0.07% | -0.17% | 101% | 99% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.26% | 0.03% | 0.03% | 100% | 100% |
| Class E | -0.04% | 0.63% | -0.54% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.49% | 0.14% | 103% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Using 8 states

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | 0.23% | 0.53% | 109% | 103% |
| Class A2 | -0.91% | 0.47% | 0.31% | 108% | 100% |
| Class B | -0.48% | 0.50% | 0.40% | 110% | 100% |
| Class C | -0.23% | 0.41% | 0.51% | 108% | 102% |
| Class E | -0.29% | 0.46% | 0.40% | 111% | 101% |
| Overall | -0.47% | 0.42% | 0.43% | 109% | 101% |
| Class D | -0.26% | 0.68% | 0.53% | 108% | 100% |
| Class F | -0.15% | 0.31% | 0.40% | 111% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.29% | 0.12% | 106% | 101% |
| Class A2 | -0.56% | -0.16% | -0.28% | 106% | 101% |
| Class B | -0.45% | -0.11% | -0.14% | 106% | 101% |
| Class C | -0.26% | -0.16% | -0.14% | 108% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.05% | -0.12% | 106% | 101% |
| Class D | -0.28% | -0.24% | -0.05% | 105% | 101% |
| Class F | -0.23% | -0.10% | -0.11% | 106% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.62% | -0.05% | 0.12% | 107% | 101% |
| Class E | -0.28% | 0.71% | 0.85% | 105% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.45% | 0.87% | 0.40% | 107% | 101% |
| Class F | -0.62% | -0.45% | 0.27% | 104% | 101% |

In summary, the reported BD-rates variations of the combined tools are:

AI:		–0.47%,     0.42%,     0.43%  (Y, Cb, Cr)  at 109% encoding time and  100% decoding time
	RA:	–0.38%,   –0.05%,   –0.12%  (Y, Cb, Cr)  at 106% encoding time and  101% decoding time
	LB:		–x.xx%,   –x.xx%,   –x.xx%  (Y, Cb, Cr)  at xxx% encoding time and xxx% decoding time

While with JVET-Q0243 the BD-rate variations are:

AI:		–0.43%,     0.07%,    0.14%  (Y, Cb, Cr)  at 111% encoding time and   99% decoding time
	RA:	–0.34%,     0.05%,    0.25%  (Y, Cb, Cr)  at 106% encoding time and   98% decoding time
	LB:		–0.40%,     0.21%,    0.13%  (Y, Cb, Cr)  at 104% encoding time and 100% decoding time

It is therefore observed a cumulative gain from the combination of both tools.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).