| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0629 |
| --- | --- |

| Title: | CE2-related: Palette mode excluding small blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

#### Spec change for Q0629 and Q0504

# Specification changes

To apply the palette mode to non 4:4:4 color format, the changes to the VVC draft 7 are highlighted in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_format_idc = = 3 ) { |  |
| sps_palette_enabled_flag | u(1) |
| if( chroma_format_idc = = 3 ) { |  |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER && (cbWidth* cbHeight > (treeType!= DUAL_TREE_CHROMA? 16:16*SubWidthC*SubHeightC)) ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| … |  |
| } |  |

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| palettePredictionFinished = 0 |  |
| … |  |
| if( palette_escape_val_present_flag ) { |  |
| for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) |  |
| for( sPos = minSubPos; sPos < maxSubPos; sPos++ ) { |  |
| xC = x0 + TraverseScanOrder[ log2CbWidth][ log2CbHeight ][ sPos ][ 0 ] |  |
| yC = y0 + TraverseScanOrder[ log2CbWidth][ log2CbHeight ][ sPos ][ 1 ] |  |
| if( !( treeType = = SINGLE_TREE && cIdx != 0 && xC % SubWidthC != 0 && yC % SubHeightC != 0 ) ) { |  |
| if( PaletteIndexMap[ cIdx ][ xC ][ yC ] = = MaxPaletteIndex ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| … |  |

8.4.5.3 Decoding process for palette mode

Inputs to this process are:

- a location ( xCbComp, yCbComp ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current coding block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of treeType, the variables startComp and numComps are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(444)

numComps = ChromaArrayType  = =  0 ? 1 : 3		(445)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(446)

numComps = 1		(447)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(448)

numComps = 2		(449)

…

The variable localDualtree is derived as follows:

localDualtree = treeType != SINGLE_TREE && ( slice_type != I | | ( slice_type = = I &&

qtbtt_dual_tree_intra_flag = = 0 ) ) ? 1 : 0

When one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1;

- cIdx is equal to 2;

the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows:

If localDualtree is equal to 1, the following applies:

  - If treeType is equal to DUAL_TREE_LUMA

for( i = 0; i < num_signalled_palette_entries[ startComp ]; i++ ) {
        CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth – 1 )
        CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth – 1 )
}

  - Otherwise, if treeType is equal to DUAL_TREE_CHROMA

for( i = 0; i < num_signalled_palette_entries[ startComp ]; i++ ) {
	         CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth – 1 )
        }

  - CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]

  - startComp = 0

  - numComps = 3

The array PredictorPaletteEntries are derived or modified as follows:

for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ )
	for( compIdx = startComp; compIdx < (startComp + numComps); compIdx++ )
		newPredictorPaletteEntries[ compIdx ][ i ] = CurrentPaletteEntries[ compIdx ][ i ]
newPredictorPaletteSize = CurrentPaletteSize[ startComp ]
for( i = 0; i < PredictorPaletteSize[ startComp ]  &&  newPredictorPaletteSize < 63; i++ )
	if( !PalettePredictorEntryReuseFlags[ i ] ) {
		for( compIdx = startComp; compIdx < (startComp + numComps); compIdx++ )	(460)
			newPredictorPaletteEntries[ compIdx ][ newPredictorPaletteSize ] =
				PredictorPaletteEntries[ compIdx ][ i ]
		newPredictorPaletteSize++
	}
for( compIdx = startComp; compIdx < ( startComp + numComps ); compIdx++ )
	for( i = 0; i < newPredictorPaletteSize; i++ )
		PredictorPaletteEntries[ compIdx ][ i ] = newPredictorPaletteEntries[ compIdx ][ i ]
PredictorPaletteSize[ startComp ] = newPredictorPaletteSize

If qtbtt_dual_tree_intra_flag is equal to 0 or slice_type is not equal to I, the following applies:

PredictorPaletteSize[ 1 ] = newPredictorPaletteSize

It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to 63, inclusive.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).