| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0662 |
| --- | --- |

| Title: | Cross-check of JVET-Q0525 Non-CE4: Support large rotation and flipping in affine and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

JVET-Q0525 proposes two solutions to address the discontinuity problem asserted in affine motion compensation and PROF when large rotation and flipping happen. It is reported that the description in JVET-Q0525 matches the software provided by the proponent. It is also reported that the overall BD-rate results with JVET-CTC case match the ones provided in JVET-Q0525.

# Introduction

JVET-P0525 proposes two solutions to address the discontinuity problem asserted in affine motion compensation and PROF when large rotation and flipping happen. There are two steps. The first step detects large rotation and flipping based on affine motion vectors and defines eight processing modes. Based on the processing mode, the second step 1) modifies the PROF motion vector refinements and 2) rotates or flips the samples after subblock-based motion compensation.

# Results

The results are provided for the JVET-CTC case.

Note that encoder run times are inaccurate due to different cluster loads.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.00% | 98% | 100% |
| Class A2 | -0.03% | -0.04% | -0.02% | 95% | 99% |
| Class B | 0.01% | -0.01% | 0.03% | 95% | 100% |
| Class C | 0.00% | 0.01% | -0.05% | 95% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 96% | 100% |
| Class D | 0.02% | -0.07% | 0.04% | 96% | 100% |
| Class F | -0.02% | -0.04% | -0.02% | 105% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | 0.16% | 91% | 99% |
| Class C | 0.03% | -0.23% | -0.03% | 93% | 101% |
| Class E | 0.10% | 0.35% | -0.03% | 93% | 99% |
| Overall | 0.03% | -0.05% | 0.05% | 92% | 100% |
| Class D | -0.01% | 0.08% | -0.05% | 92% | 100% |
| Class F | 0.00% | -0.30% | 0.57% | 93% | 101% |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.