| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0685-v4 |
| --- | --- |

| Title: | A unified MTS and LFNST signaling based on the DC coefficient combined with JVET-Q0057 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Muhammed Coban, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com {xiaoyuxiu, yiwenchen, tsung-chuanma,jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Qualcomm Inc., Kwai Inc. |  |  |

_____________________________

# Abstract

This document proposes to extend the DC coefficient based restriction in LFNST for MTS signaling by combining with JVET-Q0057. The proposed additional change in this document can be viewed as a unification of the coefficient-level restriction in MTS and LFNST signaling. The experimental results show that the proposed changes achieve 0.10% AI and 0.04% RA BD-rates under CTC.

# Introduction

In the current VVC specification working draft, the MTS and LFNST indices (mts_idx and lfnst_idx) are signalled after residual coding if the following set of conditions are satisfied:

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| … |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |

where:

- LfnstDcOnly (initialized as one) is set to zero according to the following conditions depending on the last scanning position indicating whether there exists a DC coefficeint or not:

| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| --- | --- |
| LfnstDcOnly = 0 |  |

- MtsZeroOutSigCoeffFlag is set to zero according to the following conditions depending on the last significant coefficient position:

| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| --- | --- |
| MtsZeroOutSigCoeffFlag = 0 |  |

However, the problem in the above condition for setting MtsZeroOutSigCoeffFlag  is that MTS can lead to undesired non-zero coefficeints outside the top-left 16x16 region in a 32x32 TU. In order to resolve this problem, JVET-Q0057 proposes to fix this problem by introducing a CG-based restriction as follows:

| if( ( coded_sub_block_flag[ xS ][ yS ] ¦ ¦ i = = lastSubBlock ) && cIdx = = 0 && ( xS > 3 ¦ ¦ yS > 3 ) { |  |
| --- | --- |
| MtsZeroOutSigCoeffFlag = 0 |  |
| } |  |

On top of the fix provided in JVET-Q0057, this proposal further extends the DC-coefficient based restriction (handled by the variable LfnstDcOnly) on LFNST for MTS signaling.

# Proposed Method

This contribution document proposes a unified LFNST and MTS signaling scheme by restricting both of them based on the DC coefficient.

In current VVC draft, when only DC coefficient is available, LFNST index is not signalled and inferred as 0. This document proposes to apply the same approach, that is, if only DC coefficient is available, MTS signaling is also skipped where DST-7 and DCT-8 combinations (lfnst_idx > 1) are not allowed. Since it is proposed to signal MTS index if there are coeffients other than DC-coefficient, the cbf flag check below can be replaced with the MtsDcOnly = = 0  condition as:

| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |

where MtsDcOnly (initialized as one) is set to zero according to the following condition:

| if (lastSubBlock > 0 ¦ ¦ lastScanPos > 0) |  |
| --- | --- |
| MtsDcOnly = 0 |  |

In overall, this document proposes the following changes in highlighted green on the VVC draft text (version JVET-P2001-vE) in addition to the changes in JVET-Q0057 highlighted in yellow:

#### 7.3.9.4 Coding tree syntax

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| MtsDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

…

#### 7.3.9.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if (lastSubBlock > 0 ¦ ¦ lastScanPos > 0) |  |
| MtsDcOnly = 0 |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( i < lastSubBlock && i > 0 ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| if( ( coded_sub_block_flag[ xS ][ yS ] ¦ ¦ i = = lastSubBlock ) && cIdx = = 0 && ( xS > 3 ¦ ¦ yS > 3 ) { |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| … |  |

Editorially, the above conditions can be futher simplified as follows, since some of the conditions are not necessarily needed (the same variable name for MtsDcOnly and LfnstDcOnly can also be used):

| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| --- | --- |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if (lastSubBlock > 0 ¦ ¦ lastScanPos > 0) { |  |
| MtsDcOnly = 0 |  |
| LfnstDcOnly = 0 |  |
| } |  |

# Experimental Results

The proposed method is implemented on top of VTM-7.0 and evaluated under common test conditions (CTC).

Table 1.  Results of the proposed method with changes highlighted in green and yellow over VTM-7.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.13% | 0.14% | 100% | 100% |
| Class A2 | -0.09% | 0.04% | -0.04% | 98% | 98% |
| Class B | -0.08% | 0.05% | -0.04% | 102% | 104% |
| Class C | -0.14% | 0.05% | 0.00% | 98% | 91% |
| Class E | -0.13% | -0.03% | 0.17% | 97% | 88% |
| Overall | -0.10% | 0.01% | 0.03% | 99% | 97% |
| Class D | -0.09% | 0.08% | -0.09% | 101% | 95% |
| Class F | -0.11% | 0.02% | 0.15% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | -0.02% | 100% | 102% |
| Class A2 | -0.08% | 0.03% | 0.09% | 97% | 98% |
| Class B | -0.03% | 0.14% | 0.22% | 100% | 102% |
| Class C | -0.06% | 0.06% | 0.01% | 97% | 91% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.06% | 0.09% | 98% | 98% |
| Class D | -0.06% | 0.00% | 0.30% | 100% | 99% |
| Class F | -0.05% | 0.10% | 0.08% | 100% | 99% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.29% | -0.12% | 99% | 99% |
| Class C | 0.00% | -0.26% | -0.24% | 96% | 95% |
| Class E | -0.12% | 0.66% | -0.66% | 96% | 96% |
| Overall | -0.02% | -0.04% | -0.29% | 98% | 97% |
| Class D | -0.05% | -0.08% | 0.43% | 98% | 98% |
| Class F | -0.01% | -0.19% | 0.84% | 98% | 100% |

Table 2.  Results in JVET-0057 proposal, reflecting the changes highlighted in yellow over VTM-7.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 102% | 101% |
| Class A2 | -0.01% | -0.01% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.01% | -0.03% | 102% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | -0.03% | 99% | 93% |
| Overall | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Class D | 0.00% | 0.03% | 0.00% | 102% | 106% |
| Class F | 0.00% | 0.01% | 0.00% | 101% | 104% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.00% | 102% | 102% |
| Class A2 | -0.05% | 0.02% | -0.02% | 99% | 99% |
| Class B | 0.02% | 0.02% | 0.06% | 101% | 102% |
| Class C | 0.00% | -0.04% | 0.03% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.03% | 101% | 101% |
| Class F | 0.00% | -0.02% | -0.02% | 101% | 101% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.15% | 0.01% | 101% | 101% |
| Class C | 0.01% | -0.12% | -0.05% | 99% | 100% |
| Class E | 0.01% | -0.01% | -0.07% | 97% | 94% |
| Overall | 0.00% | -0.10% | -0.03% | 99% | 99% |
| Class D | 0.01% | -0.05% | 0.04% | 101% | 102% |
| Class F | -0.01% | -0.03% | 0.01% | 99% | 100% |

Table 3.  Results of the proposed method with changes highlighted in green and yellow over the VTM-7.0 under CTC configurations with LFNST is disabled

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with LFNST disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.17% | -0.06% | 100% | 100% |
| Class A2 | -0.09% | -0.06% | -0.03% | 100% | 98% |
| Class B | -0.08% | -0.05% | 0.01% | 100% | 99% |
| Class C | -0.13% | -0.02% | -0.09% | 98% | 89% |
| Class E | -0.11% | -0.11% | 0.01% | 98% | 92% |
| Overall | -0.09% | -0.02% | -0.03% | 99% | 95% |
| Class D | -0.07% | -0.08% | -0.11% | 99% | 94% |
| Class F | -0.08% | 0.12% | 0.14% | 101% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 with LFNST disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.06% | -0.12% | 101% | 103% |
| Class A2 | -0.03% | -0.14% | 0.05% | 97% | 97% |
| Class B | -0.03% | 0.09% | 0.01% | 99% | 100% |
| Class C | -0.06% | 0.10% | -0.08% | 96% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | -0.03% | 98% | 98% |
| Class D | -0.02% | 0.13% | 0.13% | 99% | 97% |
| Class F | 0.01% | 0.07% | 0.05% | 100% | 100% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.29% | -0.12% | 99% | 99% |
| Class C | 0.00% | -0.26% | -0.24% | 96% | 95% |
| Class E | -0.12% | 0.66% | -0.66% | 96% | 96% |
| Overall | -0.02% | -0.04% | -0.29% | 98% | 97% |
| Class D | -0.05% | -0.08% | 0.43% | 98% | 98% |
| Class F | -0.01% | -0.19% | 0.84% | 98% | 100% |

# Conclusions

This document proposes to unify the LFNST and MTS signaling scheme by restricting both of them based on the DC coefficient. As proposed changes provide 0.1% AI average BD-rate gains, this can be viewed as a unification/coding gain proposal.  It is recommended to adopt the proposed method in VVC.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

- J. Lainema, “On MTS signaling,” JVET-Q0055, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020.

- M. Coban, M. Karczewicz, H.E. Egilmez, V. Seregin, “Coefficient group based restriction on MTS signaling,” JVET-Q0057, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020.

- X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, “MTS signaling based on last significant coefficient position,” JVET-Q0516, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020.

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).