| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0686-v1 |
| --- | --- |

| Title: | Chroma LFNST Simplification and Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Muhammed Coban, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VTM-7.0, the LFNST signaling for chroma blocks depends on the luma transform skip flag. This document proposes to remove this dependency by restricting chroma LFNST signaling based on chroma transform skip without any changes in luma LFNST signaling. It is also proposed to remove chroma LFNST when single-tree partition mode is used in order to reduce the worst-case latency without a significant coding loss. The experimental results show that the proposed change has negligable impact in terms of average BD-rates.

# Introduction

In the current VVC specification working draft, the LFNST index is signalled if the following set of conditions are satisfied:

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

where the  transform_skip_flag[ x0 ][ y0 ][ 0 ]  = =  0  is one of the conditions determining whether to signal lfnst_idx or not based on transform skip for luma component (i.e., transform_skip_flag[ x0 ][ y0 ][ cIdx ] for cIdx = 0).

# Proposed Method

This contribution document proposes two changes to the current VVC design to (i) remove the signaling dependency  between luma transform skip flag and chroma LFNST signaling, and (ii) reduce the worst-case delay in LFNST:

- The following new set of conditions are proposed to remove the dependency:

| LfnstTransformNotSkipFlag = ( treeType != DUAL_TREE_CHROMA ? transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 : ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 )) |  |
| --- | --- |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && LfnstTransformNotSkipFlag transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |

where luma LFNST signaling depends only on luma transform skip flag for both dual-tree and single-tree partition modes. When dual-tree mode is used, the chroma LFNST depends only on chroma transform skip flags.

- Chroma LFNST is disabled when single-tree partition mode is used to reduce worst-case latency.

In overall, the following set of highlighted changes are proposed on the VVC draft text (version JVET-P2001-vE):

#### 7.3.9.5 Coding unit syntax

…

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| LfnstTransformNotSkipFlag = ( treeType != DUAL_TREE_CHROMA ? transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 : ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 )) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && LfnstTransformNotSkipFlag transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

…

### 8.7.4 Transformation process for scaled transform coefficients

### 8.7.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array res[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable applyLfnstFlag is derived as follows:

- If treeType is equal to SINGLE_TREE and lfnst_idx is not equal to 0 and transform_skip_flag[ xTbY ][  yTbY ][ cIdx ] is equal to 0 and cIdx is equal to 0 and both nTbW and nTbH are greater than or equal to 4, applyLfnstFlag is set to 1.

- Otherwise, if treeType is not equal to SINGLE_TREE and lfnst_idx is not equal to 0 and transform_skip_flag[ xTbY ][  yTbY ][ cIdx ] is equal to 0 and both nTbW and nTbH are greater than or equal to 4, applyLfnstFlag is set to 1.

- Otherwise, applyLfnstFlag is set to 0.

When applyLfnstFlag is equal to 1 lfnst_idx is not equal to 0 and transform_skip_flag[ xTbY ][  yTbY ][ cIdx ] is equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(1149)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(1150)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(1151)

nLfnstSize = 1  <<  log2LfnstSize		(1152)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(1153)

…

# Experimental Results

The proposed method is implemented on top of VTM-7.0 and evaluated under common test conditions (CTC). Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1.  Results of the proposed method over VTM-7.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.05% | 101% | 100% |
| Class A2 | 0.00% | -0.02% | -0.02% | 99% | 97% |
| Class B | 0.00% | -0.02% | -0.07% | 102% | 104% |
| Class C | -0.01% | 0.04% | 0.03% | 99% | 95% |
| Class E | 0.00% | -0.02% | 0.01% | 99% | 92% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class D | -0.01% | -0.06% | 0.00% | 102% | 104% |
| Class F | -0.01% | -0.06% | 0.04% | 102% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.04% | 0.27% | 0.36% | 100% | 102% |
| Class C | 0.03% | -0.01% | 0.23% | 97% | 93% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.06% | 0.08% | 99% | 99% |
| Class F | 0.02% | 0.29% | 0.28% | 100% | 100% |

# Conclusions

This document proposes to restrict chroma LFNST signaling based on the chroma transform skip flags as a clean-up and removes chroma LFNST for single-tree partition mode to reduce the worst-case latency. It is recommended to adopt the proposed method in VVC.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).