| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0xxx-WD |
| --- | --- |

| Title: | Non-CE2: On palette predictor initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong Luo, Yan Ye, Ru-Ling Liao, Mohammed Golam Sarwer, Jie Chen, Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | jiancong.luo@alibaba-inc.com yunghsua@qti.qualcomm.com |
| Source: | Alibaba Group US, Qualcomm Incorporated |  |  |

_____________________________

## Proposed syntax and semantic changes

A new APS type is added for predictor palette initializer APS:

Table 6 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | PLT_APS | Predictor palette patameters |
| 3..7 | Reserved | Reserved |

7.3.2.5	Adaptation parameter set syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = PLT_APS ) |  |
| predictor_palette_initializer( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The syntax structure of the proposed predictor palette initializer is shown as follows:

Table 3: predictor palette initializer syntax structure

| predictor_palette_initializer( ) { | Descriptor |
| --- | --- |
| joint_predictor_palette_present_flag | u(1) |
| if( joint_predictor_palette_present_flag ) { |  |
| luma_entry_bitdepth_minus8 | ue(v) |
| chroma_entry_bitdepth_minus8 | ue(v) |
| num_predictor_palette_entries_minus1 | ue(v) |
| for ( comp = 0; comp < 3; comp++ ) { |  |
| for ( i=0; i<= num_predictor_palette_entries_minus1; i++ ) { |  |
| predictor_palette_entry[comp][i] | u(v) |
| } |  |
| } |  |
| separate_predictor_palette_present_flag | u(1) |
| } |  |
| if( separate_predictor_palette_present_flag ) { |  |
| chroma_predictor_palette_present_flag | u(1) |
| if (chroma_predictor_palette_present_flag == 1) { |  |
| chroma_entry_bitdepth_minus8 | ue(v) |
| num_chroma_predictor_palette_entries_minus1 | ue(v) |
| for ( comp = 1; comp < 3; comp++ ) { |  |
| for ( i=0; i<= num_chroma_palette_entries _minus1; i++ ) { |  |
| predictor_palette_entry[comp][i] | u(v) |
| } |  |
| } |  |
| luma_predictor_palette_present_flag |  |
| } |  |
| if (luma_predictor_palette_present_flag == 1) { |  |
| luma_entry_bitdepth_minus8 | ue(v) |
| num_luma_predictor_palette_entries_minus1 | ue(v) |
| for ( i=0; i<= num_luma_predictor_palette_entries_minus1; i++ ) { |  |
| predictor_palette_entry[0][i] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

joint_predictor_palette_present_flag equals to 1 specifies joint predictor palette syntax structure is present. joint_predictor_palette_present_flag equals to 0 specifies joint predictor palette syntax structure is not present.

separate_predictor_palette_present_flag equals to 1 specifies separate predictor palette syntax structure is present. separate_predictor_palette_present_flag equals to 0 separate predictor palette syntax structure is not present. separate_predictor_palette_present_flag is only signalled when joint_predictor_palette_present_flag equals to 1. When not present, it is inferred to 1.

chroma_predictor_palette_present_flag equals to 1 specifies the chroma predictor palette is present. chroma_predictor_palette_present_flag equals to 0 specifies the chroma predictor palette is not present.

luma_predictor_palette_present_flag equals to 1 specifies the luma predictor palette is present. luma_predictor_palette_present_flag equals to 0 specifies the luma predictor palette is not present. luma_predictor_palette_present_flag is only signalled when chroma_predictor_palette_present_flag equals to 1. When not present, the it is inferred to 1.

num_predictor_palette_entries_minus1, num_luma_predictor_palette_entries_minus1, num_chroma_predictor_palette_entries_minus1 specify the number of predictor palette entries - 1 for joint palette, luma palette and chroma palette respectively. Their values are in range from [0, PaletteMaxPredictorSize].

predictor_palette_entry[comp][i] is the i-th predictor palette entry of component comp. The bit depth of predictor_palette_entry is specified by the variable BitDepthY for Luma component and BitDepthC for chroma component.

luma_bit_depth_entry_minus8 + 8 and chroma_bit_depth_entry_minus8 + 8 are the length of predictor_palette_entry for luma and chroma components.

In the picture header, when sps_palette_enabled_flag equals to 1, a flag is signaled to indicate if the predictor_palette_aps_id is present. The predictor_palette_aps_id is the APS id of the APS that contains the predictor palette initializer which is used to initialize the predictor palette the picture associates to the picture header.

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_palette_enabled_flag ) { |  |
| predictor_palette_initializer_present_flag | u(1) |
| if( predictor_palette_initializer_present_flag ) |  |
| predictor_palette_aps_id | u(3) |
| } |  |
| … |  |
| } |  |

## Proposed initialization process changes

9.3	CABAC parsing process for slice data

9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in clause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.9.2 and one or more of the following conditions are true:

- The CTU is the first CTU in a slice.

- The CTU is the first CTU in a tile.

- The value of entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

- When ending the parsing of the CTU syntax in clause 7.3.9.2, entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables as specified in clause 9.3.2.3 is invoked with TableStateIdx0Wpp, TableStateIdx1Wpp, and TableMpsValWpp as outputs.

The storage process for palette mode predictor is applied as follows:

- When ending the parsing of the CTU syntax in clause 7.3.9.2, entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette mode predictor as specified in clause 9.3.2.7 is invoked with arrays PredictorPaletteSizeWPP and PredictorPaletteEntriesWpp as ouput.

The whole CABAC parsing process for a syntax element synEl is illustrated in Figure 11.

9.3.2	Initialization process

9.3.2.1	General

Outputs of this process are initialized CABAC internal variables.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the initialization process of predictor palette is specified in 9.3.2.6.

–	Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1488)

–	The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the synchronization process for context variables as specified in clause 9.3.2.4 is invoked with TableStateIdx0Wpp, TableStateIdx1Wpp, TableMpsValWpp as inputs and the synchronization process for palette predictor as specified in clause 9.3.2.8 is invoked with array PredictorPaletteSizeWPP and PredictorPaletteEntriesWpp as inputs.

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the initialization process of predictor palette is specified in 9.3.2.6

–	Otherwise, the initialization process for context variables is invoked as specified in clause 9.3.2.2 and the initialization process of predictor palette is specified in 9.3.2.6

The decoding engine registers ivlCurrRange and ivlOffset both in 16 bit register precision are initialized by invoking the initialization process for the arithmetic decoding engine as specified in subclause 9.3.2.5.

9.3.2.6

The predictor palette is initialized as follows:

- If predictor_palette_initializer_present_flag equals to 0, the array PredictorPaletteSize[ compIdx ], with compIdx = 0, 1 is initialized to 0.

- Otherwise, PredictorPaletteSize[ compIdx ], with compIdx = 0, 1 is derived as follows:

startComp = ( treeType = = DUAL_TREE_CHROMA )  ?  1 : 0

numComps = ( treeType = = SINGLE_TREE ) ? 3 : 	( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1

PredictorPaletteSize[startComp] =  (treeType = = SINGLE_TREE )  ? 	num_predictor_palette_entries_minus1 + 1 : (treeType = = DUAL_TREE_CHROMA) ? 	num_chroma_predictor_palette_entries_minus1 + 1 : num_luma_predictor_palette_entries_minus1 + 1

for( compIdx = startComp; compIdx < ( startComp + numComps ); compIdx++ )
	for( i = 0; i < PredictorPaletteSize[startComp]; i++ )
		PredictorPaletteEntries[ compIdx ][ i ] = predictor_palette_entry[ compIdx ][ i ]

9.3.2.7	Storage process for palette predictor

Inputs to this process are:

–	palette predictor size in array PredictorPaletteSize[ Idx ], with Idx = 0, 1

–	palette predictor in array PredictorPaletteEntries[ Idx ], with Idx = 0, 1, 2

Outputs of this process are:

–	The arrays tablePaletteSizeWPP, and tablePaletteEntriesWPP containing the values in array PredictorPaletteSize and PredictorPaletteEntries in clause 7.4.10.6 and 8.4.5.3

The storage process for context variables is illustrated as follows:

- If the current CTU is the first CTU in tile row and if entropy_coding_sync_enabled_flag equal to 1, involke the storage of palette predictor in arrays tablePaletteSizeWPP and tablePaletteEntriesWPP.

- Otherwise, arrays tablePaletteSizeWPP and tablePaletteEntriesWPP are not updated.

9.3.2.8	Synchronization process for palette predictor

Inputs to this process are:

–	The arrays tablePaletteSizeWPP, and tablePaletteEntriesWPP containing the values of the palette predictor size and entries used in the storage process of palette predictor.

Outputs of this process are:

–	The initialized palette predictor size and entries in array PredictorPaletteSize and PredictorPaletteEntries.

For each values in array PredictorPaletteSize and PredictorPaletteEntries are initialized to the corresponding entries in arrays tablePaletteSizeWPP and tablePaletteEntriesWPP.

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, 16th Meeting: Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).