| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0710 |
| --- | --- |

| Title: | Joint solution for GEO parameter adjustment (JVET-Q0168 and JVET-Q0336) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami, Masaru Ikeda, Teruhiko Suzuki (SONY) Naeri Park, Junghak Nam, Hyeongmun Jang, Jaehyun Lim, Seung Hwan Kim (LGE) | Email: | Yoshitaka.Morigami@sony.com naeri.park@lge.com |
| Source: | Sony Corporation and LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a joint solution of the JVET-Q0168 and JVE-Q0336 for GEO parameter adjustment. In the CE4 common base (JVET-P0884/P0885) there seems to be 3 inconsistencies in terms of region assignment A & B, offset change in direction and magnitude in specific angle indexes. This contribution proposes to fix those inconsistencies for the design cleanups. With proposed modifications, overall memory requirement can be reduced to 43 bytes by additional calculations utilizing pattern symmetry, while there is no impact in BD-Rate in RA and LB.

Proposed method

Method 1:

- Alignment on distance interval considering block shape (JVET-Q0168 [1])

- Alignment of edges with clock-wise movement (JVET-Q0339 [2])

Method 2:

- Alignment on distance interval considering block shape (JVET-Q0168 [1])

- Alignment of edges with clock-wise movement (JVET-Q0339 [2])

- Alignment of region A & B assignment in angle index 9 and 21 (JVET-Q0168 [1])

Proposed working draft changes

In the common base spec text [1], the newly added texts are highlighted and removed texts are marked as stricken-red-font text.

Method 1:

#### 8.5.7.2 Weighted sample prediction process for geo merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable angleIdx specifying the angle index of the geo partition,

- a variable distanceIdx specifying the distance index of the geo partition,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nW, nH and nCbR is derived as follows:

nCbWY = ( cIdx = = 0 )  ?  nCbW  :  nCbW * SubWidthC	(1027)

nCbHY = ( cIdx = = 0 )  ?  nCbH  :  nCbH * SubHeightC	(1028)

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − BitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

The values of the following variables are set:

–	hwRatio is set to nH / nW

–	displacementX is set to angleIdx

–	displacementY is set to (displacementX + 6)%24

–	hwOffset is set to Clip3(-2, 3, Log2(hwRatio))

–	If angleIdx >=10 && angleIdx <=20, PART1 and PART2 are set equal to A and B respectively, otherwise PART1 and PART2 are set equal to B and A respectively.

–	rho is set to the following value using the look-up tables denoted as Dis, specified in Table 8-12:

rho = (Dis[displacementX]<< 8) + (Dis[displacementY] << 8)

If one of the following conditons is true, variable shiftHor is set equal to 0:

angleIdx % 12 is equal to 6

angleIdx % 12 is not equal to 0 and hwRatio ≥ 1

3 − hwOffset < angleIdx % 12 < 9 + hwOffset

Otherwise, shiftHor is set equal to 1.

If shiftHor is equal to 0, offsetX and offsetY are derived as follows:

offsetX = ( 256 − nW ) >> 1

offsetY = ( 256 − nH ) >> 1 + angleIdx < 12 ? (distanceIdx * nH) >> 3 : −((distanceIdx * nH) >> 3)

Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows:

offsetX = ( 256 − nW ) >> 1 + angleIdx < 12 angleIdx % 18 >= 6 ? (distanceIdx * nW) >> 3 : −((distanceIdx * nW) >> 3)

offsetY = ( 256 − nH ) >> 1

–	The variables xL and yL are derived as follows:

xL = ( cIdx = = 0 ) ? x : x * SubWidthC

yL = ( cIdx = = 0 ) ? y : y * SubHeightC

–	The variable weightIdx and weightIdxAbs specifying the weight of the prediction sample is derived as follows:

- The variable weightIdx and weightIdxAbs are calculated using the look-up table Table 37 as follows:

weightIdx = (((xL + offsetX)<<1) + 1)*Dis[displacementX]

+ (((yL + offsetY )<<1) + 1))*Dis[displacementY] − rho.

weightIdxAbs = Clip3(0, 26, abs(weightIdx)).

- The value of wValue is derived according to according to Table 38 as follows:

wValue = weightIdx <= 0 ? GeoFilter[weightIdxAbs] : 8 − GeoFilter[weightIdxAbs]

NOTE – The value of sample wValue with located at (x , y)  can also be derived from  wValue located at ( x-shiftX , y-shiftY ). If the angleIdx is larger than 4 and smaller than 12, or angleIdx is larger than 20 and smaller than 24, shiftX is the tangent of the split angle and shiftY is 1, otherwise shiftX is 1 of the split angle and shiftY is cotangent of the split angle. If tangent (resp. cotangent) value is infinity, shiftX is 1 (resp. 0) or shift Y is 0 (reps. 1).

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, ( predSamplesLPART1[ x ][ y ] * wValue + 	(1032)
		predSamplesLPART2[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

Table 37 - Look-up table Dis for derivation of geoc partitioning distance.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 8 | 8 | 8 | 4 | 4 | 2 | 0 | −2 | −4 | −4 | −8 | −8 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | −8 | −8 | −8 | −4 | −4 | −2 | 0 | 2 | 4 | 4 | 8 | 8 |

Table 38 - Filter weight look-up table GeoFilter for derivation of geo partitioning filter weights.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter[idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter[idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

#### 8.5.7.3 Motion vector storing process for geo merge mode

This process is invoked when decoding a coding unit with MergeGeoFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable angleIdx specifying the angle index of the geo partition,

- a variable distanceIdx specifying the distance index of the geo partition,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The  value of the following variables are set:

- displacementX is set to angleIdx, displacementY is set to (displacementX + 6)%24

–	hwRatio is set equal to nCbH / nCbW

–	hwOffset is set to Clip3(-2, 3, Log2(hwRatio))

If one of the following conditons is true, variable shiftHor is set equal to 0:

angleIdx % 12 is equal to 8

angleIdx % 12 is not equal to 0 and hwRatio ≥ 1

3 − hwOffset < angleIdx % 12 < 9 + hwOffset

Otherwise, shiftHor is set equal to 1.

partIdx is set to  angleIdx >= 10 && angleIdx <=20 ? 1 : 0.

If shiftHor is equal to 0, offsetX and offsetY are derived as follows:

–	offsetX = ( 64 – numSbX ) >> 1

–	offsetY = ( 64 − numSbY ) >> 1 + angleIdx < 12 ? (distanceIdx * nCbH) >> 5 : −((distanceIdx * nCbH) >> 5)

Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows:

–	offsetX = ( 64 – numSbX ) >> 1 + angleIdx < 12 angleIdx % 18 >= 6 ? (distanceIdx * nCbW) >> 5 : −((distanceIdx * nCbW) >> 5)

–	offsetY = ( 64 – numSbY ) >> 1

The value of the variable rho is derived according to the equation below and according to the Dis lookup table specified in Table 8-12:

–	rho = (Dis[displacementX]<<8) + (Dis[displacementY] << 8) .

motionOffset is set equal to the following value using the look-up tables denoted as Dis, specified in Table 8-11 and Table 8-12:

–	motionOffset = 3*Dis[displacementX]+ 3*Dis[displacementY] .

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variable motionIdx is calculated using the look-up table Table 8-12  as following:

motionIdx = (((xSbIdx + offsetX) <<3) + 1)*Dis[displacementX]

+ (((xSbIdx + offsetY <<3) + 1))*Dis[displacementY] – rho + motionOffset

- The variable sType is derived as follows:

sType = abs(motionIdx)  < 32 ? 2 : motionIdx <= 0 ?  1 – partIdx : partIdx

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(1038)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(1039)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(1040)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(1041)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(1042)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(1043)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(1044)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(1045)

- Otherwise, if sType is equal to 1 or ( sType is equal to 2 and predListFlagA + predListFlagB is not equal to 1 ), the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(1046)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(1047)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(1048)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(1049)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(1050)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(1051)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(1052)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(1053)

- Otherwise (sType is equal to 2 and predListFlagA + predListFlagB is equal to 1), the following applies:

predFlagL0 = 1		(1054)

predFlagL1 = 1		(1055)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(1056)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(1057)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(1058)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(1059)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(1060)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(1061)

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(1062)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(1063)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(1064)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(1065)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(1066)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(1067)

Method 2:

#### 8.5.7.2 Weighted sample prediction process for geo merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable angleIdx specifying the angle index of the geo partition,

- a variable distanceIdx specifying the distance index of the geo partition,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nW, nH and nCbR is derived as follows:

nCbWY = ( cIdx = = 0 )  ?  nCbW  :  nCbW * SubWidthC	(1027)

nCbHY = ( cIdx = = 0 )  ?  nCbH  :  nCbH * SubHeightC	(1028)

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − BitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

The values of the following variables are set:

–	hwRatio is set to nH / nW

–	displacementX is set to angleIdx

–	displacementY is set to (displacementX + 6)%24

–	hwOffset is set to Clip3(-2, 3, Log2(hwRatio))

–	If angleIdx >=109+hwOffset && angleIdx <=20+hwOffset, PART1 and PART2 are set equal to A and B respectively, otherwise PART1 and PART2 are set equal to B and A respectively.

–	rho is set to the following value using the look-up tables denoted as Dis, specified in Table 8-12:

rho = (Dis[displacementX]<< 8) + (Dis[displacementY] << 8)

If one of the following conditons is true, variable shiftHor is set equal to 0:

angleIdx % 12 is equal to 6

angleIdx % 12 is not equal to 0 and hwRatio ≥ 1

3 − hwOffset < angleIdx % 12 < 9 + hwOffset

Otherwise, shiftHor is set equal to 1.

If shiftHor is equal to 0, offsetX and offsetY are derived as follows:

offsetX = ( 256 − nW ) >> 1

offsetY = ( 256 − nH ) >> 1 + angleIdx < 12 ? (distanceIdx * nH) >> 3 : −((distanceIdx * nH) >> 3)

Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows:

offsetX = ( 256 − nW ) >> 1 + angleIdx < 12 angleIdx % 18 >= 6 ? (distanceIdx * nW) >> 3 : −((distanceIdx * nW) >> 3)

offsetY = ( 256 − nH ) >> 1

–	The variables xL and yL are derived as follows:

xL = ( cIdx = = 0 ) ? x : x * SubWidthC

yL = ( cIdx = = 0 ) ? y : y * SubHeightC

–	The variable weightIdx and weightIdxAbs specifying the weight of the prediction sample is derived as follows:

- The variable weightIdx and weightIdxAbs are calculated using the look-up table Table 37 as follows:

weightIdx = (((xL + offsetX)<<1) + 1)*Dis[displacementX]

+ (((yL + offsetY )<<1) + 1))*Dis[displacementY] − rho.

weightIdxAbs = Clip3(0, 26, abs(weightIdx)).

- The value of wValue is derived according to according to Table 38 as follows:

wValue = weightIdx <= 0 ? GeoFilter[weightIdxAbs] : 8 − GeoFilter[weightIdxAbs]

NOTE – The value of sample wValue with located at (x , y)  can also be derived from  wValue located at ( x-shiftX , y-shiftY ). If the angleIdx is larger than 4 and smaller than 12, or angleIdx is larger than 20 and smaller than 24, shiftX is the tangent of the split angle and shiftY is 1, otherwise shiftX is 1 of the split angle and shiftY is cotangent of the split angle. If tangent (resp. cotangent) value is infinity, shiftX is 1 (resp. 0) or shift Y is 0 (reps. 1).

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, ( predSamplesLPART1[ x ][ y ] * wValue + 	(1032)
		predSamplesLPART2[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

Table 37 - Look-up table Dis for derivation of geoc partitioning distance.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 8 | 8 | 8 | 4 | 4 | 2 | 0 | −2 | −4 | −4 | −8 | −8 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | −8 | −8 | −8 | −4 | −4 | −2 | 0 | 2 | 4 | 4 | 8 | 8 |

Table 38 - Filter weight look-up table GeoFilter for derivation of geo partitioning filter weights.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GeoFilter[idx] | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 |
| idx | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |  |
| GeoFilter[idx] | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 |  |

#### 8.5.7.3 Motion vector storing process for geo merge mode

This process is invoked when decoding a coding unit with MergeGeoFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable angleIdx specifying the angle index of the geo partition,

- a variable distanceIdx specifying the distance index of the geo partition,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The  value of the following variables are set:

- displacementX is set to angleIdx, displacementY is set to (displacementX + 6)%24

–	hwRatio is set equal to nCbH / nCbW

–	hwOffset is set to Clip3(-2, 3, Log2(hwRatio))

If one of the following conditons is true, variable shiftHor is set equal to 0:

angleIdx % 12 is equal to 8

angleIdx % 12 is not equal to 0 and hwRatio ≥ 1

3 − hwOffset < angleIdx % 12 < 9 + hwOffset

Otherwise, shiftHor is set equal to 1.

partIdx is set to  angleIdx >=109+hwOffset && angleIdx <=20+hwOffset ? 1 : 0.

If shiftHor is equal to 0, offsetX and offsetY are derived as follows:

–	offsetX = ( 64 – numSbX ) >> 1

–	offsetY = ( 64 − numSbY ) >> 1 + angleIdx < 12 ? (distanceIdx * nCbH) >> 5 : −((distanceIdx * nCbH) >> 5)

Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows:

–	offsetX = ( 64 – numSbX ) >> 1 + angleIdx < 12 angleIdx % 18 >= 6 ? (distanceIdx * nCbW) >> 5 : −((distanceIdx * nCbW) >> 5)

–	offsetY = ( 64 – numSbY ) >> 1

The value of the variable rho is derived according to the equation below and according to the Dis lookup table specified in Table 8-12:

–	rho = (Dis[displacementX]<<8) + (Dis[displacementY] << 8) .

motionOffset is set equal to the following value using the look-up tables denoted as Dis, specified in Table 8-11 and Table 8-12:

–	motionOffset = 3*Dis[displacementX]+ 3*Dis[displacementY] .

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variable motionIdx is calculated using the look-up table Table 8-12  as following:

motionIdx = (((xSbIdx + offsetX) <<3) + 1)*Dis[displacementX]

+ (((xSbIdx + offsetY <<3) + 1))*Dis[displacementY] – rho + motionOffset

- The variable sType is derived as follows:

sType = abs(motionIdx)  < 32 ? 2 : motionIdx <= 0 ?  1 – partIdx : partIdx

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(1038)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(1039)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(1040)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(1041)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(1042)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(1043)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(1044)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(1045)

- Otherwise, if sType is equal to 1 or ( sType is equal to 2 and predListFlagA + predListFlagB is not equal to 1 ), the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(1046)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(1047)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(1048)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(1049)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(1050)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(1051)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(1052)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(1053)

- Otherwise (sType is equal to 2 and predListFlagA + predListFlagB is equal to 1), the following applies:

predFlagL0 = 1		(1054)

predFlagL1 = 1		(1055)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(1056)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(1057)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(1058)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(1059)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(1060)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(1061)

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(1062)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(1063)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(1064)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(1065)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(1066)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(1067)

Experimental results

The proposed method has been implemented on top of the VTM7.0 common base for GEO merge mode. The experimental results of the Method1 reportedly show -0.02%, -0.01% BD-rate changes with negligible changes in encoding and decoding run-time compared to VTM7.0 common base as anchor in RA, LB configurations. Also, the experimental results of the Method2 reportedly show -0.01%, -0.02% BD-rate changes with negligible changes in encoding and decoding run-time compared to VTM7.0 common base as anchor in RA, LB configurations.

Method 1:

|  | Over VTM 7.0 Common base |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | -0.02% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.01% | 0.05% | -0.04% | 102% | 100% |  |  |  |  |  |
| Class B | -0.02% | 0.00% | -0.02% | 100% | 101% | -0.03% | -0.15% | -0.02% | 101% | 102% |
| Class C | -0.07% | 0.09% | 0.00% | 100% | 102% | -0.05% | -0.23% | -0.24% | 100% | 100% |
| Class E |  |  |  |  |  | 0.05% | -0.03% | 0.24% | 100% | 101% |
| Overall | -0.02% | 0.02% | -0.02% | 101% | 101% | -0.01% | -0.15% | -0.03% | 100% | 101% |
| Class D | -0.02% | -0.04% | 0.06% | 100% | 100% | -0.03% | -0.01% | 0.19% | 100% | 100% |
| Class F | -0.02% | -0.02% | -0.01% | 100% | 100% | -0.08% | 0.01% | -0.33% | 100% | 100% |

Method 2:

|  | Over VTM 7.0 Common base |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.02% | 101% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | -0.04% | -0.11% | 102% | 100% |  |  |  |  |  |
| Class B | -0.02% | 0.01% | -0.01% | 100% | 101% | -0.01% | -0.19% | 0.13% | 101% | 101% |
| Class C | -0.03% | 0.02% | -0.05% | 100% | 100% | -0.05% | 0.07% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  | 0.15% | 0.03% | -0.06% | 101% | 99% |
| Overall | -0.01% | -0.02% | -0.04% | 101% | 100% | 0.02% | -0.05% | 0.05% | 100% | 100% |
| Class D | -0.03% | -0.07% | 0.06% | 100% | 102% | -0.01% | -0.34% | 0.14% | 100% | 101% |
| Class F | -0.03% | 0.01% | 0.04% | 100% | 100% | -0.11% | -0.03% | -0.47% | 100% | 101% |

Conclusion

This contribution proposed design cleanups to have more consistencies on wedge partition mode. With proposed modifications, overall memory requirement can be reduced to 43 bytes by additional calculations utilizing pattern symmetry, while there is no impact in BD-Rate. It is suggested that these modifications are adopted into the next version of VVC.

Reference

- Han Gao, et al., “CE4 Common Base: Geometric inter prediction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0079, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Yoshitaka Morigami, et al., “CE4-related: On GEO cleanups”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0168, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Naeri Park, et al., “CE4-related: Adjustment of the distance on the GEO mode”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0339, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).