| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0723-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0227: Dependent Quantization with Qp offset adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz | Email: | heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The cross checker reports that the source code provided by InterDigital was carefully analyzed and found to be consistent with the description in the input document JVET-Q0227.  The cross checker could exactly reproduce the results reported in JVET-Q0227.  The measured encoder and decoder run time ratios are similar to those reported in JVET-Q0227.

# Source Code Analysis

The source code was carefully analyzed. It is confirmed that the implementation matches the description.

The contribution JVET-Q0227 proposes an QP-dependent QP offset for dependent quantization. Instead of a fixed offset of 1 (as in VTM-7), it is proposed to derive the QP offset as follows:

- If qP > (30 + QpBdOffset), qPOffset = 2

- Otherwise, if qP > (23 + QpBdOffset), qPOffset = 1

- Otherwise, qPOffset = 0

# Simulation results

Simulations were conducted for all configurations and test sequences specified in the Common Test Conditions.  The simulation results for JVET-Q0227 are compared to VTM-7.0. The complete simulation results are provided in the accompanying Excel documents. A summary of the results is shown in the following tables.  The bit rates and PSNR values exactly match the results reported in JVET-Q0227. Encoding/Decoding runtimes are reported to be of same order as the ones reported by the proponent.

Table 1:  Average simulation results for the proposed QP offset modification.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.24% | 98% | 101% |
| Class A2 | -0.32% | 0.34% | 0.26% | 97% | 97% |
| Class B | -0.06% | 0.33% | 0.17% | 99% | 97% |
| Class C | 0.01% | 0.45% | 0.37% | 99% | 96% |
| Class E | -0.03% | 0.30% | 0.41% | 99% | 97% |
| Overall | -0.07% | 0.29% | 0.28% | 98% | 97% |
| Class D | 0.00% | 0.49% | 0.49% | 99% | 99% |
| Class F | -0.08% | 0.28% | 0.34% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.21% | -0.26% | 96% | 97% |
| Class A2 | -0.20% | -0.34% | -0.47% | 99% | 97% |
| Class B | -0.08% | -0.32% | -0.31% | 98% | 98% |
| Class C | -0.02% | -0.35% | -0.34% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.23% | -0.34% | 98% | 98% |
| Class D | -0.05% | -0.38% | -0.44% | 99% | 101% |
| Class F | -0.07% | -0.08% | -0.17% | 98% | 99% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.22% | -0.12% | 98% | 99% |
| Class C | -0.26% | 0.04% | 0.04% | 99% | 96% |
| Class E | -0.04% | 0.64% | -0.54% | 96% | 101% |
| Overall | -0.18% | 0.08% | -0.17% | 98% | 98% |
| Class D | -0.02% | 0.44% | 0.19% | 97% | 98% |
| Class F | -0.34% | -0.57% | 0.52% | 98% | 98% |