| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0780 |
| --- | --- |

| Title: | BoG Report on CE5 Related Contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Ching-Yeh Chen | Tel: Email: | chingyeh.chen@mediatek.com asegall@sharplabs.com |
| Source: | Chairs |  |  |

_____________________________

# Abstract

This contribution provides DRAFT notes of the BoG on CE5 related contributions.  The BoG met during the following time:

January 10th from 8:00-10:30AM

January 10th from 5:00-6:45PM

January 10th from 8:00-10:00PM

The mandate of the BoG was to review all CE5 related contributions.

Recommendations were determined during the BoG meetings.  However, it was largely agreed that most of the recommendations should be revisited by the BoG subject to JVET decisions on CE5.  Thus, the BoG has not yet closed and would plan to reconvene as needed.

# Notes

Notes from the BoG discussion are captured below.

## CE5 related – Cross-component adaptive loop filtering (24)

Contributions in this category were discussed Friday 10 Jan. 8000–1030 in BoG (chaired by Andrew Segall).

JVET-Q0074 CE5-related: Performance of CE5 anchor with 8-bit dynamic range for CC-ALF coefficients [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

No need to present.

JVET-Q0124 CE5-related: JC-CCALF with Power of 2 Weight Values [C.-W. Kuo (Panasonic)]

This document proposes to use power of 2 values for Joint Chroma Cross Component Adaptive Loop Filter (JC-CCALF) weight candidates to avoid multiplication or division in Cr refinement value derivation. In method 1, the weight candidates are restricted to ±1. In method 2, the weight candidates are extended to 16 weights: ±1/8, ±1/4, ±1/2, ±1, ±2, ±4, ±8, and ±16. Compared with the JC-CCALF in CE5-3.2, negligible coding difference and runtime change are observed.

Simulation results with joint YUV performance (AHG13 8:1:1 metric) are summarized as follows as compared to a VTM anchor.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| CCALF (CE5 anchor) | 0.17% | -8.31% | -7.06% | -1.40% | 0.14% | -10.14% | -9.82% | -1.88% | 0.12% | -13.00% | -9.48% | -2.15% |
| JC-CCALF (CE5-3.2) | 0.10% | -5.84% | -4.87% | -0.99% | 0.16% | -8.63% | -8.27% | -1.56% | 0.12% | -12.71% | -9.47% | -2.12% |
| weights = ±1 | 0.10% | -6.15% | -5.02% | -1.04% | 0.15% | -9.05% | -8.73% | -1.66% | 0.12% | -12.86% | -9.89% | -2.18% |
| weights = ±1/8 to ±16 | 0.10% | -6.06% | -5.09% | -1.04% | 0.14% | -8.91% | -8.56% | -1.64% | 0.12% | -12.88% | -9.83% | -2.17% |

The proponent asserts that the proposal is a simplification of the method studied in CE5-3.2.  Additionally, it is observed that there is a small improvement in the performance relative to the proposal studied in CE5-3.2.

One participant commented that the proposal looks interesting and simplification appears clear.

It was later discussed during the presentation of JVET-Q0125, that JVET-Q0125 might be a more preferable way forward.

JVET-Q0653 Cross-check of JVET-Q0124 (CE5-related: JC-CCALF with Power of 2 Weight Values) [Y.-W. Chen (Kwai Inc.)] [late]

Cross-check was not presented.

JVET-Q0125 CE5-related: JC-CCALF with Alternative Filter Weight [C.-W. Kuo (Panasonic)]

This contribution proposes to use individual JC-CCALF filter weight for each alternative filter. Specifically, instead of using various weight values as in JC-CCALF, only sign changes are applied for 4 alternative filters, e.g., +--+. In decoder, the current CTU selects the filter weight according to the parsed alternative filter index. Compared to the JC-CCALF in CE5-3.2, coding gain improvement and negligible runtime change are observed.

Simulation results with joint YUV performance (AHG13 8:1:1 metric) are summarized as follows.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| CCALF (CE5 anchor) | 0.17% | -8.31% | -7.06% | -1.40% | 0.14% | -10.14% | -9.82% | -1.88% | 0.12% | -13.00% | -9.48% | -2.15% |
| JC-CCALF (CE5-3.2) | 0.10% | -5.84% | -4.87% | -0.99% | 0.16% | -8.63% | -8.27% | -1.56% | 0.12% | -12.71% | -9.47% | -2.12% |
| alternative filter weight | 0.10% | -6.65% | -5.56% | -1.14% | 0.15% | -9.39% | -9.16% | -1.74% | 0.11% | -13.03% | -10.11% | -2.22% |

The propoponent asserts that the proposal provides coding efficiency improvement of the method studied in CE5-3.2.  Using the YUV metric, the gain appears to be 0.1% relative to the method 1 in JVET-Q0125. (The JVET-Q0125 method proposes to share a single +/-1 for all filters.)

It was commented that the filter weights are signaled in the slice header, as is done for JVET-CE5-3.2.  It was commented that this could be included in the APS.

It was commented by multiple participants that the method in JVET-Q0125 would be more interesting than JVET-Q0124 as a way forward.

It was commented that the method requires additional searching at the encoder for the weights of the different filters.

Recommendation: Revisit proposal if CE5-3.2 is adopted.

JVET-Q0585 CE5-related: cross-check report of JVET-Q0125 on JC-CCALF with Alternative Filter Weight [E. François (InterDigital)] [late]

The cross-checker reported that the method looks interesting and provides both a coding efficiency improvement and simplification compared the method studied in CE5-3.2.

JVET-Q0165 CE5-related: On the CC-ALF filtering process [Z. Zhang, J. Ström, K. Andersson (Ericsson)]

The proponent asserts that a common feature to all tests in CE5 is that the filters in CC-ALF are not limited to be DC neutral, which means that it is not guaranteed that chroma unchanged if luma is flat. The main aspect with this contribution is to address that issue. A common feature of CE5-1.1 and CE5-2.2 is to remove the multiplications of CC-ALF filtering process. This contribution presents results of a DC neutral design on top of CE5 anchor, CE5-2.1 and CE5-2.2.

Compared to the CE5-CC-ALF anchor, the modified version of CE5-2.1 has a Y/U/V BD-rate impact of
 -0.02%/-0.02%/-0.19% (AI), 0.00%/-0.28%/-0.31% (RA) and 0.03%/-0.51%/-0.63% (LDB).

Compared to CE5-2.1, the modified version of CE5-2.1 has a Y/U/V BD-rate impact of 
0.00%/-0.55%/-0.54% (AI), -0.02%/-0.36%/-0.37% (RA) and 0.03%/-0.27%/-0.50% (LDB).

Compared to the CE5-CC-ALF anchor, the modified version of CE5-2.2 has a Y/U/V BD-rate impact of
-0.03%	/0.15%/-0.06% (AI), -0.01%/-0.13%/-0.13% (RA) and 0.00%/-0.55%/-0.63% (LDB).

Compared to CE5-2.2, the modified version of CE5-2.2 has a Y/U/V BD-rate impact of
0.00%	/-0.58%/-0.65% (AI), -0.01%/-0.47%/-0.57% (RA) and -0.01%/-0.47%/-0.53% (LDB).

It was commented that there are two changes in the proposal.  The first is to reduce the number of signaled coefficients from 8 to 7.  And, the second change is to limit the residual correction to be a zero-sum filter (e.g., DC neurtral).

It was commented that the method requires the calculation of the difference between samples, which increases the bit-depth of the multiplication.

It was asked if there would be a change in performance if the bit-depth of the weight C_sum was limited to the bit-depth of the signaled coefficients.  The proponent asserted that the impact may be small, but the results were unknown.

It was commented that the approach reduces the number of multiplies (or shifts) by one, but it requires an additional seven adds/subtracts to calculated the residual values.  As a result, the proposed method may have higher complexity than the methods studied in CE5. It was further commented that it may be possible to reuse the residual calculation in ALF to not require the additional 7 adds/subtracts.

It was commented that a similar method is proposed in JVET-Q0190.

JVET-Q0544 Crosscheck of JVET-Q0165 Test 1 (CE5-related: On the CC-ALF filtering process) [O. Chubach (MediaTek)] [late]

No issues were reported by the cross-checker.

JVET-Q0607 Crosscheck of JVET-Q0165 (CE5-related: On the CC-ALF filtering process) [N. Hu (Qualcomm)] [late]

No issues were reported by the cross-checker.  Furthermore, it was reported that the encoder was observed to be very straightforward.

JVET-Q0167 Non-CE5: Multiplication simplification for ALF and CC-ALF [J. Ström, Z. Zhang, K. Andersson (Ericsson)]

Document to be re-categorized as an ALF related document.

JVET-Q0608 Crosscheck of JVET-Q0167 (Non-CE5: Multiplication simplification for ALF and CC-ALF) [N. Hu (Qualcomm)] [late]

See above.

JVET-Q0190 CE5-related: On CC-ALF modifications related to coefficients and signalling [O. Chubach, C.-Y. Chen, C.-Y. Lai, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution contains two aspects related to CC-ALF. Aspect 1 limits the sum of the coefficients of CC-ALF filters to be equal to 0. This way, it is guaranteed that CC-ALF has a zero response at DC frequency, which results in CC-ALF behaving as a high pass, band pass, or other zero-DC filter. Aspect 2 implies updating the syntax of CC-ALF such that it is aligned to ALF process: disabling CC-ALF when luma ALF is disabled.

The combined results of Aspect 1 and Aspect 2 are as follows (over the CC-ALF common base, Y/Cb/Cr):
In AI: -0.01% / -0.06% / -0.11%
In RA: 0.00% / -0.10% / -0.06%
In LB: -0.01% / -0.22% / -0.05%

The results of the two aspects were reported separately.  For the first aspect, the YUV improvement for AI, RA and LDB was 0.00%, 0.00% and 0.00% respectively.

For the second aspect, the YUV improvement for AI, RA and LDB were reported to be -0.02%, -0.02%, and -0.04%.

For the combined result, the YUV improvement for AI, RA and LDB

It was commented that the presented slides do not match the uploaded slides, which resulted in some confusion amongst participants.  The proponent was requested to upload a revision of the slides.

It was commented that the implementation includes a conformance constraint tha the inferred coefficient is required to be in the range of the signaled coefficients.

It was asked if for Aspect 2, if enabling CC-ALF should be conditioned on if chroma ALF is enabled, as opposed to the proposal that conditions enabling CC-ALF on if luma ALF is enabled.

One participant suggested that there had not been study on if an encoder might want to use CC-ALF when luma ALF was disabled.

One participant commented that the Aspect 2 seems well motivated.

Another participant commented that there is a loss for Aspect 2 for Class F for LDB.

It was commented one aspect of JVET-Q0304 is related to the first aspect of this proposal.

It was commented that JVET-Q0253 and JVET-Q0382 may be related to the second aspect of this proposal.

In further discussion it was suggested to recommend that the conditional signaling of CC-ALF on luma ALF be incorporated to a future design.

Recommendation: Adopt the Aspect 2 (Disabling CC-ALF when luma ALF is disabled) from JVET-Q0190-v1.docx subject to decision on CC-ALF.

JVET-Q0648 Cross-check of JVET-Q0190 (CE5-related: On CC-ALF modifications related to coefficients and signaling) [T.-C. Ma (Kwai Inc.)] [late]

The cross-checker expressed support for the proposal and asserted that it was consistent with how chroma ALF is enabled.

JVET-Q0251 CE5-related: Unified cross component adaptive loop filter [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. To get an offset for a chroma sample, the filter is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample. In an APS, a Cb (Cr resp.) CC-ALF filter set with up to 4 filters could be signalled and a filter from this Cb (Cr resp.) filter set could be applied to a Cb (Cr resp.) coding tree block. On the other hand, in an APS, a chroma adaptive loop filter (ALF) filter set with up to 8 filters could be signalled and a filter from this chroma filter set could be applied to a chroma coding tree block. In this contribution, CC-ALF filters are unified for the two chroma components, where Cb and Cr components share the same filters in an APS, which is the same method used in a regular chroma ALF. Under common test conditions, compared with CE5 anchor, the average BD rate of the proposed method is

AI   : -0.01%(Y), 0.05%(U), -0.17%(V)

RA  : 0.02%(Y), -0.04%(U), -0.06%(V)

LDB: 0.xx%(Y), 0.xx%(U), 0.xx%(V)

LDP: 0.04%(Y), -0.12%(U), -0.40%(V)

Proposes to allow the signaling of eight filters in the APS and then selecting any of these eight filters for the Cb and Cr component.  It is asserted that this unifies the design with the current chroma ALF design, as well as providing some additional flexibility.

One participant suggested that this may be a good direction.

It was commented that the approach would not be applicable if the joint chroma CC-ALF solution would be adopted.

Recommendation: Revisit if the Joint Chroma desing would not be selected.

JVET-Q0736 Cross-check result of JVET-Q0251: CE5-related: Unified cross component adaptive loop filter [Z. Zhang (Ericsson)] [late]

The cross-checker reported that the encoder was different than the common base.  The cross-checker reported that they investigated this in more detail and concluded that the impact of the difference was very minor.

JVET-Q0253 CE5-related: High level syntax modifications for CCALF [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

This contribution proposes to decouple the high level syntax control of Adaptive loop filter (ALF) and Cross component adaptive loop filter (CCALF) completely by introducing a new sequence level syntax element called “sps_ccalf_enabled_flag” and the corresponding new entries for CCALF in the picture header and slice header.  This decoupling of high level syntax control of ALF and CCALF reportedly provides encoders with better flexibility while encoding sequences with different signal characteristics.

It was commented that results were not provided.

One participant questioned if a SPS level flag is needed. A flag in the picture header may be sufficient.

One participant that CC-ALF should be treated as a sub-set of the ALF process, and so enabling CC-ALF should be dependent on if ALF is enabled.

One participant commented that the benefit would be to save 2-bits in the picture header.

It was commented that one aspect of JVET-Q0382 is the same as this proposal.

It was commented by one participant that SPS level control may not be needed.

Recommendation: No action on the SPS level control aspect

JVET-Q0296 Non-CE5: Adaptive precision for CCALF coefficients [L.-H. Xu, J. Yao, J.-Q. Zhu, K. Kazui (Fujitsu)]

This contribution proposes a method to increase the precision of CCALF coefficients without computational complexity or coefficients length increment. It is proposed to use an adaptive bitshift instead of constant bitshift to fix the floating filter coefficients when encoding. Simulation results show 0.01%(Y), -0.29%(U), -0.24%(V) BD-rates change for AI and 0.01%(Y), -0.40%(U), -0.41(V) BD-rates change for RA.

The proposal signals a bit-shift for each filter in the bit-stream.  The coefficient bit-depth is maintained at 6 bits.  The signaled bit-shift values were in the range of 5 to 12.

It was further reported that using the range of 7 to 11 had similar performance.

It was noted that the CE5 Common Base used a fixed bit-shift of 7.

A question was raised on how the encoder would determine the adaptive shift value.  It was described that the encoder determined the maximum value of the coefficents and then selected the value of the shift based on this maximum value.

One participant asked why the dynamic range was chosen.

The proponent commented that the approach could be applied to most methods of signaling the filter that used multiplication (as opposed to methods that use power of two).

It was commented that the approach may create an issue when creating a DC neutral filter.

It was commented that the approach may require more bits for the multiplication.

Current recommendation is no action, but revisit after decision regarding CCALF status.

JVET-Q0301 CE5-related: CCALF filter for 4:2:2 and 4:4:4 color format [J. Choi, J. Nam, J. Heo, J. Lim, S. Kim (LGE)]

This contribution proposes to use simplified cross-component ALF (CCALF) filter shape for 4:2:2 and 4:4:4 color format. Considering the different chroma sampling structure of 4:2:0, 4:2:2 and 4:4:4, the CCALF filter shape is reduced from 3x4 diamond (non-square 8-tap) to 3x3 diamond (square 5-tap) in 4:2:2 and 4:4:4 color format, so that the symmetrical luma samples can be used for the chroma refinement and the number of CCALF operations could be reduced by 38% without any syntax change of the CCALF common base.

Experimental results for the proposed method with 4:4:4 test sequences show 0.08%, -1.33%, -1.95% BD-rate changes on Y, Cb, and Cr components in AI, 0.10%, -1.44%, -2.32% in RA, and -0.04%, -2.49%, -3.64% in LDB configuration, compare to the VTM7.0 anchor.

In addition, compared to the common base of CCALF, it provides -0.01%, 0.15%, 0.16% BD-rate changes in AI, -0.02%, 0.17%, 0.22% in RA, and 0.02%, 0.36%, 0.26% in LDB configuration

In the proposal, the syntax is not changed.  This means that 8 filter coefficients are still transmitted for the 4:2:2 and 4:4:4 case, but only 5 of the filter coefficients are used.

It was commented that the results show that the loss is higher for camera captured content.

It was also commented that the results presented do not follow the CTC.

It was commented that JVET-Q0467 is also related to this contribution.

It was commented that using a different filter shape for different color formats may not be desireable.

It was suggested that the proponent discuss with additional hardware experts to determine if the complexity of the filter for 4:2:2 and 4:4:4 content is a concern.

JVET-Q0660 Crosscheck of JVET-Q0301 "CE5-related: CCALF filter for 4:2:2 and 4:4:4 color format" [G. Li (Tencent)] [late]

Cross checker reported that all results matched.

JVET-Q0304 Non-CE5: Non-Linear Cross Component Adaptive Loop Filter [J. Yao, L.-H. Xu, J.-Q. Zhu, K. Kazui (Fujitsu)]

This contribution proposes a “Non-Linear Cross Component Adaptive Loop Filter (NL-CCALF)” approach that is assereted to CCALF more efficient by using a clipping operation to reduce the impact of neighbor sample values when they are too different with the current sample value being filtered.

Additionally, the contribution proposes to enforce a DC neutrality constraint on the CC-ALF filter.

The results reportedly show the average BDR gains for AI, RA and LDB are -0.08%, -0.11% and -0.16% respectively, when compared to the CE5 Anchor and using the AHG13 YUV metric.

The tool is enabled when signaling the APS.

It is asserted by the proponent that the clipping approach is the same as what is used in luma ALF.

It was commented that the approach is the same as what was proposed in JVET-Q0165.

It was commented that the gains for the clipping approach for luma ALF were bigger than was is reported here.  It was further noted that the gain for luma ALF were on the order of 0.5%.

It was commented by multiple participants that it was unclear if the added complexity justified the gains. It was further commented that in software the approach may double the number of operations.

A participant commented that the run-time performance of the approach were reported to be faster than the anchor.  The proponent responded that the run-time information was not reliable.

Recommendation: Further study

DC neutrality discussion:

The previous proposals on DC neutrality was then discussed.

It was commented that the DC neutrality could be achieved in either a normative or non-normative approach.

It was then commented that the DC neutrality constraint may be more difficult to achieve non-normatively in the case that CC-ALF uses a multiplication free design.

More than one participant expressed that the benefit of the DC neutrality constraint is not clear.

It was commented that there may be an increase in complexity due to the increase in the number of additions.  However, this may not be a significant issue.

Recommendation: Further study

JVET-Q0310 Non-CE5: Suggested text for CC-ALF padding process with raster scan slices [Y. Wang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

This contribution presents suggested text changes for CC-ALF padding process with raster-scan slices. More specifically, it is proposed to align the luma sample padding process in CC-ALF to ALF for raster-scan slices. It is reported that no performance changes are observed in the common test conditions.

It was commented that the design intent of CC-ALF is to re-use the boundary handling of ALF.

It was commented that a similar change was proposed in JVET-Q0073 and JVET-Q0085.

Recommendation: (Bugfix) Align boundary handling of CC-ALF specification text with current ALF boundary handing.

JVET-Q0311 Non-CE5: On CC-ALF padding for ALF virtual boundaries [Y. Wang, H. Liu, L. Zhang, K. Zhang, Z. Deng, Y. Wang (Bytedance)]

In this contribution, two methods are proposed on CC-ALF padding for virtual boundaries. In method #1: filtering chroma samples between ALF virtual boundary and CTU bottom boundary are disabled. In method #2, a different symmetric padding method is proposed to replace existing CC-ALF padding method for ALF virtual boundaries. Compared with CE5 common base, {Y, U, V} BD-rate changes of these two methods are reportedly shown as follows:

Method #1: {0.00%, 0.22%, 0.19%}, {0.00%, 0.16%, 0.22%}, and {0.00%, 0.15%, 0.24%} for AI, RA, and LDB, respectively.

Method #2: {0.00%, 0.00%, 0.01%}, {0.00%, -0.01%, 0.00%}, and {0.01%, -0.07%, 0.11%} for AI, RA, and LDB, respectively.

It was commented that the boundary padding method for the CC-ALF common base is currently the same as the ALF boundary padding method for design consistency.

It was commented that the shape of the filter may affect this proposal.

It was commented that the proposal would require performing the padding process twice – one time for luma ALF and another for CC-ALF.

JVET-Q0537 Crosscheck of JVET-Q0311 (Non-CE5: On CC-ALF padding for ALF virtual boundaries) [T. Ikai (Sharp)] [late]

JVET-Q0382 CE5-related: On high level syntax of CC-ALF [F. Chen, L. Xu, L. Wang (Hikvision)]

This proposal reports the modifications on the high level syntax of cross component adaptive loop filter (CC-ALF). First, it introduces a new SPS level syntax for CC-ALF to decouple of high level syntax control of ALF and CCALF. Second, a bug that CC-ALF related syntax in picture header (PH) is still signalled even if two chroma arrays are unavailable is fixed. Third, the numbers of filters for CC-ALF in slice and picture header are removed.

It was reported that Aspect 2 and Aspect 3 are also proposed in JVET-Q0171, JVET-Q0250, JVET-Q0326, and JVET-Q0520.

Recommendation: Incorporate the second and third aspects into the CC-ALF design

JVET-Q0467 CE5-related: Simplified CCALF [G. Li, X. Li, X. Xu, S. Liu (Tencent)]

In this contribution, a CCALF filter was proposed to reduce computational complexity and signaling cost. The proposed methods were implemented on top of CE5 anchor software, and simulations were conducted with common test conditions. It is reported that the proposed method 1 has following performance with 25% less multiplications:

- Over VTM-7, BD rate changes are 0.16%, -7.83%, -6.75% for AI, 0.13%, -9.92%, -9.65% for RA, 0.09%，-13.09%，-9.67% for LB, 0.15%, -12.84%, -9.61% for LP.

- Over CE5 Anchor, BD rate changes are -0.01%, 0.54%, 0.37% for AI, -0.01%, 0.29%, 0.25% for RA, -0.03%， -0.08%， -0.11% for LB, 0.01%, 0.10%, -0.30% for LP.

The proposed method 2 has following performance with 37.5% less multiplications:

- Over VTM-7, BD rate changes are 0.14%, -7.37%, -6.32% for AI, 0.14%, -9.59%, -9.13% for RA, 0.12%，-12.31%，-8.88% for LB, 0.13%, -12.28%, -8.76% for LP.

- Over CE5 Anchor, BD rate changes are -0.03%, 1.07%, 0.87% for AI, 0.00%, 0.70%, 0.83% for RA, 0.00%， 0.88%， 0.82% for LB, -0.01%, 0.77%, 0.73% for LP.

It was asserted by the proponent that “method 1” may also provide benefit for 4:2:2 and 4:4:4 content. However, results are not yet available.  The proponent reported that they could be provided.

JVET-Q0624 Crosscheck of JVET-Q0467 (CE5-related: Simplified CCALF) [J. Choi (LGE)] [late]

No issues were reported.

JVET-Q0494 CE5-related: Joint clip operation for CCALF and chroma ALF [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

According to Core Experiment (CE5) [1] common base software, when both chroma ALF and CCALF are enabled, range clipping operations are performed on modified chroma samples separately in each of the two modules to prevent overflow or underflow. In this contribution, it is proposed to perform only one range-clipping operation after chroma samples processed by ALF and CCALF. The proposed change is implemented based on CE5 [1] software. Simulation results show that the proposed change results in a BD-rate change (YUV) of 0.00%, 0.00%, 0.00% for AI, 0.00%, 0.00%, 0.00% for RA, XXX%, XXX%, XXX% for LDB, respectively.

It was commented that the benefit of the clipping may be architecture dependent.

It was commented that clipping the output of chroma ALF reduces the storage requirement if an architecture needs to buffer the result.

Revisit: It was requested that the proponent compute the dynamic range of the output of chroma ALF.

JVET-Q0531 Crosscheck of JVET-Q0494 (CE5-related: Joint clip operation for CCALF and chroma ALF) [O. Chubach (MediaTek)] [late]

JVET-Q0673 Crosscheck of JVET-Q0494 (CE5-related: Joint clip operation for CCALF and chroma ALF) [J. Heo (LGE)] [late]

JVET-Q0171 AHG9: On CC-ALF modifications related to HLS [O. Chubach, C.-Y. Chen, C.-Y. Lai, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

See notes for JVET-Q0382.

JVET-Q0250 CE5-related: Removing number of filters for CC-ALF in slice and picture header [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)]

See notes for JVET-Q0382.

JVET-Q0326 CE5-related: On CC-ALF slice and picture header syntax [X. W. Meng (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

See notes for JVET-Q0382.

JVET-Q0580 Crosscheck report of JVET-Q0326 [X. Xu (Tencent)] [late]

JVET-Q0520 AHG9: Cleanups on signaling for CC-ALF, BDPCM, ACT and Palette [Y. Wang, L. Zhang, K. Zhang, W. Zhu (Bytedance)]

Items 1 and 2 of this contribution belong to this category.

In current VVC and the suggested specification for cross-component adaptive loop filter (CC-ALF), certain coding tools, including CC-ALF, chroma BDPCM, ACT, and Palette modes are only allowed under certain conditions. More specifically, CC-ALF should be enabled for non-4:0:0 colour format, chroma BDPCM, ACT and Palette mode are only enabled for 4:4:4 colour format without separate plane coding. However, the corresponding syntax elements and semantics are not well defined to reflect these restrictions. In this contribution, two aspects are proposed to cleanup signalling for CC-ALF, and a third aspect is proposed to cleanup signalling for BDPCM, ACT, and Palette.

Aspect #1: Syntax elements related to CC-ALF are not signalled if ChromaArrayType is equal to 0.

Aspect #2: ALF and CC-ALF are controlled by two separate flags in SPS.

Aspect #3: BDPCM, ACT, and Palette mode for chroma are signalled under the condition of “ChromaArrayType is equal to 3”

For Aspect #1: See the discussion above.

Recommendation: Agree that any syntax elements related to CC-ALF should not be signalled if ChromaArrayType is equal to 0.

Recommendation: Track and/or Plenary discuss if CC-ALF should be controlled by flag in SPS.

JVET-Q0559 CE5-related: CCALF coefficient derivation using combined neighbors [K. Panusopone, S. Hong, L. Wang, J. Lainema (Nokia)] [late]

This contribution describes CCALF filter configuration that uses combined luma neighboring pixels. With the proposed combination, less correlated pixels provide less contribution to CCALF process resulting in improving performance. Simulation results show the proposed method 1 has BD-rate of approximately -0.02%, 0.24%, 0.05% for AI, -0.01%, 0.02%, 0.01% for RA, and 0.00%, 0.02%, -0.19% for LB, respectively, compared to CE5 common base. For the proposed method 2, simulation results show BD-rate of approximately 0.01%, -0.42%, -0.89% for AI, 0.00%, -0.67%, -0.93% for RA, and -0.01%, -1.31%, -1.53% for LB, respectively, compared to CE5 common base.

The document proposes two methods for modifying the filtering process.  The first method averges the two samples at the left of the kernel and averages the two samples at the right of the kernel.  This results in a filter with 6 unique coefficients.

The second method extends the filter shape to be 5x4.  The second method uses a similar approach as the first method.  Specifically, for each column that is not in the middle column, the two sample values are averaged before filtering.  This results in a filtering operation with 8 filter values.

It was commented that a filter that has built in symmetries should be tested with different chroma types.

The presenter reported that they were running simulations with the HDR CTC and would share when they are finished.

It was commented that the method studied in CE5-1.1 uses the same filter support.  However, the symmetries are not the same.

It was commented that it would be desireable to test the methods on 4:4:4 content.

One participant expressed support for method 2.