| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0793-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0561 (CE3-related: Transquant bypass mode for lossless coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 zhi-yi.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

The document reports the results of crosschecking JVET-Q0561 proposed by Qualcomm. JVET- Q0561 proposes to bring back HEVC like transquant bypass mode, where a CU-level transquant bypass flag is for each CU that indicates transform and quantization steps are bypassed, into VVC for lossless coding. The verification tasks were done, and the BD-rate results match those in JVET- Q0561.

Verification

Qualcomm provided the source code of their proposed methods described in JVET-Q0561 [1]. Three verifications were carried out by setting the following parameters in the provided code:

Test 1:

#define QC_TEST_SELECT 		1

Test 2:

#define QC_TEST_SELECT 		2

set BDPCM=1 for all sequences

Test 3:

#define QC_TEST_SELECT 		3

set BDPCM=1 for all sequences

For all tests, “TransquantBypassEnable=1” and “CUTransquantBypassFlagForce =1” are added on top of the provided “lossless.cfg” configuration file.

The tests were conducted under the test conditions in CE3 using VTM7.0 [2]. Table 1, Table 2, and Table 3 show the results using VTM7.0 as the anchor and enabling the Test 1, Test 2, and Test 3 in the test, respectively. The BD-rates of the three tests match those in JVET- Q0561.

Table1. JVET-Q0561-Test1: Lossless coding with transquant bypass mode (no BDPCM)

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.4 | -5.56% | 2.2 | 2.4 | -6.21% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.86% | 1.7 | 1.9 | -8.09% |  |  |  |
| Class B | 2.2 | 2.3 | -4.08% | 2.3 | 2.4 | -3.87% | 2.3 | 2.4 | -3.88% |
| Class C | 1.9 | 2.0 | -5.07% | 2.4 | 2.6 | -5.50% | 2.4 | 2.6 | -5.42% |
| Class D | 1.9 | 2.0 | -4.68% | 2.8 | 2.9 | -5.38% | 2.7 | 2.9 | -5.36% |
| Class E | 2.8 | 2.9 | -3.91% |  |  |  | 3.1 | 3.3 | -6.31% |
| Class F | 5.3 | 5.5 | -3.08% | 33.7 | 34.7 | -3.73% | 50.7 | 52.1 | -3.54% |
| TGM | 11.8 | 12.2 | -3.19% | 107.1 | 109.4 | -2.59% | 124.9 | 128.1 | -2.84% |
| Overall | 2.1 | 2.2 | -5.32% | 2.2 | 2.3 | -5.62% | 2.6 | 2.7 | -5.00% |
| Enc Time[%] |  | 93% |  |  | 95% |  |  | 94% |  |
| Dec Time[%] |  | 105% |  |  | 103% |  |  | 107% |  |

Table2. JVET-Q0561-Test2: Lossless coding with transquant bypass mode (Luma BDPCM)

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.4 | -5.96% | 2.2 | 2.4 | -6.48% |  |  |  |
| Class A2 | 1.6 | 1.8 | -10.06% | 1.7 | 1.9 | -8.37% |  |  |  |
| Class B | 2.2 | 2.3 | -5.64% | 2.3 | 2.5 | -4.33% | 2.3 | 2.4 | -4.36% |
| Class C | 1.9 | 2.1 | -6.51% | 2.4 | 2.6 | -5.91% | 2.4 | 2.6 | -5.86% |
| Class D | 1.9 | 2.1 | -7.54% | 2.8 | 2.9 | -5.88% | 2.7 | 2.9 | -5.74% |
| Class E | 2.8 | 3.0 | -7.07% |  |  |  | 3.1 | 3.4 | -7.23% |
| Class F | 5.3 | 5.8 | -6.94% | 33.7 | 35.6 | -5.23% | 50.7 | 52.8 | -4.62% |
| TGM | 11.8 | 12.5 | -5.59% | 107.1 | 110.6 | -3.52% | 124.9 | 128.4 | -3.09% |
| Overall | 2.1 | 2.3 | -6.86% | 2.2 | 2.4 | -5.99% | 2.6 | 2.7 | -5.58% |
| Enc Time[%] |  | 111% |  |  | 100% |  |  | 99% |  |
| Dec Time[%] |  | 92% |  |  | 101% |  |  | 108% |  |

Table3. JVET-Q0561-Test3: Lossless coding with transquant bypass mode (luma and chroma BDPCM)

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.4 | -6.75% | 2.2 | 2.4 | -6.94% |  |  |  |
| Class A2 | 1.6 | 1.8 | -10.50% | 1.7 | 1.9 | -8.42% |  |  |  |
| Class B | 2.2 | 2.3 | -6.35% | 2.3 | 2.5 | -4.53% | 2.3 | 2.4 | -4.54% |
| Class C | 1.9 | 2.1 | -7.24% | 2.4 | 2.6 | -6.08% | 2.4 | 2.6 | -6.00% |
| Class D | 1.9 | 2.1 | -8.98% | 2.8 | 2.9 | -6.15% | 2.7 | 2.9 | -5.92% |
| Class E | 2.8 | 3.1 | -9.24% |  |  |  | 3.1 | 3.4 | -7.73% |
| Class F | 5.3 | 5.9 | -8.52% | 33.7 | 36.0 | -5.94% | 50.7 | 53.3 | -5.23% |
| TGM | 11.8 | 12.9 | -8.26% | 107.1 | 111.9 | -4.50% | 124.9 | 129.8 | -4.00% |
| Overall | 2.1 | 2.3 | -7.79% | 2.2 | 2.4 | -6.20% | 2.6 | 2.7 | -5.83% |
| Enc Time[%] |  | 121% |  |  | 103% |  |  | 101% |  |
| Dec Time[%] |  | 88% |  |  | 100% |  |  | 106% |  |

References

[1] A. Nalci, M. Karczewicz, H. Wang, H. E. Egilmez, M. Coban (Qualcomm), “CE3-related: Transquant Bypass Mode for Lossless Coding,” Document of Joint Video Experts Team, JVET-Q0561.

[2] VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

Patent rights declaration(s)

MediaTek Inc. may not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).