| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0815 |
| --- | --- |

| Title: | BoG on ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang Xiaoyu Xiu | Email: | lizhang.idm@bytedance.com xiaoyu.xiu@kwai.com |
| Source: | BoG coordinator |  |  |

_____________________________

# Abstract

This BoG is related to the Adaptive Colour space Transform (ACT). The BoG was held in Jan. 14th, from 2:00pm to 4:35pm.

# Target of the BoG

“Selecting the best approaches of resolving the issues above, suggest a combined solution and coordinate the drafting of a combined spec text”

# Decisions from Track B

The following text are copied from the meeting notes, JVET_Q_notes_d8.

- There is a bug with possible QP outside valid range after ACT QP adjustment, in this context several solutions: JVET-Q0098, JVET-Q0142, JVET-Q0166, JVET-Q0241, JVET-Q0425, JVET-Q0510/JVET-Q0511. It would be desirable if the solution

  - has no effect on chroma QP used in deblocking (note it is assumed that the ACT specific QP offset should not have impact on deblocking, as deblocking is done after inverse ACT in RGB domain. Even though ACT is same as in in HEVC this problem does not exist there, as HEVC has no chroma QP adaptivity in deblocking).

  - does take QpPrimeTsMin into consideration

  - does not influence the case ACT off

- There is a bug with text spec on QP offsets (JVET-Q0241 aspect 1)

- Several proposals for signaling/adapting ACT QP offsets. Global offsets should be included. Local chroma QP offset should also be applied to ACT coded blocks, but not defining a separate list of offsets for ACT

- Current ACT design should not be modified

- Clipping at input of inverse ACT is necessary: JVET-Q0513

- Disable ACT for max transform size 64: JVET-Q0162

# Review of JVET-Q080

Abstract: In the 16th JVET meeting, the adaptive color transform (ACT) for 444 video coding was adopted in VVC.  The QP adjustments of (-5, -5, -3) are applied to the transform residuals to compensate the dynamic range change of residuals signals before and after color transform. In this contribution, it’s proposed to signal additional ACT QP offsets at picture parameter set and slice header to provide flexibility of QP control for ACT coded block.

It is commented that ACT QP offsets are applied in the deblocking filtering process. Proponents clarified that the offsets are not (-5, -5, -3) which are used for compensation of transform matrix normalization differences, and those signaled ACT QP offsets are applied to the deblocking filter process.

Experts confirmed that all of the ACT QP offsets are not applied in the deblocking filter process in HEVC SCC. Concerns were raised that why ACT QP offsets should be applied in the deblocking filter process.

It is commented that PPS syntax elements are referring to the SPS syntax elements which are not allowed. Errors are existing in the proposal.

It is commented that instead of checking the SPS ACT enabling flag, it is better to add a syntax element (i.e., a flag in PPS) to indicate the presence of ACT QP offsets, as proposed in JVET-Q0425.

It is commented that equation 1133 and 1136, Clip3 shall be replaced by max function since the values shall be (63+QpBdOffset). Errors are existing in the proposal.

What’s the justification of ACT QP offsets signaled in slice headers in addition to PPS?  Proponents wanted to follow the design of QP offsets for chroma.

Any reason for adding additional ACT QP offsets in RGB domain for ACT coded blocks? Proponents wanted to allow flexibility for ACT coded blocks.

The proposal applied the ACT QP offset to non-ACT blocks. CU-level ACT flag checking is missing in the proposal. Errors are existing in the proposal.

# Solutions on the table

The following aspects that are related to ACT QP offset signaling and usage are summarized as follows:

- On signaling of ACT QP offsets

- Fixed QP offsets (-5, -5, -3) for JCCR off and -5 for JCCR On without any signaling: JVET-Q0098, JVET-Q0142, JVET-Q0241, JVET-Q0511

- Global signaling of ACT QP offsets: JVET-Q0425, JVET-Q0510, JVET-Q0808

  - Signal in PPS: JVET-Q0425, JVET-Q0510, JVET-Q0808

    - Signal offsets of the three-color components respectively

    - Signal offset for ACT and JCCR coded blocks

  - Signal in SH: JVET-Q0808

    - Override mechanism

- On QP clipping in dequantization process

- Directly use (-5, -5, -3) for JCCR off and -5 for JCCR On, only lower bound is clipped: JVET-Q0098, JVET-Q0142, JVET-Q0241, JVET-Q0511

- Use the signaled ACT QP offsets without modification, both upper and lower bound are clipped: JVET-Q0425, JVET-Q0510

- Use the signaled ACT QP offsets plus (-5, -5, -3) for JCCR off and -5 for JCCR On, lower bound is clipped: JVET-Q0808

- On QP usage in Deblocking filter process

- None of the ACT QP offsets are applied: JVET-Q0098, JVET-Q0142, JVET-Q0241, JVET-Q0511, JVET-Q0425, JVET-Q0510

- Signalled ACT QP offsets are applied: JVET-Q0808

Discussions:

Why ACT QP offsets are required to be signalled?

One motivation is for the adjustment of color transform offsets. One expert pointed out that using different ACT QP offset (i.e., different from (-5, -5, -3)) could bring small coding gain (~0.2%) under low QP settings with limited settings tried.

Another motivation is for flexibility. Encoder could select the optimum QP offsets for ACT coded blocks. Some other experts also have concerns why such flexibility is needed.

Two experts expressed that for the purpose of (-5, -5, -3), it is for the transform matrix normalization. However, if additional ACT QP offsets for ACT coded blocks are signaled for global delta QP purpose, the additional ACT QP offsets may be used in the deblocking filter process. One proponent asked for evidence to prove that not applying the ACT QP offsets to deblocking is a problem.

Options to be discussed:

- Common base (the simplest solution) should include the following aspects, to be provided by P. de Lagrange

- Fixed ACT QP offsets (-5, -5, -3) without being signaled, to be used in the dequantization process: JVET-Q0098, JVET-Q0142, JVET-Q0241, JVET-Q0511

- Fix the clipping issue due to negative QP values: JVET-Q0098, JVET-Q0142, JVET-Q0241, JVET-Q0425, JVET-Q0511, JVET-Q0808

  - has no effect on chroma QP used in deblocking.

  - does take QpPrimeTsMin into consideration

  - does not influence the case ACT off

- Clipping at input of inverse ACT is necessary: JVET-Q0513

- Disable ACT for max transform size 64: JVET-Q0162

- JVET-Q0808 + JVET-Q0513 + JVET-Q0162

- JVET-Q0510/JVET-Q0425 + JVET-Q0513 + JVET-Q0162