| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020; Document: JVET-P0703-v1. | Document: JVET-Q0821 |
| --- | --- |

| Title: | Cross-check of JVET-Q0496 (AHG16: On motion shift derivation of SbTMVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haiwei Sun Che-Wei Kuo | Tel: Email: | +65-6550-5499 haiwei.sun@sg.panasonic.com chewei.kuo@sg.panasonic.com |  |
| Source: |  |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-Q0496 on the derivation of motion shift of SbTMVP. In JVET-Q0496, it is proposed to obtain the motion shift from the above neighboring block (Method #1), or from a block outside a predefine regions such as a 64x64 VPDU (Method #2). It is reported that the results matched those provided in JVET-Q0496. Software implementation matches the proposal of the contribution.

# Introduction

The way the subblock TMVP (SbTMVP) is constructed introduces dependency on the left neighboring block. In hardware encoder implementations, the motion estimation stage of multiple blocks is typically performed in parallel or at least pipelined to increase the throughput. Therefore, below two methods are proposed to make the derivation of SbTMVP more hardware friendly.

Method #1: to derive the motion shift from the above neighboring block (B1) instead of the left neighboring block (A1)

Method #2: to derive motion shifts for CUs within one predefine region such as VPDU (64x64 block) using the same left block immediately adjacent to the VPDU

# Simulation results

BD-Rate results of the tests on top of the VTM-7.0 compared to anchor with RA and LDB configurations are shown in following tables. The coding time is not reliable.

Table 1: Results of method #1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.04% | 102% | 97% |
| Class A2 | 0.00% | 0.00% | 0.02% | 97% | 96% |
| Class B | 0.00% | 0.05% | 0.02% | 100% | 98% |
| Class C | 0.01% | 0.02% | 0.06% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 101% | 98% |
| Class D | -0.02% | -0.03% | -0.04% | 105% | 94% |
| Class F | 0.01% | -0.04% | 0.00% | 86% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | 0.10% | 98% | 98% |
| Class C | -0.03% | -0.14% | -0.06% | 110% | 111% |
| Class E | 0.01% | 0.16% | -0.72% | 99% | 100% |
| Overall | -0.01% | -0.07% | -0.16% | 102% | 103% |
| Class D | -0.07% | 0.78% | 0.25% | 107% | 98% |
| Class F | 0.04% | -0.37% | -0.12% | 98% | 95% |

Table 2: Results of method #2

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | -0.02% | 104% | 99% |
| Class A2 | 0.00% | 0.01% | 0.03% | 98% | 99% |
| Class B | 0.02% | 0.03% | 0.01% | 99% | 99% |
| Class C | 0.05% | 0.04% | 0.06% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.02% | 0.02% | 101% | 99% |
| Class D | 0.03% | -0.03% | 0.02% | 105% | 94% |
| Class F | 0.02% | 0.00% | 0.01% | 87% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.08% | 0.23% | 96% | 96% |
| Class C | 0.00% | 0.11% | 0.03% | 110% | 109% |
| Class E | -0.01% | 0.33% | -0.52% | 99% | 99% |
| Overall | 0.02% | 0.09% | -0.02% | 101% | 101% |
| Class D | 0.02% | -0.35% | -0.10% | 106% | 98% |
| Class F | 0.05% | -0.24% | 0.64% | 98% | 96% |

# References

Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang (Kwai Inc.), “AHG16: On motion shift derivation of SbTMVP”, JVET-Q0496, 17th Meeting: Brussels, BE, 7–17 January 2020.