| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0016-v1 |
| --- | --- |

| Title: | JVET AHG report: Implementation studies (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu | Email: | minhua.zhou@broadcom.com jicheng.ajc@alibaba-inc.com eric.chai@ublnx.com kiho14.choi@samsung.com sriram.sethuraman@ittiam.com thsieh@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | AHG16 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG16: Implementation studies, between the 17th JVET meeting in Brussels, BE (7–17 January 2020) and the 18th meeting by teleconference (15–24 April 2020).

The AHG was established with the following three mandates:

- Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty.

- Solicit hardware analysis of complex tools.

- Provide feedback on potential solutions to address identified issues.

# Activities

A kick-off message was sent to the main JVET email reflector (jvet@lists.rwth-aachen.de) on 02/18/2020.

The following three issues need to be resolved:

- The newly adopted CCALF was identified to have a line buffer issue in 4:2:2/4:4:4 coding. For 4:2:2/4:4:4 chroma format, the luma de-blocking is two lines behind the Chroma de-blocking, but the CCLAF uses the co-located luma lines (after de-blocking and SAO) for processing, resulting in the need to buffer two Chroma lines for each Chroma component. JVET-R0233, JVET-R0291 and JVET-R0321 were submitted to address this cost issue.

- After the adoption of slice_ts_residual_coding_disabled_flag at the last meeting, the transform skip can now be combined with the regular transform coefficients coding, but its interaction with the dependent quantization and sign data hiding is not clearly specified. Potential solutions can be found in JVET-R0153 and other related contributions.

- The availability check of the upper-left neighboring sample used in CCLM is broken in a corner case. When the raster scan slice is in use, the upper-left neighboring sample may still be unavailable even if both the left and top neighbors are available.  JVET-R0314 and JVET-R0375 provide solutions for bug fixes.

# Related contributions

The following contributions are identified for the AHG.

## In-loop filters (9)

- JVET-R0233, “AHG16: Line buffer problem of CC-ALF for 4:2:2 and 4:4:4 sequences”,	N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-R0291, “AHG16: On ALF attenuation near virtual boundaries”, F. Bossen (Sharp)

- JVET-R0312, “AHG2/AHG16: A fix on chroma ALF virtual boundary position”, Y. Wang, L. Zhang, H. Liu, K. Zhang, Z. Deng (Bytedance)

- JVET-R0322, “CCALF virtual boundary issue for 4:4:4 and 4:2:2 format”, 	X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)

- JVET-R0208, “AHG16: Rounding correction for ALF virtual boundary processing”,	A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)

- JVET-R0313, “AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control”,	Y. Wang, L. Zhang, H. Liu, K. Zhang (Bytedance)

- JVET-R0128, “AHG16: On CCALF clipping”	M. G. Sarwer, Y. Ye, J. Luo (Alibaba)

- JVET-R0133, “AHG16: On Clipping values for Non-linear ALF	“, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)

- JVET-R0289, “AHG16: On deblocking filter process”, 	N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)

## TSRC disabling/dependent quantization/sign data hiding (8)

- JVET-R0050, “AHG9: HLS on dependent quantization and sign data hiding”,	S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-R0083, “AHG14: Residual coding constraints for transform skip blocks”, A. Nalci, H.E. Egilmez, M. Coban, V. Seregin, M. Karczewicz (Qualcomm), M. G. Sarwer, Y. Ye, J. Luo (Alibaba)

- JVET-R0116, “AHG11/AHG14: On sign data hiding of transform skip block”,	M. G. Sarwer, Y. Ye, J. Luo (Alibaba), A. Nalci, H. E. Egilmez, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-R0141, “Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks”,	T. Hashimoto, E. Sasaki, T. Aono, T. Ikai (Sharp)

## JVET-R0153, “AHG9/AHG16: On slice_ts_residual_coding_disabled_flag”,	J. Choi, S. Yoo, J. Heo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-R0154, “AHG9/16: On sign data hiding for BDPCM blocks”,	S. Yoo, J. Choi, J. Lim, S. Kim (LGE)

- JVET-R0271, “AHG9/AHG14: High-level constraints of dependent quantization and sign data hiding”, 	A. Nalci, M. Coban, M. Karczewicz (Qualcomm)

- JVET-R0325, “AHG14: Disabling dependent quantization and sign bit hiding for transform skip mode”,	T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)

## Intra prediction/CCLM bug fixes/LMCS (7)

## JVET-R0280, “AHG16: Cleanup of intra reference sample filter selection”,	J. Heo, H. Jang, J. Choi, J. Nam, M. Koo, J. Lim, S. Kim (LGE)

## JVET-R0281, “AHG16: Cleanup MIP flag signaling”,	J. Heo, H. Jang, J. Choi, J. Lim, S. Kim (LGE)

## JVET-R0288, “AHG16: Reference samples for ISP”,	F. Bossen (Sharp)

## JVET-R0314, “AHG2/AHG16: Fixes on CCLM	“, Y. Wang, K. Zhang, L. Zhang, H. Liu (Bytedance)

## JVET-R0375, “AHG2/AHG16: CCLM bug fix in luma reference down-sampling	“, L. Pham Van, G. Van Der Auwera, J. Chen, V. Seregin, M. Karczewicz (Qualcomm)

## JVET-R0290, “AHG16: LMCS constraint cleanup”,	F. Bossen (Sharp)

## JVET-R0330, “AHG16: On clipping average luma value for chroma residual scaling factor derivation”, X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai)

## Palette (2)

## JVET-R0309, “AHG16: Clean-up on palette predictor update for local dual tree.”, H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)

## JVET-R0310, “AHG16: Clean-up by removing parsing dependency for palette”, H. Jang, J. Nam, S. Yoo, N. Park, S. Kim, J. Lim (LGE)

## Implementation (3)

## JVET-R0316, “AHG16: Normative constraints on BT and TT split under MER	“, Y. Wang, K. Zhang, L. Zhang, H. Liu, Z. Deng (Bytedance)

## JVET-R0224, “AHG16: Realization of RPR based real-time VVC decode and playback on ARM based mobile devices”,  	J. Shingala, A. Natesan, A. Chelawat (Ittiam)

## JVET-R0390: “AHG16: VVC multi-thread decoder and performance analysis”, 	S. Gudumasu, T. Poirier, F. Urban, F. Hiron, P. de Lagrange (interdigital)

## Other AHG16-related contributions (1)

## JVET-R0223, “AHG16: On DMVR and wraparound motion compensation”,	J. Luo, J. Chen, Y. Ye (Alibaba)

# Recommendations

The AHG recommends reviewing the input contributions (see JVET-R0340).