| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0043 |
| --- | --- |

| Title: | AHG9: On subpicture interaction with other tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li, Kiyofumi Abe | Tel: Email: | Jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to disable tool combinations between (1) subpicture and field coding (2) subpicture and Gradual Decoder Refresh (GDR). It is asserted that the concept between subpicture and field coding are mutually exclusive. And subpicture and GDR do not work well together.

# Introduction

In last meeting, combinations between subpicture and other coding tools were taken into consideration. In current specifications, following tool combinations are disallowed:

- Reference picture resampling with subpictures

- Reference picture wraparound with subpicture treated as picture

- virtual boundary with subpictures:

In this proposal, it is proposed to further extend the constraint to field coding and GDR.

For field coding, it is asserted that there is no clear use case of subpictures that the corresponding CVLS conveys subpictures as fields.

- Field coding is targeting for devices, like televisions, that do not display a frame all at once, and usually of resolution equal or lower than HD;

- Subpicture targets at (1) viewport dependent decoding devices, like a head-mount-display (HMD), (2) 360 equirectangular and cubemap projection formatted video (3) E-sports.[1]-[4]

The use case for field coding and subpictures are mutually exclusive, it is proposed to disable field coding when subpicture is present.

For GDR, it is proposed to decouple the concept of subpictures and GDR;

- GDR is for real-time implementation of video encoders for the benefits such as bit-rate smoothing, maintaining Intra data bit-rate under control, low delay and natural error resilience limiting temporal error propagation along frames.[5] Subpictures are mainly used for viewport dependent 360 video streaming, wherein the source might be independently coded for each segments of subpictures and packaging is sometimes done during end-to-end transmission, it is difficult to synchronize those independent coded subpictures to meet the “sliding window updating” functionality of GDR.

- On the other hand, applying GDR requires enabling using of virtual boundaries, however, virtual boundaries are disabled when subpictures are present in current specification. Therefore, current specification and software does not support functionality of GDR when subpicture is used; It is therefore not making much sense to still set GDR enabling flag to true while the functionalities are never supported.

# Proposed methods

## To disable tool combination between subpicture and field coding.

It is proposed to implicitly set field_seq_flag as 0 when subpic_info_present_flag is 1.

The proposed syntax change is as below:

| if( !subpic_info_present_flag ) { |  |
| --- | --- |
| field_seq_flag | u(1) |
| } |  |

field_seq_flag equal to 1 indicates that the CLVS conveys pictures that represent fields. field_seq_flag equal to 0 indicates that the CLVS conveys pictures that represent frames. When general_frame_only_constraint_flag is equal to 1, the value of field_seq_flag shall be equal to 0.

When field_seq_flag is not present, it is set to value 0.

Alternatively, a semantic constraint could be made instead of syntax change:

It is a requirement of bitstream conformance that, when subpic_info_present_flag is equal to 1, the value of field_seq_flag shall be equal to 0.

## To disable tool combination between subpicture and GDR.

It is proposed to decouple subpicture and GDR.

Syntax change is as below, note that signaling gdr_enabled_flag is moved after subpic_info_present_flag:

| if( !subpic_info_present_flag ) { |  |
| --- | --- |
| gdr_enabled_flag | u(1) |
| } |  |

gdr_enabled_flag equal to 1 specifies that GDR pictures may be present in CLVSs referring to the SPS. gdr_enabled_flag equal to 0 specifies that GDR pictures are not present in CLVSs referring to the SPS.

When gdr_enabled_flag is not present, it is set to value 0.

Alternatively, a semantic constraint could be made instead of syntax change:

It is a requirement of bitstream conformance that, when subpic_info_present_flag is equal to 1, the value of gdr_enabled_flag shall be equal to 0.

# Reference

[1] B. Choi, "AHG12: On sub-picture design", JVET-N0055, March 2019

[2] M. M. Hannuksela, "Use cases and proposed design choices for adaptive resolution changing (ARC)", JVET-M0259, January 2019

[3] Sachin Deshpande, "WD6 of ISO/IEC 23090-2 OMAF 2nd edition", w18587, OMAF, July Gothenburg,

http://wg11.sc29.org/doc_end_user/documents/127_Gothenburg/wg11/w18587.zip

[4] Prakash Kolan, "Streaming E-Sports Event", m46551, MPEG-I Systems, January 2019

http://phenix.it-sudparis.eu/mpeg/doc_end_user/current_document.php?id=66208&id_meeting=177

[5] J.-M. Thiesse, "AHG14: Intra Refresh Test conditions and Anchors generation Proposal", JVET-L0161, Oct 2018

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).