| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0050-v3 |
| --- | --- |

| Title: | AHG9: HLS on dependent quantization and sign data hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der. Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes five high-level modifications related to dependent quantization and sign data hiding, summarized as follows:

- When sps_dep_quant_enabled_flag is equal to 1, a new sequence parameter set (SPS) syntax element sps_dep_quant_enabled_pic_present_flag is further signalled to indicate whether ph_dep_quant_enabled_flag is present in the picture header.

- When sps_sign_data_hiding_enabled_flag is equal to 1, a new SPS syntax element sps_sign_data_hiding_enabled_pic_present_flag is further signalled to indicate whether pic_sign_data_hiding_enabled_flag is present in the picture header.

- pic_sign_data_hiding_enabled_flag is renamed as ph_sign_data_hiding_enabled_flag.

- Simplification of deriving the variable signHidden by setting the value of signHidden equal to 0 if pic_sign_data_hiding_enabled_flag is equal to 0 in the syntax table (editorial only).

- Signal sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag independently but dependent quantization and sign data hiding are still mutually exclusive for each picture.

In version 2 of this contribution, the proposed modifications to Specification only for items 1 and 2 are provided.

In version 3 of this contribution, the proposed modifications to Specification only for items 1 and 2 further integrated with JVET-R0271 Variant #3 are provided.

Introduction

In Versatile Video Coding (VVC) Draft 8 as specified in [1], the new syntax elements sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are added to the SPS to indicate whether dependent quantization and sign bit hiding, respectively, may be enabled for pictures referring to the SPS. When sps_dep_quant_enabled_flag or sps_sign_data_hiding_enabled_flag is equal to 1, the syntax element ph_dep_quant_enabled_flag or pic_sign_data_hiding_enabled_flag is further signalled in the picture header (PH) to indicate whether dependent quantization or sign bit hiding is enabled for the current picture for each of the coded pictures referring to the SPS.

Proposed modifications

Modification 1: Further signalling the new syntax element sps_dep_quant_enabled_pic_present_flag

In Modification 1, when the syntax element sps_dep_quant_enabled_flag is equal to 1, a new syntax element sps_dep_quant_enabled_pic_present_flag is further signalled in the SPS. When sps_dep_quant_enabled_pic_present_flag is equal to 0, ph_dep_quant_enabled_flag is not coded in the current PH referring to the SPS and it is inferred to be equal to sps_dep_quant_enabled_flag. In this way, when a VVC encoder chooses to always enable dependent quantization for all pictures in a coded layer video sequence (CLVS), the bit for signalling ph_dep_quant_enabled_flag can be saved for each of the PHs referring to the SPS by setting the value of sps_dep_quant_enabled_pic_present_flag equal to 0.

Modification 2: Further signalling the new syntax element sps_sign_data_hiding_enabled_pic_present_flag

In Modification 2, when the syntax element sps_sign_data_hiding_enabled_flag is equal to 1, a new syntax element sps_sign_data_hiding_enabled_pic_present_flag is further signalled in the SPS. When sps_sign_data_hiding_enabled_pic_present_flag is equal to 0, pic_sign_data_hiding_enabled_flag is not coded in the current PH referring to the SPS and it is inferred to be equal to sps_sign_data_hiding_enabled_flag. In this way, when a VVC encoder chooses to always enable sign data hiding for all pictures in a CLVS, the bit for signalling pic_sign_data_hiding_enabled_flag can be saved for each of the PHs referring to the SPS by setting the sps_sign_data_hiding_enabled_pic_present_flag equal to 0.

Modification 3: Renaming pic_sign_data_hiding_enabled_flag as ph_sign_data_hiding_enabled_flag

In Modification 3, the syntax element pic_sign_data_hiding_enabled_flag is renamed as ph_sign_data_hiding_enabled_flag in consistence with naming for other PH syntax elements.

Modification 4: Simplification of deriving signHidden

In VVC Draft 8, the value of the variable signHidden is set equal to 0 when ph_dep_quant_enabled_flag is equa to 1 or pic_sign_data_hiding_enabled_flag is equal to 0. However, when ph_dep_quant_enabled_flag is equal to 1, pic_sign_data_hiding_enabled_flag is always equal to 0. In the proposed modification, the derivation of the value of signHidden is simplified by removing the redundant check on wthether ph_dep_quant_enabled_flag is equa to 1 and setting the value of signHidden equal to 0 when pic_sign_data_hiding_enabled_flag is equal to 0.

Modification 5: Signalling sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag independently

In VVC Draft 8, sps_sign_data_hiding_enabled_flag is signalled only when sps_dep_quant_enabled_flag is equal to 0. In the proposed modification, sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are independently in the SPS. In this way, when both sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are both equal to 1, sign data hiding may still be enabled for the pictures with ph_dep_quant_enabled_flag equal to 0. For each picture, dependent quantization and sign data hiding are still mutually exclusive.

Modified syntax tables and semantics

The modified parts of the syntax tables based on VVC Draft 8 are written in red and provided below:

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag ) |  |
| sps_dep_quant_enabled_pic_present_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled _flag | u(1) |
| if( sps_sign_data_hiding_enabled _flag ) |  |
| sps_sign_data_hiding_enabled_pic_present_flag | u(1) |
| …. |  |
| } |  |

- 7.3.2.7 Picture header structure syntax

| picture_header_structure ( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| ….. |  |
| if( sps_dep_quant_enabled_pic_present_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_pic_present_flag && ! ph_dep_quant_enabled_flag ) |  |
| phic_sign_data_hiding_enabled_flag | u(1) |
| …. |  |
| } |  |

- 7.3.10.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( ph_dep_quant_enabled_flag ¦ ¦ !phic_sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 | u(1) |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) | u(1) |
| …. |  |
| } |  |

The modified parts of the semantics based on VVC Draft 8 are written in red and provided below:

- 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for pictures referring to the SPS. sps_dep_quant_enabled_flag equal to 1 specifies that dependent quantization may be enabled for pictures referring to the SPS.

sps_dep_quant_enabled_pic_present_flag equal to 1 specifies that the syntax element ph_dep_quant_enabled_flag is present in PHs referring to the SPS. sps_dep_quant_enabled_pic_present_flag equal to 0 specifies that the syntax element ph_dep_quant_enabled_flag is not present in PHs referring to the SPS. When sps_dep_quant_enabled_pic_present_flag is not present, it is inferred to be equal to 0.

sps_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for pictures referring to the SPS. sps_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding may be enabled for pictures referring to the SPS. When sps_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

sps_sign_data_hiding_enabled_pic_present_flag equal to 1 specifies that the syntax element ph_sign_data_hiding_enabled_flag is present in PHs referring to the SPS. sps_sign_data_hiding_enabled_pic_present_flag equal to 0 specifies that the syntax element ph_sign_data_hiding_enabled_flag is not present in PHs referring to the SPS. When sps_sign_data_hiding_enabled_pic_present_flag is not present, it is inferred to be equal to 0.

….

- 7.4.3.3 Picture header RBSP semantics

….

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to the value of sps_dep_quant_enabled_flag.

phic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. phic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When phic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0. the following applies:

–	If ph_dep_quant_enabled_flag is equal to 1, the value of ph_sign_data_hiding_enabled_flag is inferred to be equal to 0.

–	Otherwise, the value of ph_sign_data_hiding_enabled_flag is inferred to be equal to sps_sign_data_hiding_enabled_flag.

….

Modified specification only for Modifications 1 and 2

Two methods are proposed for signalling the presence of ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag. In Method 1, two separate syntax flags are added to the SPS to specify whether ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag are present in the picture header, respectively. In Method 2, one shared syntax flag is added to the SPS to specify whether ph_dep_quant_enabled_flag or pic_sign_data_hiding_enabled_flag is present in the picture header. The proposed modifications to VVC Draft 8 only for Modifications 1 and 2 are written in red and provided below.

Method 1: Two separate present flags

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag ) |  |
| sps_dep_quant_enabled_pic_present_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| else { |  |
| sps_sign_data_hiding_enabled _flag | u(1) |
| if( sps_sign_data_hiding_enabled _flag ) |  |
| sps_sign_data_hiding_enabled_pic_present_flag | u(1) |
| } |  |
| …. |  |
| } |  |

- 7.3.2.7 Picture header structure syntax

| picture_header_structure ( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| ….. |  |
| if( sps_dep_quant_enabled_pic_present_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_pic_present_flag && ! ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| …. |  |
| } |  |

The modified parts of the semantics based on VVC Draft 8 are written in red and provided below:

- 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for pictures referring to the SPS. sps_dep_quant_enabled_flag equal to 1 specifies that dependent quantization may be enabled for pictures referring to the SPS.

sps_dep_quant_enabled_pic_present_flag equal to 1 specifies that the syntax element ph_dep_quant_enabled_flag is present in PHs referring to the SPS. sps_dep_quant_enabled_pic_present_flag equal to 0 specifies that the syntax element ph_dep_quant_enabled_flag is not present in PHs referring to the SPS. When sps_dep_quant_enabled_pic_present_flag is not present, it is inferred to be equal to 0.

sps_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for pictures referring to the SPS. sps_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding may be enabled for pictures referring to the SPS. When sps_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

sps_sign_data_hiding_enabled_pic_present_flag equal to 1 specifies that the syntax element pic_sign_data_hiding_enabled_flag is present in PHs referring to the SPS. sps_sign_data_hiding_enabled_pic_present_flag equal to 0 specifies that the syntax element pic_sign_data_hiding_enabled_flag is not present in PHs referring to the SPS. When sps_sign_data_hiding_enabled_pic_present_flag is not present, it is inferred to be equal to 0.

….

- 7.4.3.3 Picture header RBSP semantics

….

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0 sps_dep_quant_enabled_flag.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0 sps_sign_data_hiding_enabled_flag.

Method 2: One shared present flag

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled _flag | u(1) |
| if( sps_dep_quant_enabled_flag ¦ ¦ sps_sign_data_hiding_enabled _flag ) |  |
| sps_dep_quant_or_sign_data_hiding_enabled_pic_present_flag | u(1) |
| …. |  |
| } |  |

- 7.3.2.7 Picture header structure syntax

| picture_header_structure ( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| ….. |  |
| if ( sps_dep_quant_or_sign_data_hiding_enabled_pic_present_flag ) { |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| } |  |
| …. |  |
| } |  |

The modified parts of the semantics based on VVC Draft 8 are written in red and provided below:

- 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for pictures referring to the SPS. sps_dep_quant_enabled_flag equal to 1 specifies that dependent quantization may be enabled for pictures referring to the SPS.

sps_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for pictures referring to the SPS. sps_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding may be enabled for pictures referring to the SPS. When sps_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

sps_dep_quant_or_sign_data_hiding_enabled_pic_present_flag equal to 1 specifies that the syntax element ph_dep_quant_enabled_flag or pic_sign_data_hiding_enabled_flag is present in PHs referring to the SPS. sps_dep_quant_or_sign_data_hiding_enabled_pic_present_flag equal to 0 specifies that the syntax elements ph_dep_quant_enabled_flag and pic_sign_data_hiding_enabled_flag are not present in PHs referring to the SPS. When sps_dep_quant_or_sign_data_hiding_enabled_pic_present_flag is not present, it is inferred to be equal to 0.

….

- 7.4.3.3 Picture header RBSP semantics

….

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0 sps_dep_quant_enabled_flag.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0 sps_sign_data_hiding_enabled_flag.

Modified specification only for Modifications 1 and 2 integrated with JVET-R0271 Variant #3

The proposed modifications to VVC Draft 8 for Modifications 1 and 2 further integrated with JVET-R0271 Variant #3 are provided below, where our proposed modifications are written in red and the proposed modifications by JVET-R0271 Variant #3 are highlighted in yellow.

Method 1: Two separate present flags

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( sps_dep_quant_enabled_flag ) |  |
| sps_dep_quant_enabled_slice_present_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| else { |  |
| sps_sign_data_hiding_enabled _flag | u(1) |
| if( sps_sign_data_hiding_enabled _flag ) |  |
| sps_sign_data_hiding_enabled_slice_present_flag | u(1) |
| } |  |
| …. |  |
| } |  |

- 7.3.2.7 Picture header structure syntax

| picture_header_structure ( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| ….. |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && ! ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| …. |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_slice_present _flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_slice_present_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( ! slice_dep_quant_enabled_flag && ! slice_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| … |  |
| } |  |

The modified parts of the semantics based on VVC Draft 8 are written in red and provided below:

- 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for pictures referring to the SPS. sps_dep_quant_enabled_flag equal to 1 specifies that dependent quantization may be enabled for pictures referring to the SPS.

sps_dep_quant_enabled_slice_present_flag equal to 1 specifies that the syntax element slice_dep_quant_enabled_flag is present in SHs referring to the SPS. sps_dep_quant_enabled_slice_present_flag equal to 0 specifies that the syntax element slice_dep_quant_enabled_flag is not present in SHs referring to the SPS. When sps_dep_quant_enabled_slice_present_flag is not present, it is inferred to be equal to 0.

sps_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for pictures referring to the SPS. sps_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding may be enabled for pictures referring to the SPS. When sps_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

sps_sign_data_hiding_enabled_slice_present_flag equal to 1 specifies that the syntax element slice_sign_data_hiding_enabled_flag is present in SHs referring to the SPS. sps_sign_data_hiding_enabled_slice_present_flag equal to 0 specifies that the syntax element slice_sign_data_hiding_enabled_flag is not present in SHs referring to the SPS. When sps_sign_data_hiding_enabled_slice_present_flag is not present, it is inferred to be equal to 0.

….

- 7.4.3.3 Picture header RBSP semantics

….

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

7.4.8.1 General slice header semantics

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0 sps_dep_quant_enabled_flag.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0 sps_sign_data_hiding_enabled_flag.

Method 2: One shared present flag

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| ….. |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled _flag | u(1) |
| if( sps_dep_quant_enabled_flag ¦ ¦ sps_sign_data_hiding_enabled _flag ) |  |
| sps_dep_quant_or_sign_data_hiding_enabled_slice_present_flag | u(1) |
| …. |  |
| } |  |

- 7.3.2.7 Picture header structure syntax

| picture_header_structure ( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| ….. |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && ! ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| …. |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( sps_dep_quant_or_sign_data_hiding_enabled_slice_present_flag ) { |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| } |  |
| if( ! slice_dep_quant_enabled_flag && ! slice_sign_data_hiding_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| … |  |
| } |  |

The modified parts of the semantics based on VVC Draft 8 are written in red and provided below:

- 7.4.3.3 Sequence parameter set RBSP semantics

….

sps_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for pictures referring to the SPS. sps_dep_quant_enabled_flag equal to 1 specifies that dependent quantization may be enabled for pictures referring to the SPS.

sps_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for pictures referring to the SPS. sps_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding may be enabled for pictures referring to the SPS. When sps_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

sps_dep_quant_or_sign_data_hiding_enabled_slice_present_flag equal to 1 specifies that the syntax element slice_dep_quant_enabled_flag or slice_sign_data_hiding_enabled_flag is present in SHs referring to the SPS. sps_dep_quant_or_sign_data_hiding_enabled_slice_present_flag equal to 0 specifies that the syntax elements slice_dep_quant_enabled_flag and slice_sign_data_hiding_enabled_flag are not present in SHs referring to the SPS. When sps_dep_quant_or_sign_data_hiding_enabled_slice_present_flag is not present, it is inferred to be equal to 0.

….

- 7.4.3.3 Picture header RBSP semantics

….

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0 sps_dep_quant_enabled_flag.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0 sps_sign_data_hiding_enabled_flag.

7.4.8.1 General slice header semantics

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0 sps_dep_quant_enabled_flag.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0 sps_sign_data_hiding_enabled_flag.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).